SA WG2 Meeting #170	S2-2506889

25 – 29 Aug, 2025, Goteborg , SE

Source:	Apple

Title:	[WT#6, Computing] Work task description for 6G Computing

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the work task scoping for 6G Computing – enable distributed deployment of compute resources, monitor and enforce end-to-end QoS and enable federation of compute resources.

# 1. Justifications

Architectural Motivation:

5G has defined enablers for native support of Edge Computing. 3GPP SA6 has also defined the architecture for Edge Enabler Layer (EEL) that would help translate the 5G Services for the application domain. At the same time, there are many Over-The-Top approaches defined by various stakeholders in the broader cloud computing ecosystem. 6G provides an opportunity to enhance on the 3GPP framework and enable newer approaches to offer computing offload. The aim of these enhancements are to cater new demands for compute offload and enable new service models between MNOs and other stakeholders like Application Service Providers.

Gap Analysis:

5G Edge computing enabled discovery and selection of Edge Application server also considering the UE’s point of attachment. 5G also allowed AF to influence traffic routing towards an Edge Application Server.

For the applications anticipated  in 6G era, a more distributed and agile compute offload architecture is useful. The aim of such deployment in 6G system is to have a better control on the end-to-end latency and enable more informed decisions at application layer on compute offload.

More distributed compute resource deployments are helpful to offload computing task to the most optimal site closer to the UE’s point of access. But in a scenario, where UE changes access network without changing location this may lead to a situation that compute resources selected within the 3GPP network domain is no longer accessible. It would be helpful to have the possibility for federating compute resources across networks (3GPP and non-3GPP) such that UE’s access is not limiting the selection of optimal compute resource.

For a compute offload architecture, there may be a need to share metadata about computing aspects broader than what was done for influence on traffic routing or EAS discovery. It is important to carefully analyse and minimize data to be shared and also to build sufficient guard rails in the architecture.

Technical Impact:

This work task is mostly expected to reuse architectural elements defined in 5G. There may be new NFs defined or functionalities of existing NFs may be enhanced.

Having possibility to access 6G compute resources agnostic to the UE’s access network would make the feature more appealing from a migration perspective (i.e., when UE moves out of 6G RAN, but stays within NG-RAN coverage of the operator).

**** First Change ****

# Annex A.X. WT#X Scope

WT#6: Study aspects on support of computing for UE, core network and application server in 6G service hosting environment.

The work task focusses on the following aspects for a distributed deployment of compute resources in 6GC/Service hosting environment

- Whether and how to enable 3rd party AFs to manage compute resources for supporting compute offload

- Whether and how to enable 6GC to expose compute resources towards 3rd party AFs / Subscribers.

- Whether and how to monitor and enforce end-to-end QoS for traffic between UE and compute resources.

- Whether and how to enable 6GC to select, reselect and release appropriate compute resources considering energy efficiency, UE mobility and application requirements.

- Whether and how to enable 6GC to federate compute resources of multiple MNOs/Non-cellular Network operators ?

  - Whether and how to enable application layer service continuity when computing resources within 6GC/service hosting environment are accessed from outside the network (UE connectivity through non-cellular)

- Whether and how to preserve privacy considering new requirements on data to be shared for compute offload.

NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

**** End of Changes ****