3GPP TSG-SA2 Meeting #170	S2-2506929

# Gothenburg, Sweden, 2025-08-25 – 2025-08-29

Source:	Ericsson, T-Mobile USA Inc.

Title:	[General aspects] Architecture assumptions and requiremnts for 6G architecture

Document for:	Approval

Agenda Item:	20.6.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution:  Proposals for Architecure Assumptions and requirements for 6GS.

# 1	Discussion

To not start from a blank sheet when studying 6GS and 6GC, it is important to state what things from 5GC that should be inherit. The proposal below, states some of the concepts that we believe should be the corner stones when starting to study 6GC. These should be be included in the Archtecture requirments and assumptions in the TR.

# 2	Proposal

The following architecture assumptions and requirements are proposed for 6GC

-	The functional split between core and RAN in 6GS should follow the same principles as for 5GS.

-	The framework of SBA specified in clauses 7.1 and 7.2.1 in TS 23.501, shall also apply for 6GC control plane, i.e.:

-	support of NF (Network Functions) which contains one or more NF services.

-	support of registrations a service, where NFs register their NF profile and NF service consumers can discover them using a discovery service.

-	support of communication mechanisms “request-response” and “subscribe-notify”

-	support of direct and indirect communication.

-	support NF set.

-	roaming interface(s) between 6GS and 5GS shall be backwards compatible, preferably the same as roaming interface between 5GSs.

-	Due to ecosystem of exposure interfaces to external parties, the exposure interfaces to 3rd party AFs should be maintained.

-	Inherit the data structures related to the UE from 5GC. This to minimize need for data transformation between 5GS and 6GS when UE moves between systems and when roaming.

-	5G RAN and 6G RAN should have separate paging areas.

# 3.	P-CR

# ********************* start change ********************

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

The following architectural assumptions are assumed for 6GS study:

-	The framework of SBA specified in clauses 7.1 and 7.2.1 in TS 23.501 [ref], is assumed to also apply for 6GS control plane.

-	AF should be able to obtain existing services from north bound APIs (exposure) also from 6GS.

-	(Relevant) Data structures related to the UE are inherited from 5GS. Enahancements and some modifications are not precluded.

## 4.2	Architectural Requirements

Editor's Note: This clause defines the architectural requirements that serve as the foundation for the study.

The following are the architectural requiments for the 6G study:

-	The functional split between Core and RAN in 6GS shall follow the same principles as for 5GS.

NOTE 1:	As stated in the SID, the duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

-	Tracking areas in 5G RAN and 6G RAN are separate.

-	6GS should not require change in roaming agreements between VPLMN and HPLMN.

********************* end change ********************