SA WG2 Meeting #170	S2-2506935

25 – 29 Aug, 2025, Goteborg, SE

Source:	Ericsson

Title:	[WT#1.2.x, QoS] QoS Framework

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides content on WT and KI 1.2.x QoS Framework with 5GS QoS Framework as a baseline.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

5G QoS Framework provides means to enable support for differentiated connectivity where services with different QoS requirements can be provided with the adequate treatment. While the overarching framework has the right ambition level, it needs to be investigated if certain components can be improved, for example, some of which were included over generations and may be outdated and others overly complex and/or limiting. Finally, the existing QoS functionalities need to be scrutinized with respect to their adoption by the industry.

In the WT it is assumed that the 5G QoS framework is used as baseline. The prime intention of the QoS Framework is to allow for predictable (“deterministic with a certain degree of confidence”) packet treatments of the user plane with different policies and performance targets. The aim of the study is to simplify the QoS framework, preferably reduce the number of QoS parameters and QoS characteristics and improve the specification reading.

Architectural Motivation:

This WT addresses several aspects, for example

- The descriptions of the QoS Framework concepts should clearly distinguish between 6G System provider side actions from the 6G System usage perspective and avoid any ambiguity.

- The 5G QoS Framework evolved for 6G shall provide the 6G System provider with features and parameters to achieve predictable packet treatment from a 6G System usage perspective.

- Study what legacy QoS parameters (TS23.501 Cl 5.7.2) and QoS characteristics (TS 23.501 Cl 5.7.3) are needed and if any changes to the definitions of the parameters and characteristics would be beneficial.

  - 5G supports a large number of QoS parameters and QoS characteristics, which are partially depending on each other. The QoS parameter / QoS characteristics are sometimes inconsistently defined, sometimes from a provider side and sometimes from the intended usage side (i.e. application functionality). It is currently not clear, which metrics should be a QoS parameter and which a QoS characteristics.

  - Study whether the QoS parameters and QoS characteristics can be simplified and clarify the dependencies between and need for QoS Parameters and QoS Characteristics.

  - Review whether all QoS parameters and QoS characteristic are needed, how parameters & characteristics relate to each other and whether the number of parameters / characteristics can be reduced.

- Study what information, for example, QoS Parameters and QoS Characteristics, shall be provided to the RAN, UE and other 3GPP entities.

- Study what feedback, if any, and for what purpose from RAN with regards to realization of a certain packet treatment shall be supported.

**** First Change (all new text) ****

# Annex A.X WT#1.2.X: Evolution of QoS Framework for 6G

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

This WT assumes that the 5G QoS Framework is used as a baseline.

- The QoS Framework concept should clearly distinguish between the 6G System provider perspective (predictable packet treatments like target bitrates and/or packet delay) and the 6G System usage perspective by the application (like application traffic load and pattern).

- Study what functionality described by legacy QoS parameters (TS23.501 [ref] Cl 5.7.2) and QoS characteristics (TS 23.501 Cl 5.7.3) is needed (e.g. for defining 6G system targets), and if any changes/simplifications (“refactoring”) to the identified parameters/characteristics set and their definitions would be beneficial. Investigate if any new functionality is needed requiring new QoS metrics.

- Study what information, e.g., QoS parameters and QoS characteristics shall be provided to the RAN, UE and other 3GPP entities.

- Study what feedback, if any, from RAN to the Core Network with respect to realization of a certain packet treatment shall be supported, and for what purpose.

- Study how to ensure interworking with 5GS.

**** Second Change (all new text) ****

# 5.X. Key Issue #X: Evolution of QoS Framework for 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This Key Issue addresses following aspects:

- The QoS Framework concept should clearly distinguish between the 6G System provider perspective and the 6G System usage perspective by the application.

  - The 6G System provider perspective encompasses providing predictable packet treatments like target bitrates and/or packet delay.

    - Here, predictable refers to providing a deterministic packet forwarding treatment with a certain degree of confidence.

  - The 6G System usage perspective encompasses the end-point application characteristics like application traffic load and pattern.

- 5G supports a large number of QoS parameters and QoS characteristics, which are partially depending on each other, for example, QoS characteristic PER is specified as a function of the QoS characteristic Resource Type.

  - Study what functionality described by legacy QoS parameters (TS23.501[ref]  Cl 5.7.2) and QoS characteristics (TS 23.501 [ref] Cl 5.7.3) is needed (e.g. for defining 6G system targets), and if any changes/simplifications (“refactoring”) to the identified parameters/characteristics set and their definitions would be beneficial.

  - Study whether there is a merit for two metrics, i.e. QoS Parameter metric and QoS Characteristics metric, and relations between QoS parameters and QoS characteristics (e.g. “function of”) to identify any gaps, redundant information, or ambiguities.

  - Study whether there is any functionality that cannot be described by the legacy metrics in the QoS parameters and QoS characteristics that is beneficial to support.

- Study what information, e.g., QoS parameters and QoS characteristics shall be provided to the RAN, UE and other 3GPP entities.

- Study what feedback, if any, from RAN to the Core Network with respect to realization of a certain packet treatment shall be supported, and for what purpose.

- Study how to ensure interworking with 5GS.

**** End of Changes ****