SA WG2 Meeting #170	S2-2506940

25 – 29 August, 2025, Goteborg, Sweden

Source:	Ericsson

Title:	[WT#7] WT and KI to support NTN

Document for:	Agreement

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes a WT update and a KI for WT#7 (NTN).

# 1. Justifications

Architectural Motivation:

The RAN 6G SID (RP-251881) includes NTN as part its scope and it is covered with the following bullet:

“Aim at a harmonized 6G Radio design for TN and NTN, including their integration.”

It can thus be expected that a 6G radio will support NTN already from the first release. The actual outcome and final scope will however be dependent on RAN WG progress. The features from overall system and Core Network perspectives should thus match the RAN scope. Therefore, the SA2 work on WT#7 is dependent on RAN outcome.

Gap Analysis and Technical Impact:

Support for satellite access has been addressed in EPS/5GS and one of the main goals were to re-use the system features that are available for Terrestrial Networks (TN) also for NTN, with adjustments done as needed to cater for NTN-specific aspects such as e.g. differentiation of satellite vs. TN, large cell sizes, gNBs onboard satellites, etc. It is expected that a similar approach can be used for 6G, i.e. the main features would be common for TN and NTN. NTN-specific adjustments should be avoided if possible, and done only if strictly needed to address NTN-specific challenges.

In our view the 6G study for NTN should be based on the following considerations:

- Coordination and alignment with RAN are needed. The SA2 scope is dependent on RAN scope and progress.

- Features should be common for both TN and NTN in 6G, i.e. the main features available for TN in 6G should be used also to support NTN in 6G, with NTN-specific adjustments done only if strictly needed to address NTN-specific challenges.

- Same type of business relation as between TN operators, i.e. both RAN sharing and roaming, should be supported also for NTN.

- The support for NTN in the initial release of 6G should focus on the basic aspects to support satellite access, e.g. similar to 5G rel-17.

- The study of the NTN-specific challenges can start when the general 6G work has progressed a bit, since it is beneficial to discuss NTN-specific adaptations when the baseline is clearer. At the same time, NTN should be taken into account in the general work to ensure common solutions for TN and NTN. If possible, the NTN-specific challenges should be solved in similar ways as in 5G.

**** First Change ****

## Annex A.1.7. WT#7 Scope

WT#7: Study how to support 6G RAT for NTN, based on RAN decisions. The following aspects are included:

-	How 6G satellite access is supported in 6G system, considering the progress on WT#1.

-	Investigate whether and how to re-use the 5G satellite specific functions (e.g. earth fixed tracking areas, differentiation of TN and NTN) in 6G, considering progress on the overall 6G architecture.

NOTE:	The detailed scope for WT#7 will be coordinated and aligned with RAN.

It is assumed that solutions will support different orbit types (LEO, MEO, GEO).

NOTE: This WT is dependent on WT#1 for the general 6G architecture.

**** Second Change ****

## 5.X. Key Issue #X: Support for 6G satellite access

### 5.X.1 Description

The RAN Study Item Description includes an objective to “Aim at a harmonized 6G Radio design for TN and NTN, including their integration.”. This KI studies the corresponding system level and core network aspects.

Commonalities between TN and NTN should be maximalized, i.e. the main features available for TN in 6G should be used also to support NTN in 6G, with NTN-specific adjustments done only if strictly needed to address NTN-specific challenges.

NOTE: Coordination and alignment with RAN are needed.

This KI will study:

-	How 6G satellite access is supported in 6G system, considering the progress on the overall 6G architecture:

-	Support for satellite access in the 6G system.

-	Support for session continuity between TN and satellite access.

-	Whether and how to re-use the 5G satellite specific functions (e.g. earth fixed tracking areas, satellite specific RAT types etc) in 6G, considering progress on the overall 6G architecture:

-	Specific handling due to quasi-earth-fixed (LEO) cells, if needed

-	Location verification

-	Support for policy/charging differentiation between TN and satellite access

Features that require CN functionality on-board the satellite are not included in the scope.

**** End of Changes ****