SA WG2 Meeting #170	S2-2507014

25 – 29 Aug, 2025, Goteborg, SE

Source:	Huawei, HiSilicon

Title:	[WT#4] Key Issue proposal for integrated sensing and communication

Document for:	Approval

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Scope and Work Tasks of 6G ISAC are discussed and proposed.

- Summary

Aligned with the Working Task #4 of Integration of Sensing and Communication in 6G SID, this paper analyses the 6G ISAC use case/requirement and the sensing capability supported by 5GA network in R20, then proposes to leverage multiple sensing modes and integrate heterogeneous data sources for sensing result calculation to address 6G demands as in the proposed KI description.

1. Justifications

1.1	Background and motivation

1.1.1	State of the arts

Sensing technology is redefining the role of telecom operators, enables them to evolve beyond connectivity by capturing real-world data (environmental, spatial) through network infrastructure. This evolution creates new avenues for innovation and revenue generation. Operators can now monetize actionable insights – optimizing urban traffic via anonymized mobility patterns, predicting infrastructure damages, or enabling smart logistics. These capabilities position operators as strategic partners across industries like logistics, energy, and smart cities. Simultaneously, sensing enhances network efficiency through real-time monitoring and predictive maintenance.

To this end, 3GPP initiated the study of 5G Advanced (5GA) sensing in RAN1 starting with Rel-19. Architecture-related investigations were later expanded in Rel-20. Within SA2, the work focuses on the 5GA sensing framework, particularly on architectural design aspects such as service authorization and revocation, discovery and selection of sensing entities/functions, sensing data collection and result exposure, and configuration parameter provisioning.

However, the Rel-20 specification narrows its scope to gNB-based mono-static sensing for UAV target use cases, as outlined in RP-251861 and SP-250809 during TSG #108 as follows:

In SP-250809: The study item aims at investigating the function enhancement, end to end service operations and procedures to support Integrated Sensing and Communication with the considerations of a sub-set of use cases (e.g. commercial, automotive, public safety and emergency services) based on the SA1 requirements. In this study, the scope is gNB-based sensing, aligned with Rel-20 5G-A RAN ISAC study scope.

In RP-251861: Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

gNB-based sensing modes rely exclusively on the gNB to manage the air interface signal. In these modes, the gNB transmits the sensing signal and receives the reflected echo signal from objects. In gNB-based monostatic sensing mode, the transmitter and receiver are co-located at the same gNB.

1.1.2	Motivation

Current Rel-20 5GA network architecture supports only gNB-based mono-static sensing for UAV-specific use cases and has limitations to address the diverse requirements of emerging 6G applications. To promote vertical industries through sensing capability, it is imperative to support ISAC system via targeted 6G technical advancements.

The use cases and gaps are further clarified in clause 1.2.

1.2	Gap Analysis

1.2.1	Use Case: Environment reconstruction

1.2.1.1	Description

The Environmental object reconstruction is the way to generate the objects within environment, it creates detailed digital representations of physical surroundings. It enables applications across smart environments (factories, homes, cities, transportation) by efficiently providing rich, low-cost environmental data for both static objects (like buildings or machinery) and dynamic targets (like vehicles or robots), thereby enhancing automation, safety, and operational efficiency. Typical applications including:

-	Smart Transportation: Essential for autonomous driving safety/comfort, requiring advanced moving target detection, tracking, and micro-feature perception (e.g., micro-Doppler, precise classification) beyond rough localization;

-	Smart City: Enhances digital twinning and dynamic city modelling beyond initial camera-based approaches;

-	Smart Home: Enables more use cases (e.g., fall detection) with better reliability for improved consumer experience.

By leveraging 3GPP Integrated Sensing and Communication (ISAC) technology via the non-invasively collecting sensing data alongside communication signals, one can characterize finer object details. Reconstruction accuracy is measured by positioning/velocity estimation, missed detection, and false alarm rates over object segments.

KPI for the environment reconstruction is as the following table 1.2.1.1-1 (from TR 22.870 table 7.5.6-1) shows. For positioning estimation sub-meter accuracy is required.

Table 1.2.1.1-1 KPIs for environment object reconstruction (From TR 22.870 Table 7.5.6-1:)

| Scenario | Object Type | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Spatial resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Environment object reconstruction | Building (note 1) | 95 | [0.5-5] (note 2) | [0.5-5] (note 2) | N/A | N/A | [0.5] | N/A | [10,000-600,000] | [10-60] | [1-5] | [1-5] |
|  | Vehicle (note 1) | 95 | 0.5 | 0.5 | 1.5 | N/A | [0.5] | [0.5] | 100 | 0.1 | 5 | 5 |
| NOTE 1: a sensing target in above KPI refers to a [segment/part] of the target object to be detected and/or tracked, whereas the size of each part is comparable to corresponding spatial resolution of object reconstruction. The percentage of missed detection/false alarm represent missed/falsely reconstruction of parts of the object statistically. NOTE 2: Considering a variety of dimensions, shapes, and functionalities of urban buildings, a range of KPI values are needed to measure and provide the flexibility of reconstruction accuracy. |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.2.1.2	Gaps

The sensing mechanisms defined in 5GA have limitations to meet the requirements of environment reconstruction:

-	As highlighted in Clause 1.1, current 5GA supports only gNB-based mono-static sensing.

-	The gNB-based mono-static sensing has limited sensing coverage due to the NLOS condition.

-	For environment reconstruction in use cases such as smart transportation and smart cities, detected objects have stringent accuracy requirements (e.g., object size, location).

Figure 1.2.1.2-1: Multiple sensing modes/entities cooperation for environment reconstruction

As shown in the Figure 1.2.1.2-1, to address these limitations, 6G Sensing can leverage additional sensing sources, i.e., UE related 3GPP sensing technologies, optionally complemented with other non-3GPP access network sensing technologies (e.g., the IEEE 802.11bf) and information which is derived from non-3GPP Sensors such as Camera, to fulfil the new requirements emerged in 6G era.

1.2.1.3	Proposals

To improve the sensing performance, various sensing data sources should be considered:

-	6G System shall support all possible sensing modes, including RAN-based sensing, UE-based sensing and RAN-UE coordinated sensing for various use cases;

-	6G System should support the reporting/fusion of the 3GPP sensing data provided by RAN/UE/AF sources to achieve desired accuracy and reliability;

-    6G System should support reporting/fusion of non-3GPP sensing data from non-3GPP Sensors (e.g., Camera);

-	6G System should support the reporting/fusion of non-3GPP sensing data derived by non-3GPP access network (e.g. IEEE 802.11bf).

1.3	Technical Impacts:

The following impacts are foreseen:

-	Overall system architecture:

-	Mechanism of sensing configuration parameters provisioning to RAN/UE;

-	Mechanism of sensing data report by RAN/UE;

-	Capabilities for NF(s) designed for sensing (e.g., Sensing Control Function, Sensing Processing Function), to support the data fusion of multiple sensing sources;

-	Interaction between network functionalities/services:

-	Interfaces and protocols between NF(s) designed for sensing and other NFs/AF/UE;

NOTE:	6G Sensing may use data framework which will be discussed in WT#5.

2. Text Proposal

It is proposed to capture the following changes in TR 23.801-01.

**** First Change ****

Annex A.1. WT#1 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT.

-	Control plane: Support the interactions between the Sensing functionalities and RAN nodes for sensing configuration, and among the Sensing functionalities for sensing service control;

-	Exposure: Support the exposure of the Sensing Result to the sensing service consumers;

-	NAS: Support the interactions between UE and Sensing functionalities for sensing service;

**** Second Change ****

Annex A.4. WT#4 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT.

-	The selection of the sensing mode(s);

-	Authorization of the sensing service consumer and UE;

-	The discovery and (re-)selection of the sensing entities;

-	Configuration of the selected sensing entities;

-	Aspects regarding 3GPP sensing data collection, and sensing result calculation;

-	Exposure of the Sensing Result;

-	Sensing continuity;

-	Collection of other types of data from non-3GPP Sensors and data fusion;

**** Third Change ****

Annex A.5. WT#5 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT.

6G Sensing (WT# 4) may use data framework defined in this WT.

**** Fourth Change ****

5.X. Key Issue #X: Sensing

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

5.X.1	Description

This Key Issue focuses on the 6G architecture, enhancing functionalities, and optimizing end-to-end service operations and procedures to support diverse 6G use cases and requirements for Integrated Sensing And Communication, especially for leveraging multiple sensing modes and integrating heterogeneous data sources for sensing result calculation. The following aspects will be studied:

-	How to support the system architecture and procedures used to accommodate sensing services for 6G, especially:

-	The selection of the sensing mode(s) (i.e., RAN-based sensing, UE-based sensing and RAN-UE coordinated sensing);

-	Authorization of the sensing service consumer is authorized to access the Sensing Result;

-	Authorization of the UE as the sensing entity;

-	The discovery and (re-)selection of the sensing entities;

-	Configuration of the selected sensing entities;

-	Aspects regarding 3GPP sensing data collection, and sensing result calculation based on the collected sensing data;

-	Exposure of the Sensing Result to the sensing service consumer.

-	Sensing continuity for the moving of the Object(s) and the UE(s) as the Sensing entity(ies);

-	Collection of other types of data from non-3GPP Sensors (e.g., Camera) and making data fusion with the 3GPP sensing data.

NOTE 1:	Within the scope of the WT#4 description, the non-3GPP Sensors are intended as devices which are not using 3GPP based Sensing technologies to derive information which can be integrated in the sensing processing to provide the 3GPP Sensing Results. The exact definition will be determined during the study.

NOTE 2:	Whether the 6G Sensing considers the information derived by the sensing performed on Non-3GPP Access network, such as based on IEEE 802.11bf, will be determined when the framework for supporting the non-3GPP access network in WT #1.3 is considered stable.

NOTE 3:	6G Sensing may use data framework which will be discussed in WT#5.

NOTE 4:	Functionalities related to RAN nodes will be defined together with the RAN working groups.

**** End of Changes ****