SA WG2 Meeting #170	S2-2507098

# Goteborg, Sweden, 25 – 29 August 2025                       (revision of S2-25xxxxx)

Source: 	CATT

Title: 	KI#1, New sol: AIoT service support for T2 RRC-based option

Document for: 	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes a new solution for KI#1.

1	Discussion

1) WT#1 of FS_AmbientIoT_Ph2_ARC SID (SP-250834)

WT#1: Study the support of AIoT services in Topology 2, including:

-	Study and investigate how to support AIoT services under the RRC-based option for UE Reader connectivity.

NOTE 3:	Rel-19 TR solution and interim conclusion should be used as basis for rel-20 work. Solution submitted to rel-20 need to align with rel-19 TR interim conclusion. No rel-19 AIoT Device impact from this WT is expected.

2) Rel-19 Interim conclusions

TR 23.700-13 v19.0.0 captured the following interim conclusions for Topology 2.

8.1.3	Architecture to Support Topology 2

8.1.3.1	General

The following principles apply:

-	Architecture: Two architecture options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

-	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

Editor’s note:	Additional subscription information, e.g. validity information, for the UE Reader is FFS.

-	UE Reader subscription information is available to AMF and AIoTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Editor’s note:	Whether and how to enable authorization to the UE is FFS.

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g. taking into account assistance information received from the AIoTF) from the gNB.

NOTE 1:	Assistance information is further clarified in other clauses.

NOTE 2:	The details of UE Reader radio resource management will be defined by RAN WG2.

8.1.3.2	User-plane option

…

8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIoTF. The related protocol stack is shown in Figure 8.1.3.3-1.

Editor's note:	How addressing works for UL for Option B is FFS.

Editor's note:	Further details how the RRC option works are FFS.

Figure 8.1.3.3-1: Protocol Stack for the RRC option

8.1.3.4	Interim Conclusion for authorization

The following interim conclusion are agreed for the UE Reader authorization in Topology 2:

-	The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader.

-	The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader.

Editor's note:	How to authorize the UE Reader will be determined later.

This paper proposes a new solution for KI#1 based on Rel-19 interim conclusions.

2	Proposal

It is proposed to update the TR 23.700-30 as followings.

# * * * * Start of change (All Text New) * * * *

6.X	Solution #X: AIoT service support for T2 RRC-based option

6.X.0	High-level Solution Principles

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

Documentation of more than one solution from the same company for the same KI is allowed but it shouldn't be expected that all of them will be documented in the TR.

This solution proposes Inventory Procedure and Command procedure for Topology 2 RRC-based option, based on the interim conclusions of TR 23.700-13 [xx] and the related procedures defined for Topology 1 as described in TS 23.369 [xx].

6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Clause(s) may be added to capture details.

This solution is based on the Inventory Procedure and Command procedure as defined in TS 23.369 [xx].

Figure 6.X.1-1 describes the Protocol Stack for the RRC option.

Figure 6.X.1-1: Protocol Stack for the RRC option

6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

6.X.2.1	Inventory Procedure

Figure 6.X.2.1-1 describes the Inventory Procedure.

Figure 6.X.2.1-1: Inventory procedure

1.	UE performs registration procedure and includes the UE Reader capability in the Registration Request message to AMF. The AMF determines whether the UE is authorized to act as UE Reader based on UE Reader capability and subscription data from UDM, and provides authorization information to UE and NG-RAN.

2.	The AF invokes Nnef_AIoT_Inventory (AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], UE Reader list) service operation request to the NEF.

3.	The NEF may further authorize the AF request. The NEF determines the Target Area information from the External Target Area information, and selects one or multiple AIOTF(s) to handle the request.

4.	The NEF invokes the Naiotf_AIoT_Inventory (AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], UE Reader list) service operation towards to the selected AIOTF(s).

5.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization. The AIOTF generates a Correlation ID and determines assistance information. The AIOTF sends the AIoT Inventory Service Response to the AF via the NEF.

6.	AIOTF performs UE Reader Selection, based on, e.g. Target Area information, AF provided UE Reader list, or stored UE Reader list.

7.	The AIOTF sends Namf_AIoT_MessageDelivery message (NGAP AIoT information, NG-RAN ID, AIOTF ID, Message Type for NGAP AIoT information) to the AMF and further to NG-RAN.

Inventory Request message (NGAP AIoT information) includes the Correlation ID, the AIoT Identification Information to be included in the paging message, assistance information, and UE Reader list.

7a.	AIoT radio resource allocation for UE Reader.

8.	NG-RAN sends DL RRC message (Inventory Request) to UE Reader.

9-10. UE Reader sends Inventory Response to AIOTF via the NG-RAN.

11.	UE Reader(s) will execute the inventory operation. If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT identity.

12.	UE Reader sends an UL RRC message (AIOTF ID, NGAP AIoT information) to NG-RAN. UE Reader sends one or more Inventory Report (NGAP AIoT information) messages to the NG-RAN including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

13. NG-RAN sends an NGAP message (AIOTF ID, NGAP AIoT information) to an AMF and further to AIOTF.

14.	The AIOTF reports the progress of the AIoT inventory request to the NEF, and the NEF informs the AF of the outcome of the AIoT_Inventory request.

6.X.2.2	Command Procedure

Editor's note:	The Command Procedure will be added.

6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

# * * * * End of Changes * * * *