3GPP TSG-WG SA2 Meeting #170	S2-2507149

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[WT#3, AI Inference Offloading Service], Support of AI Inference Offloading Service in 6GS

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: This document explores the opportunities for MNO to offer AI inference offloading service in 6GS. AI inference offloading service is the mechanism offered by the mobile network in order to execute fully or partially an AI inference processing for an APP (provided by third party or the mobile network operator itself or for UE) in 6GC. This document provides the corresponding justification and lists the related WT scope and KIs to study how to enable such new service.

- Summary

AI inference offloading service is the mechanism offered by the mobile network in order to execute fully or partially an AI inference processing for an APP (provided by third party or the mobile network operator itself or for UE) in 6GC. AI inference offloading service through coordination among UE, core network and APP provider facilitates meeting the AI inference requirements (e.g., quality and latency) and improve the user experience.

This contribution explores opportunities for MNO to offer AI inference offloading services, and propose the related Key Issues to study how to enable such new service.

Clause 2.1 illustrates several benefits of performing the offload at the MNO. Firstly, AI inference offloading to MNO allows the coordination between transmission resource and AI inference processing resource as they are both controlled and scheduled by MNO. Secondly, AI Inference specific parameters (e.g., TTFT, TPOT, batch size, etc) can be taken into account by the MNO when coordinating scheduling of transmission and AI Inference processing. Thirdly, the mobile network can provide functionality to enable the splitting of the AI Inference between the UE and the 6G core taking into consideration several factors (e.g., transmission latencies, the processing capabilities etc). By virtue of the above three benefits, we argue that AI inference service offloading can improve the quality and speed of the AI Inference and the user experience.

Clause 2.2 describes some drawbacks of offloading AI inference task to public cloud compared with offloading to MNO. For example, overprovisioning of resources are required for both transmission and AI inference processing to guarantee the required latency since transmission and AI inference processing are controlled by MNO and public cloud independently. In addition, no awareness of UE mobility would lead to non-optimal application server selection and relocation, leading to de-tour. Moreover, it is also difficult for the public cloud to integrate the information inside network (e.g. sensing data) to help improve the accuracy of AI inference. Furthermore, there is no possibility to adjust the split between UE and 6G core based on the available transmission and processing status.

In summary, MNO offering the AI inference offloading service results in increased number of users that can be served simultaneously, better quality of experience when using a supported APP and potential new revenues to MNO.

Therefore, key issues to support AI inference offloading service are proposed in this contribution. These key issues aim to investigate what AI inference task related information is required by the mobile network to support such service and study how to coordinate the transmission and the AI inference processing to fulfil E2E performance requirement.

- Discussion

Mobile AI has been broadly applied in different areas such as AI glasses for real-time translation assistance or AI phone for real-time video stream recognition to assist the blind to recognize road, bus or train etc.

Mobile Terminal Resource Limitations

It is worthwhile to note that most AI applications are powered by large language model (LLM), especially multimode AI application and embodied AI robots. For such applications, normally, the larger size of AI model, the better the performance (e.g., AI Inference quality) that can be achieved. However, the support of larger size of AI model requires an increased memory size, computing capability and power consumption which may be challenging to support in current mobile devices such as smartphones, AR/VR glasses etc.

Based on the constraints mentioned above, the AI inference service can be more efficiently done by offloading complex computational tasks from the mobile device to a server.

Observation 1: due to the limited resources (e.g. battery, CPU, memory) and large size AI model, it is a challenge to perform complex AI Inference tasks locally on a mobile device.

Proposal 1: The mobile network supports the offloading of AI Inference services from a mobile device to improve the overall performance.

2.1	Motivation to enable AI inference offloading service

Based on observation 1, offloading AI task to server side is becoming increasingly necessary. This may result in MNOs potentially developing new businesses whereby they offer AI inference offloading services inside the mobile network, allowing application clients (e.g., UEs) to offload the AI Inference task into the mobile network. Following are several possibilities:

-	Application providers deploy their AI APP (including AI model) inside the mobile network where the AI inference will be processed.

-	MNO owns a generic AI model and offer the AI inference processing to the clients (i.e. UEs) from multiple applications

-	Terminal manufacturer may offload its AI model inside mobile network which assists the UE to perform the AI Inference processing

When the UE requests for an App that the AI inference task execution is offloaded to the mobile network. The MNO can control both, the resources for data transmission and AI inference processing. Hence, the MNO can ensure the E2E latency performance, resulting in an increased user experience. In addition, the MNO can also monetise the use of the mobile network for AI Inference processing.

In the following sub-clauses, we show some benefits of the operator supported AI inference offloading services.

2.1.1	Benefit 1: achieve better E2E user experience with coordinated resource allocation for transmission and AI inference processing

The latency for AI Inference services is a key parameter affecting user experience for some specific services such as for real-time video stream recognition, real-time VR applications on gaming, translation, autonomous driving, controlling UAV, among others.

(A) AI phone for object recognition. When a blind person needs to take a bus to destination A, he can use AI phone to take video stream capturing the environment information around him and send the video stream to the server. Based on the video stream taken from the AI phone, the server performs AI inference to recognize the object in the video stream e.g., bus station, and the incoming buses to the bus station. The server informs the blind person of the recognized object in real-time so that when the target bus comes into the bus station, the blind person can know that he should get in the bus.

The images are fed, for example into a large vision-language model (VLM) which is expected to understand the input information (such as images, or additional multi-model information). In addition, in order to improve AI inference accuracy, environmental information e.g. sensing data or positioning data can be acquired as further input for the AI inference.

Figure 2.1.1-1 AI Model used for the AI phone in the use case

(B) Real-time AI-Assisted VR Translation. A user (from country A) is on a trip to country B, but not capable of understanding country A’s language. The user is wearing VR glasses while walking through the streets and looking around the area and trying to understand directions or find places. The VR application assisted by LLMs or NN based AI inference processes allows to display country A’s language text translation over country B’s language text in the user’s field of view. Due to the user’s constant change of field of view, the AI inference process supporting the real-time translation requires the fulfilment of stringent latency thresholds.

Figure 2.1.1-2 AI-Assisted VR Real-Time Translation

(C) Other example of use cases with AI Inference subjected to stringent requirements have been defined in SA1 specification TS 22.870 such as:

-	the end-to-end AI for connected cars (Clause 6.2), where the passenger requests at real-time from LLMs the information about the environment surrounding the car;

-	control system and UAV swarms to complete AI based tasks (Clause 6.13), where the usage of cameras and UAVs support the actions of real-time detection of environment conditions;

-	a target object detection task supporting car collision scenarios (Clause 6.14).

E2E AI Inference Latency Aspect Relevant to all Scenarios

In all examples above the E2E latency comprises three main parts: the round-trip transmission latency between the AI hosted in the device, such as the AI phone and the AI in the server, the AI inference time in the AI server (e.g. elaboration of the input and the action decisioning inference time and the AI inference part on the device.

E2E latency=Ttransmission+TserverAI Process +TdeviceAI Process

If all parts of the AI Inference processing are offloaded to the server, the TdeviceAI Process equals to 0.

For the above example the E2E latency needs to be in the range of 200ms [4] which is equivalent to human response time.

The transmission delay is related with the allocated transmission resources (e.g., allocated bandwidth), while the AI inference processing delay is related with the capability of AI Inference processing node.

If the MNO offers the AI inference offloading service, the network can coordinate the resources allocated for transmission and the resources for AI inference processing, e.g., discover and/or adjust the resources allocated for transmission taking into account the AI inference processing and performance to guarantee the desired E2E latency.

Observation 2: AI Inference service performance should consider the E2E latency including the transmission delay and AI inference processing delay which is essential to provide a good user experience. If the MNO offers the AI inference offloading service, the resources for transmission and AI inference processing can be coordinated to ensure the desired E2E latency.

2.1.2	Benefit 2: achieve lower E2E latency with joint scheduling for transmission and AI inference processing

The E2E latency can be affected by different factors. If the AI inference processing is performed by 6GC, the 6GC can further perform dynamically fine-granularity joint control for the transmission scheduling and the AI inference processing, the performance of AI Inference service can be further optimized and increase the resource utilization (e.g., serving more users in the same region).

Taking LLM as an example, the AI Inference Processing Latency, indicating how long it takes from submitting a query to generating the full response, is given by the sum of the time to generate the first output token (i.e., Time To First Token, TTFT) and the duration of the process from the first token to the final token for the first response. The latter one is represented as TTPOT*(N-1), where TTPOT is the time per output token and the N is the number of output token for the first response.

TserverAI Process =TTTFT+TTPOT*(N-1)

The overall process is also influenced by the number of requests fed to an AI Model one time to generate predictions (inferences), called Batch Size. This parameter is used to optimize the hardware utilization (e.g., GPU parallelism) by processing multiple inputs concurrently, balancing latency, throughput, and memory constraints [3]. The batch size for AI Inference processing is usually configured by the platform enabling the AI Inference service. Since the batch size determines the number of requests submitted for the AI Inference processing, different arrival pattern of the received requests may eventually affect the time when to submit the prompts for AI Inference processing. Therefore, the AI inference processing latency will be affected and may further have impacts on user experience.

In short, the TTFT can be considered as the value of reactivity. The TPOT can be considered as the value of inference speed. The batch size can be considered as the number of parallel processing requests.

Observation 3: If the transmission scheduling and AI service processing scheduling can be coordinated to make joint decision (e.g. considering the TTFT, TPOT, batch size, number of received prompts, etc), the user experience of AI Inference service can be improved.

Proposal 2: AI Inference specific parameters (e.g. TTFT, TPOT, batch size) can be shared between transmission node (e.g., RAN) and AI Inference processing node to have coordinated scheduling between transmission and AI Inference processing to guarantee the performance.

2.1.3	Benefit 3: support of optimization on split AI inference

The AI Models such as DNN [1] and LLMs [2] are designed in a modularized way in order to enable mechanisms that can support the optimization of the AI Inference processing. This modularization allows to split an AI model, e.g. a Neural Network, into two or more parts, allowing a first part to be executed at one AI inference processing node, while a second part can be executed at another AI inference processing node. The different layers of an NN differ in terms of their computational complexity (e.g. expressed as the number of floating point operations (FLOPs)) and in terms of their input/output data size (some blocks reduce the output data compared to the input, while others may drastically increase the output data amount). A smart selection of the split location minimizes the E2E inference latency, as it allows to control the transmission latency and AI inference processing latency: The NN can be split so to optimize the E2E latency, by jointly considering i) the data to be transferred between the AI inference processing nodes (i.e. expected transmission latency), ii) the nodes’ processing capabilities and resources usage (i.e. the expected processing latency). Only the mobile network is aware of the underlying network conditions of both transmission and AI inference processing. Hence, only the mobile network can do such an optimal split decision and AI inference processing task allocation.

In addition to optimizing for E2E latency, the split decision may also minimize UE battery usage, minimize the overall network traffic load, or minimize the overall energy consumption for the whole inference process. Finally, more flexible scenarios can be supported by using split AI Inference. In case when the input data cannot be transferred for AI inference offloading, the first layer(s) of the NN can be executed at the UE. Thus, there is no strict requirement for UE to transfer all input data to network and only the remaining NN (all sub-sequent layers) are offloaded. The mobile network can control the split, i.e. in such a way to minimize the transmitted data.

At the end of this document, there is background support information which provides some detailed consideration on the split AI inference.

Observation 4: AI Inference processing can be split into separable blocks of AI models which can be assigned to different entities. This enables to fine tune several aspects of the AI inference (e.g. transmission delay, capabilities supported by UE, power consumption on UE side etc).

Proposal 3: The mobile network can control and make decision on the splitting the AI Inference processing blocks considering the flexibility supported by the AI application (e.g. whether and in how many and which processing blocks can be split) and the selection of the nodes considering different factors (e.g. transmission latencies, the processing capabilities and power consumption on UE side, the real-time AI inference performance feedbacked from the UE etc) to optimize the execution of the AI inference service.

2.2	Comparison with offloading AI inference service to public cloud

If the AI Inference services are offloaded to the public cloud which cannot be controlled by network, following drawbacks are observed:

-	The transmission delay and AI Inference processing delay are controlled by two independent systems. Transmission delay is controlled by mobile network, while the Application Server in public cloud can only control the AI Inference processing delay. In order to achieve the end-to-end performance, each system should strictly guarantee their own delay respectively by reserving enough resources. In other words, overprovisioning of resources is needed to guarantee the required latency, which cannot utilize resources effectively and will limit the system capacity (the number of serving users) for AI Services.

-	Public cloud is not aware of UE mobility. This means the transmission path switching due to mobility and the AI inference processing will be handled disjointly, lead to non-optimal routing (e.g. de-tour). For instance, the mobile network controls the procedures for mobility registration update, PDU session modification, while the Application in public cloud controls the procedures for EAS selection or EAS context relocation procedure (if MEC is used). This disjoint handling may not choose the best option from E2E system view. However, MNO has full awareness of UE mobility, the transmission capacity and the AI inference processing capacity, it can select the optimal way to ensure the E2E latency performance.

-	It is difficult for the public cloud to integrate the information inside network (e.g., sensing data around the UE) to further help improve the accuracy of AI inference (e.g. as mentioned in the above use case of AI phone).

-	The optimal split and execution of an AI-model not only depends on the processing capability of UE and server, but also on the underlying network conditions. If the AI split processing is not controlled by the network, the split inference cannot be realized in an optimized way. Due to its domain knowledge, only the network can do an optimal split decision and AI inference processing task allocation.

-	The latency on N6 interface is neither controlled nor known by mobile network and by AI Inference processing node in the public cloud, leading to unknown QoE.

-	If MNO offers AI inference offloading service, UE can just negotiate with MNO for dynamically offloading its AI inference applications with guaranteed E2E latency. On the contrary, if the AI inference processing is offloaded to the public cloud, in order to ensure the E2E latency, not only the UE shall negotiate with MNO regarding the transmission requirement, but also the public cloud shall negotiate with APP provider regarding the AI inference processing requirement, which make the service flows complicated.

Based on the above analysis, if MNO can offer AI inference offloading service inside mobile network to enable the network perform coordination between the AI Inference processing and communication, it is more flexible for MNO to guarantee the E2E latency to improve the user experience. In addition, with such coordination, the number of users that can be served simultaneously can be increased maintaining the same level configuration of computing resources. In short, offering AI inference offloading service offered by MNO not only brings new revenues to MNO but can also enables APP service provider to provide high quality of AI Inference service with relatively low cost.

2.3	Summary of Proposals

The Summary of the proposed Key Issues listed in the above clauses.

From above analysis as described in clause 2.1, benefits to users, MNO and APP providers are foreseen to introduce AI inference offloading service. Therefore, it is suggested to study how to enable the AI inference offloading service.

AI inference offloading service is the mechanism offered by the mobile network in order to execute fully or partially an AI inference processing for an APP (provided by third party or the mobile network operator itself or for UE) in 6GC. This includes the coordination among UE, core network and third party to fulfil the AI Inference task requirements to handle dynamic changes affecting the AI Inference service in order to meet the expected user experience.

Study the architecture to support AI inference offloading service to MNO considering the following issues:

-	How does UE request the AI inference offloading service for an application?

-	What is the AI Inference task related application-level information (e.g., desired E2E latency, TTFT, TPOT, batch size, AI model layers etc.) that is requested from the mobile network?

-	How to perform the coordination between the network transmission and the AI inference processing to fulfill the AI inference task requirement? Following aspects may be considered:

-	How to share the information between RAN and AI inference processing node to have timely coordinated scheduling between RAN transmission and AI inference processing.

-	The information/mechanisms needed for i) determining an optimal split of AI inference processing between AI inference processing nodes and ii) for instructing the AI inference processing nodes to execute the assigned AI inference processing.

-	How to adjust the split AI inference processing dynamically according to different factors (e.g., transmission latencies, processing capabilities, real-time feedbacked from UE) to ensure the E2E AI inference task requirement.

2.4	Reference

[1] M. G. Spina et al., "In-Network Computing and Split-AI in 6G: Enablers and Proof-of-Concept Studies," 2024 3rd International Conference on 6G Networking (6GNet), Paris, France, 2024, pp. 135-143

[2] M. Zhang, X. Shen, J. Cao, Z. Cui and S. Jiang, "EdgeShard: Efficient LLM Inference via Collaborative Edge Computing," in IEEE Internet of Things Journal, vol. 12, no. 10, pp. 13119-13131, 15 May15, 2025

[3] Mastering LLM Techniques: Inference Optimization, https://developer.nvidia.com/blog/mastering-llm-techniques-inference-optimization/

[4] Research Article: Stivers T, Enfield N J, Brown P, et al. Universals and cultural variation in turn-taking in conversation[J]. Proceedings of the National Academy of Sciences, 2009, 106(26): 10587-10592.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *

Annex A.3. WT#3 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	Define, support and enable AI inference offloading service in 6G architecture.

-	Define the required AI Inference task related application-level information to support AI inference offloading service.

-	Support the coordination between the network transmission and the AI inference processing to fulfill the requirement of AI inference offloading service, including i) the coordinated scheduling between RAN transmission and AI inference processing and ii) mechanisms for optimal split of AI inference processing

Annex A.1. WT#1 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	NAS: Support interaction between UE and NF supporting AI inference offloading service.

-	Exposure: Support exposure framework for invoking AI inference offloading service.

-  New Interface: Support interaction among NFs to support AI inference offloading service.

Annex A.6. WT#6 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	If computing service is proposed, the computing service needs to provide computing resource for AI inference offloading service.

# * * * * Second change * * * *

5.X. Key Issue #X: AI Inference Offloading Service

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The Summary of the proposed Key Issues listed in the above clauses.

AI inference offloading service is the mechanism offered by the mobile network in order to execute fully or partially an AI inference processing for an APP (provided by third party or the mobile network operator itself or for UE) in 6GC. This includes the coordination among UE, core network and third party to fulfil the AI Inference task requirements to handle dynamic changes affecting the AI Inference service in order to meet the expected user experience.

Study the architecture to support AI inference offloading service to MNO considering the following issues:

-	How does UE request the AI inference offloading service for an application?

-	What is the AI Inference task related application-level information (e.g., desired E2E latency, TTFT, TPOT, batch size, AI model layers etc.) that is requested from the mobile network?

-	How to perform the coordination between the network transmission and the AI inference processing to fulfill the AI inference task requirement? Following aspects may be considered:

-	How to share the information between RAN and AI inference processing node to have timely coordinated scheduling between RAN transmission and AI inference processing.

-	The information/mechanisms needed for i) determining an optimal split of AI inference processing between AI inference processing nodes and ii) for instructing the AI inference processing nodes to execute the assigned AI inference processing.

-	How to adjust the split AI inference processing dynamically according to different factors (e.g., transmission latencies, processing capabilities, real-time feedbacked from UE) to ensure the E2E AI inference task requirement.

# * * * * End of changes * * * *

Background Support Information: State of the art

1	Terminology and concept

This clause provides concepts and terms used in AI Inference as introduced in [1] considered in the paper.

Token: Token refers to the basic data unit processed by the AI model. It can be a word, character, part of an image, or part of speech, etc.

Tokenization: Tokenization is the process of converting input data into corresponding tokens. This process is domain dependent i.e., for NLP where the transformer is widely used, the text is converted into token using byte pair encoding. In computer vision, the image is first dissected int non overlapping patches and then linearly projected to obtain token. In case of speech processing, the speech signal is converted into spectrogram which is then treated as an image and does the tokenization in a similar manner to that of images.

Time to First Token (TTFT): TTFT is the time it takes from submitting the query (i.e., prompt) to receiving the first token (if the response is not empty). This metric shows how long a user needs to wait before seeing the model’s output. TTFT generally includes both query queuing time and prefill time. The longer the prompt, the larger the TTFT. This is because the attention mechanism requires the whole input sequence to compute and create the so-called key-value cache (i.e., KV-cache), from which point the iterative generation loop can begin. Additionally, a production application can have several queries in progress. Therefore, one query’s prefill phase may overlap with another query’s generation phase.

Figure 1-1: Diagram for TTFT

Time Per Output Token (TPOT): the average time between consecutive tokens.

Generative Time: the duration from the first token received to the final token received. It can be calculated as TPOT*(N-1), where the N is the total number of tokens for the full response.

AI Inference Processing Latency: also called as Total Time to Generation, indicating how long it takes from submitting a query to receiving the full response. It is the sum of TTFT and Generative Time. In other words, the AI Inference Processing Latency is calculated as:

TAI Inference=TTFT+TPOT*(N-1)

Figure B.1-1: Diagram for AI Inference Processing Latency

Batch Size: the number of queries fed to an AI Model at once to generate predictions (inferences). It optimizes hardware utilization (e.g., GPU parallelism) by processing multiple inputs concurrently, balancing latency, throughput, and memory constraints. The batch size for AI Inference processing is usually configured by the platform enabling the AI Inference service (e.g., vLLM)

-	Throughput: Larger batches maximize hardware utilization (e.g., GPU parallelism), increasing inferences per second.

-	Latency: Smaller batches (e.g., 1) reduce TTFT.

-	Memory: Larger batches require more VRAM. Exceeding hardware limits causes out-of-memory errors.

2	Example of using Batch Size for AI Inference Processing Optimization

When LLM models are used, the TTFT can be affected by parameters of the deployed AI model, e.g., batch size. The batch size defines the number of samples the LLM processes at once. There are three main scheduling methods used for LLM: Static Batch, Dynamic Batch, and Continuous Batch. In the static batch scenario, the prompts from users are packaged and submitted for AI Inference processing when the number of received prompts reaches the maximum batch size. For the dynamic batch, the maximum batch size and maximum waiting time are configured. When the waiting time reaches the configured maximum waiting time or the number of received prompts reached the maximum batch size, the received prompts are submitted for AI inference processing. When the LLM uses continuous batch, the received prompts can be submitted for AI Inference processing if only when the number of submitted prompts does not exceed the maximum batch size. Since the number of batch size (i.e., the number of prompts) submitted for the AI Inference processing can affect the Inference delay which may have impact on the TTFT, different scheduling methods and the arrival pattern of the prompts to be submitted for AI Inference processing may eventually affect the TTFT and further have impacts on user experience.

Taking Static Batch scheduling method as an example. There are two AI Inference services (service 1 and service 2) where the configured batch size of both AI Inference services is 10. Currently, assume that the received prompts for service 1 are 9, while the received prompts for service 2 are 3. Since both of them do not reach the configured batch size of 10, the received prompts cannot be submitted for AI Inference processing. In the next time slot, there are prompts for both services are to be sent. However, due to traffic congestion, only one prompt can be sent in the next time slot. In this case, if the prompt for service 1 is sent, the AI Inference processing for service 1 can be performed because the received prompts have reached 10. However, if the prompt for service 2 is sent, the AI Inference processing for both services cannot be performed as the received prompts for both services does not reach the configured batch size.

Figure 2-1:  Three main scheduling methods for a LLM

Based on the above analysis, if the transmission scheduling and AI service scheduling can be coordinated to make joint decision (e.g. considering the TTFT, TPOT, batch size of AI model, priority, received prompts, etc), the user experience of AI Inference service can be improved.

3	Separable AI Processing Blocks: Enabler for Joint Communication and Processing AI Inference Optimization

By design, DNNs and LLMs AI models consist of several separable blocks. Figure B.2-2 shows examples of commercial and research AI Models structured in separable processing blocks. For instance, Figure B.2-2 (I) shows examples of DNNs: VGG, GoogLetNet, and ResNet. Such examples are represented in a format of a graph. The resulting graph can be a simple graph showing a sequential organization of the separable processing blocks, or it can be a more complex graph (such as GoogLeNet) with branches of separable processing blocks.

The research documented by [4] involving 30 organizations (e.g, Google, Microsoft, Nvidia, Alibaba, among others) and more than 200 ML engineers and practitioners claims that “separable convolutions are used to reduce the complexity and computational burden” in case of DNNs. Meanwhile the same principles of separable AI processing blocks are proposed to enable collaborative LLMs execution in different proposals [5][6][7][8]. As illustrated in Figure B.2-2 (II) and (III), LLMs are composed of multiple layers and processing blocks.

Figure B.2-2 – Examples of DNN and LLMs architecture with different AI Graphs.

According with S. Liu et al. [9], applications can be designed in a “resource-friendly algorithm level” in order to specify the offloading schemes applicable to such model for supporting AI Inference subjected to performance demands and resource budget.

The design of AI applications already targeting the possible deployment scenarios and the possible mechanisms for optimization of the overall E2E AI Inference in a Mobile Network can be beneficial for both the application provides and the mobile operator.

-	From the perspective of the mobile operator, when application providers express the modularization of a designed AI application (e.g., the graph representation of the separable processing blocks) as well as the E2E transmission and processing requirements of such AI application, this enables such applications to delegate to the mobile operator the task to find and control the best fitting split for the execution of the given AI separable processing blocks. In this case, it is also the responsibility of the mobile operator to assure the maintenance of the structural flow of input and output data transmission among such blocks, avoiding thus to break the AI processing flow.

-	From the perspective of the application provider, investing in the development of applications with an architecture supporting AI inference offloading service (i.e., developing applications with separable AI processing blocks that can be split during inference) could ease the burden of application providers to be required to run and control the actual processing resources for executing such AI Application.

It is a fact that AI models advance quickly and can employ varies types of algorithms and increase in complexity in terms of functional components (e.g., LLMs depend on functionalities such as encoders, decoders, etc). However, no matter the actual type of algorithm used for AI models or the new types of functionalities that may be developed, once the AI Model is designed in a “resource-friendly algorithm level”, such as DNNs and LLMs as of today, such AI models and therefore their E2E AI inference can be network-controlled for optimizing the E2E AI Inference latency, and potentially improving the scalability of such AI applications due to the joint control of communication and computation by the mobile network.

4	References

[1] M. G. Spina et al., "In-Network Computing and Split-AI in 6G: Enablers and Proof-of-Concept Studies," 2024 3rd International Conference on 6G Networking (6GNet), Paris, France, 2024, pp. 135-143

[2] Wu, Di, et al. "In-Network Computing Empowered Mobile Edge Offloading Architecture for Internet of Things." IEEE Transactions on Services Computing (2024)

[3] M. Zhang, X. Shen, J. Cao, Z. Cui and S. Jiang, "EdgeShard: Efficient LLM Inference via Collaborative Edge Computing," in IEEE Internet of Things Journal, vol. 12, no. 10, pp. 13119-13131, 15 May, 2025

[4] V. J. Reddi et al., "MLPerf Inference Benchmark," 2020 ACM/IEEE 47th Annual International Symposium on Computer Architecture (ISCA), Valencia, Spain, 2020, pp. 446-459

[5] M. Ma, C. Gong, L. Zeng and Y. Yang, "Multi-Tier Multi-Node Scheduling of LLM for Collaborative AI Computing," IEEE INFOCOM 2025 - IEEE Conference on Computer Communications, London, United Kingdom, 2025, pp. 1-10

[6] S. Zhang, S. Zhang, Z. Qian, J. Wu, Y. Jin and S. Lu, "DeepSlicing: Collaborative and Adaptive CNN Inference With Low Latency," in IEEE Transactions on Parallel and Distributed Systems, vol. 32, no. 9, pp. 2175-2187, 1 Sept. 2021

[7] A. M. Jain and A. Jain, "Scaling LLM Inference Architectures: A Performance Analysis for Chatbot Applications," 2025 6th International Conference on Artificial Intelligence, Robotics and Control (AIRC), Savannah, GA, USA, 2025, pp. 8-16

[8] M. Becattini, R. Verdecchia and E. Vicario, "SALLMA: A Software Architecture for LLM-Based Multi-Agent Systems," 2025 IEEE/ACM International Workshop New Trends in Software Architecture (SATrends), Ottawa, ON, Canada, 2025, pp. 5-8

[9] S. Liu et al., "Enabling Resource-Efficient AIoT System With Cross-Level Optimization: A Survey," in IEEE Communications Surveys & Tutorials, vol. 26, no. 1, pp. 389-427, Firstquarter 2024