3GPP TSG-SA WG2 Meeting #170	S2-250

Gothenburg, Sweden, August 25 – 29, 2025

Source:	Samsung

Title:	KI#2: New Solution on Session control based on EIF notification

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution on control of UE’s sessions that the PCF and SMF may utilize the notification including Energy Consumption information from the EIF and update the session control parameters for a UE/Network Slice/PDU Session/QoS flow based on the received information.

# 1.	Discussion

One of the key issues for Rel-20 EnergySys Phase2 Study includes the discussion on solutions that can support adjustment of UE’s data transmissions based on the energy information. A new solution is proposed to support control of UE sessions by the PCF and SMF based on the energy consumption information received from the EIF.  The energy related information from the EIF includes the network energy consumption for a UE/Network Slice per UE/PDU Session per UE/QoS flow per UE.

# 2. Proposal

It is proposed to adopt the following changes to TR 23.700-67.

* * * Start of Change (all new text) * * * *

## 6.X	Solution #X: UE session control based on energy information from EIF

### 6.X.0	High-level solution principles

The solution proposes PCF and SMF use the energy related information received from the EIF to decide the parameters for control of UE sessions.

- The PCF may subscribe to the EIF for the desired granularity of the energy consumption information for the UE/Network Slice per UE/PDU Session per UE/QoS Flow per UE.

- The EIF collects the energy consumption information and notifies the energy consumption information in the requested granularity to the PCF.

- The PCF may decide the UE/Network Slice of the UE/PDU Session of the UE/QoS Flow of the UE that should be under energy usage control, based on the Energy Saving indication in the UE Subscription Data, energy consumption information received from the EIF, operator policy, etc.

- The PCF may inform the SMF of the selected UE/PDU Session of the UE/QoS Flow of the UE that should be under energy usage control.

- The SMF may adjust the session control parameters related to the selected UE/PDU Session of the UE/QoS Flow of the UE, based on the pre-configured information, operator policy, etc. for network energy saving.

### 6.X.1	Description

The solution addresses the Key Issue #2.

The solution proposes a session control mechanism that the PCF and the SMF sets the session control parameters for a UE/Network Slice of a UE/PDU Session of a UE/QoS Flow of a UE based on energy related information (per UE, PDU Session, QoS Flow) that is received from the EIF.

The solution assumes the Energy Saving indication for per-UE/Network Slice/DNN is stored in the Subscription Data of a UE. The PCF and/or the SMF may retrieve the information during the SM Policy Association or PDU Session Establishment. In case of the dynamic policy control, the PCF retrieves from the UDR and stores the Energy Saving indication information for UE/Network Slice/DNN and provides a policy to control the energy use by a UE during the policy association establishment or modification with the SMF. The PCF may subscribe to the EIF and receive the energy consumption information in a required granularity to control the energy consumed in the network to serve the UE/Network Slice/PDU Session/QoS flow. The PCF may decide, based on the energy related information received from the EIF, operator policy, energy saving indication, and local configuration, if network energy saving is required for the UE/Network Slice of the UE/PDU Session of the UE/QoS Flow of the UE. The PCF may provide an energy related policy for UE/Network Slice/PDU Session/QoS Flow to the SMF and request to apply a changed set of control parameters to UE/Network Slice/PDU Session/QoS Flow for the sake of network energy saving.

### 6.X.2	Procedures

Figure 6.x.2-1 illustrates a session management procedure considering energy related information from the EIF.

Figure 6.X.2-1: Session management procedure considering energy related information

1.	UE sends a PDU Session Establishment Request message including S-NSSAI, DNN, and other parmeters to set up a communication path for an application(s).

2.	The AMF sends an Nsmf_PDUSession_CreateSMContext Request message to the SMF to request a PDU session for the UE.

3.	The SMF responds to the AMF by sending an Nsmf_PDUSession_CreateSMContext Response message.

4.	The SMF performs Subscription Retrieval procedure with UDM and receives subscription data for the UE. The subscription data may include Energy Saving indication information for the UE (e.g. energy control per UE/Network Slice/PDU Session). The SMF may use this information together with operator policy and local configurations to decide control parameters for the UE’s PDU Session, e. g. inactivity timer, buffering size, etc.

5.	The SMF performs Policy Association Establishment procedure with PCF if dynamic policy is supported. The PCF may provide the SM policy for the PDU session of the UE (e.g. energy control per UE/PDU Session/QoS flow) based on the Energy Saving indication and energy related information from the EIF or OAM. The SMF may use the information to set the control parameters for the UE’s PDU Session, e. g. inactivity timer, buffering size, etc.

6.	The SMF sends an N4 Session Establishment Request message to the UPF to request a user plane resource setup at the UPF for the PDU session of the UE. The message includes the control parameters for the PDU session that was decided by the SMF in the step 5.

7.	The UPF responds to the SMF by sending an N4 Session Establishment Response message.

8.	The remaining steps for PDU Session Establishment procedure as defined in steps 11 to 21 in Figure 4.3.2.2.1-1 of TS 23.502 [3] proceeds.

9.  The PCF may subscribe to the EIF for notification of the energy-related information related to the UE/Network Slice/PDU Session/QoS Flow.

10. The EIF may send the updated energy related information as requested by the PCF in the step 9.

11.	The PCF may decide to change the policy for the UE/Network Slice/PDU Session/QoS Flow for network energy saving, e.g., to trigger the aggressive saving mode when the energy consumption in the network increases beyond the defined threshold happens, based on the energy related information included in the notification from the EIF, operator policy, local configuration, etc.

12.	The PCF performs SM Policy Modification procedure with SMF and provides the updated policy for the selected PDU session(s) of the UE/Network Slice of the UE/PDU Session of the UE/QoS Flow of the UE by including the indication for network energy saving.

13.	The SMF sends an N4 Session Modification Request to provide updated control parameters (e.g., inactivity timer, buffering size, etc. to control initiation of subsequent paging and data transmission procedures) for the PDU session(s) of the UE, based on the the indication for network energy saving received from the PCF, operator policy, local configuration, etc. The SMF may optionally send an N4 Session Release Request message to initiate the PDU Session Deactivation procedure for the selected PDU session(s) of the UE, based on the indication for network energy saving received from the PCF, operator policy, local configuration, etc.

14.	The remaining steps for PDU Session Modification or CN-initiated Deactivation procedure proceeds.

### 6.X.3	Impacts on existing services, entities and interfaces

PCF:

-	Subscribes to the EIF for energy related information and decide if the network energy saving is required for the UE/Network Slice/PDU Session/QoS Flow.

-	Provides the SMF with indication for network energy saving for the selected UE/Network Slice/PDU Session/QoS Flow in the policy information to control the energy use by a UE/PDU Session/QoS Flow.

SMF:

-	Sets session control parameters for a PDU session(s) of a UE based on the subscription data from the UDM or indication for network energy saving received from the PCF.

UDM:

-	Stores Energy Saving indication information for UE/Network Slice/DNN (PDU Session) in the subscription data.

* * * End of Change * * * *