SA WG2 Meeting #170	S2-2507319

# August 25 - 29, 2025, Goteburg, Sweden

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: Non-5tuple voice media transfer for 1 PDN connection scenario

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#1.

1	Introduction

This contribution proposes a solution for KI#1 on IMS voice service over GEO satellite, in particular addressing voice media size optimization with regards to 5tuple.

This solution assumes to use separate bearers in 1 PDN connection for SIP messages and for voice media.

With this solution, voice media can be transferred over GEO satellite without 5tuple.

2	Proposal

It is proposed to add the following text to TR 23.700-19 v020.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | X | X |  |  |
| #2 | X |  |  |  |
| #3 | X |  |  |  |
| #4 | X | X | X | X |
| #5 | X |  |  |  |
| #6 | X |  |  |  |
| #7 | X | X |  |  |
| #8 | X |  |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  |  |
| #11 | X |  |  |  |
| #12 |  | X |  |  |
| #X | X |  |  |  |

********** Next Change (All new text) **********

6.x	Solution #x: Non-5tuple voice media transfer for 1 PDN connection scenario

6.x.1	High level principles

This solution addresses KI#1, in particular voice media size optimization with regards to 5tuple.

In this solution, it is assumed that SIP messages and voice media are transferred in different bearers in 1 PDN connection. In this solution, it is proposed that P-GW uses 5tuple provided from P-CSCF via PCRF.

For UL, UE sends non-5tuple voice media packets and P-GW adds 5tuple. For DL, P-GW receives IP packets of voice media, removes 5tuple, and sends non-5tuple voice media packets to S-GW.

6.x.2	Description

The assumptions of this solution are as follows:

-	It is assumed that SIP messages and voice media are transferred in different bearers in 1 PDN connection.

The principles of this solution are as follows:

-	For UL, UE sends non-5tuple voice media packets and P-GW adds 5tuple. For DL, P-GW receives IP packets of voice media, removes 5tuple, and sends non-5tuple voice media packets to S-GW.

-	P-CSCF sends 5tuples of UL/DL voice media via PCRF to P-GW at the timing of setting up a dedicated bearer for voice media. P-GW uses these 5tuples for the above step.

NOTE:	According to TS 29.213, clause 6.2, Table 6.2.2, there are cases that P-CSCF wildcards some part of 5tuples and does not send them to PCRF. It is proposed that if P-CSCF recognizes GEO voice is being set up e.g., based on PANI, the P-CSCF informs PCRF all the 5tuple information that the P-CSCF has got from SDP exchange. The need for this specific treatment is to be checked with stage 3 group.

The advantages of this solution are as follows:

-	Voice media can be transferred over GEO satellite without 5tuple.

6.x.3	Procedures

Figure 6.x.3-1: IMS voice call setup MO

0.	During the Attach procedure, the following are performed:

-	A PDN connection is established. An IP address is allocated to UE. UE recognizes this IP address.

-	UE determines to use packets without 5tuple in this PDN connection or at least in a dedicated bearer. UE sends Extended protocol configuration options that have "CN header handling request" (a new indication) to P-GW. P-GW sends Extended protocol configuration options that have “CN header handling accepted" (a new indication) to UE. P-GW recognizes that it needs to add 5tuple for UL packets and removes 5tuple for DL packets for this PDN connection or for a dedicated bearer.

NOTE 1:	A dedicated bearer can be setup either during the Attach procedure, just after the Attach procedure, or during the SIP callsetup procedure depending on the selected solution.

1.	UE sends SIP INVITE to P-CSCF. Either during the IMS Registration procedure or with this SIP INVITE, P-CSCF recognizes that this SIP INVITE is for Geo voice.

2-4. P-CSCF, S-CSCF, and UE exchanges SIP INVITE and SIP 183 Session Progress as per the existing specifications.

5.	If a dedicated bearer for voice media is not established, P-CSCF sends to PCRF an AA-Request to establish a dedicated bearer as per the existing specifications. If a dedicated bearer for voice media is established, P-CSCF sends to PCRF an AA-Request to update the context of the dedicated bearer. In both cases, the AA-Request contains a fully populated Flow-Description, which does not use wildcards for information of IP addresses and ports.

6.	PCRF sends AA-Answer to P-CSCF.

7.	PCRF sends to P-GW an RA-Request containing the fully populated Flow-Description.

8.	If a dedicated bearer for voice media is not established, P-GW initiates to setup a dedicated bearer. If a dedicated bearer for voice media is established, P-GW updates the context for the dedicated bearer. P-GW sends RA-Answer to PCRF.

9.	P-GW recognizes 5tuple to be used for the 5tuple addition/removal handling. If a dedicated bearer for voice media is not established, P-GW proceeds to set up a dedicated bearer. Subsequent steps for setting up a dedicated bearer are omitted for the brevity of the description.

NOTE 2:	As per the existing specifications, 5tuple is used for creating PDR in P-GW. It is proposed to use the 5tuple also for the 5tuple addition/removal handling.

10.	IMS call setup procedure continues, and a voice call is set up.

11.	UE sends a voice media packet without 5tuple to P-GW by using the dedicated bearer.

12.	P-GW adds 5tuple to the voice media packet without 5tuple based on the context of the dedicated bearer.

13.	P-GW sends a voice media packet to IMS AGW.

14.	IMS AGW sends a voice media packet to P-GW.

15.	P-GW identifies that the voice media packet is to be forwarded in the dedicated bearer and removes 5tuple of the voice media packet.

16.	P-GW sends a voice media packet without 5tuple to UE by using the dedicated bearer.

6.x.4	Impacts on Services, Entities and Interfaces

P-CSCF:

-	When recognizing GEO voice is being set up e.g., based on PANI, P-CSCF informs PCRF all the 5tuple information that the P-CSCF has got from SDP exchange.

NOTE:	The need for this specific treatment is to be checked with stage 3 group.

P-GW:

-	Adding 5tuple for UL packets based on the bearer context and finding a bearer for DL packets based on the bearer context and removing 5tuple from them.

-	For the above purpose, use 5tuples that P-GW receives from P-CSCF via PCRF.

********** End of Changes **********