3GPP TSG-WG SA2 Meeting #171	S2-2508238

Wuhan, CN, 13th – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3: Conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#3 on authorization is proposed in this document.

# 1. Introduction

Based on the discussions and analyses in S2-2508235 and the architectural conclusion of KI#1 in S2-2508236, this paper provides the conclusion on KI#3 on Sensing Entity and Sensing Function Discovery and (Re-)Selection.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * *all new text

### 7.1.3	Agreed Principles for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed for KI#3:

a)	The NEF selects the Sensing Control Function(s) based on local configuration or query the NRF which matches the following criteria:

1)	The SCF Sensing Service Area of the Sensing Control Function that matches the Target Sensing Area;

b)	The Sensing Control Function may register to NRF, by invoking the Nnrf_NFManagment_NFRegister request service operation, with its NF profile, which includes the parameters defined in the bullet above.

NOTE 1:	Detailed list of parameters included in SCF profile in NRF may be further defined during normative phase.

c)	The Sensing Control Function selects the Sensing Processing Function based on local configuration or query the NRF which matches the following criteria:

1)	The SPF Sensing Service Area of the Sensing Processing Function that matches the Target Sensing Area;

d)	The Sensing Processing Function may register to NRF, by invoking the Nnrf_NFManagment_NFRegister request service operation, with its NF profile, which includes the parameters defined in the bullet above.

NOTE 2:	Detailed list of parameters included in SPF profile in NRF may be further defined during normative phase.

e)	The Sensing Control Function selects one or more the gNB(s) to satisfy the Sensing Service Request based on local configuration that match the following criteria:

1)	The Sensing Tracking Area and/or Sensing Cell of the gNB that matches with the Target Sensing Service Area;

2) 	The selection of gNB as Sensing Entity is only based on the information configured in the Sensing Control Function via OAM, or via direct N2 setup containing the gNB sensing capability by the gNB.

g)	The gNB registers its capability (i.e., Supported Sensing Tracking Area, or Sensing Cell) to the Sensing Control Function via N2 setup.

NOTE 3:	The detailed list of gNB capabilities to be considered by SCF for the purpose of selection of serving gNB will be coordinated with RAN WG3 in the normative phase.

The Figure 7.1.3-1 depicts the procedure for discovery and selection.

Figure 7.1.3-1: Procedure for discovery and selection.

0a.	The following configuration may be performed via OAM:

-	Sensing Control Function is pre-configured with the information for selection of the Sensing Processing Function (e.g., SPF IP address or FQDN, SPF Sensing Service Area);

-	NEF is pre-configured with the information for selection of the Sensing Control Function (e.g., SCF IP address or FQDN, SCF Sensing Service Area);

-	Sensing Control Function is pre-configured with the information for selection of gNB (e.g., gNB IP address or FQDN, Sensing Tracking Area, Sensing Cell);

-	gNB is pre-configure with the SCF information to send N2 setup request;

0b.	The gNB performs the N2 setup request with the SCF to register its sensing capabilities;

0c. Sensing Control Function may register to NRF, by invoking the Nnrf_NFManagment_NFRegister request service operation, with its NF profile;

0d. Sensing Processing Function may register to NRF, by invoking the Nnrf_NFManagment_NFRegister request service operation, with its NF profile,

1.	The AF sends a Sensing Service Request to the Sensing Control Function via NEF as described in KI#2 and KI#5.

2.	The NEF selects the Sensing Control Function(s) based on the local information or via the NRF as described above;

3.	The NEF forwards the Sensing Service Request to the selected SCF(s);

4.	The SCF the selects the SPF based on the local information or via the NRF as described above;

5.	Sensing Control Function sends the Sensing Service Request to the SPC;

6.	The SCF the selects the gNB as described above;

6.	The SCF send the Sensing Service Request to the selected gNB(s) as described in KI#4 and KI#6.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Principles in clause 7.1.3 are agreed as the conclusion.

# * * * * End of changes * * * *