3GPP TSG SA WG2#171	S2-2508520

Wuhan, China, October 13 - 17, 2025

Source:	Lenovo

Title:	Evaluations and proposed Interim conclusion for KI#2

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract of the contribution: Proposes evaluations and interim conclusion for KI#2 Authorization and Revocation to Support Sensing Service

# 1. Introduction

This pCR discusses the evaluations and proposes the interim conclusion for KI#2 Authorization and Revocation to Support Sensing Service. This paper mainly focuses on the following issues,

- Which node performs authorization for the sensing service request?

- Whether a new network function is needed to store the sensing authorization information?

- Which node can trigger revocation of the sensing service?

# 2. Discussion

There’re 14 solutions which have been proposed to address KI#2, i.e., Solution#2, Solution#3, Solution#6, Solution#8-12, Solution#19, Solution#27, Solution#33, and Solution#36-38.

- Solution#2 proposed that UE operates as a sensing service consumer while requesting sensing service to the core network. The UE is authorized based on its subscription data which includes the dedicated S-NSSAI for Sensing Service. UE sends sensing service request towards SF via PDU session associated with dedicated S-NSSAI for sensing service.

- Solution#3 proposed that NEF/Gateway performs authorization for sensing service consumer, and Sensing Function performs authorization for requested parameters.

- Solution#6 proposed that the Gateway sensing NF verifies whether the AF is authorized for the received sensing service request based on the AF's authorization information (e.g. based on service level agreement) and determines the target sensing service area(s) and the candidate sensing mode to support the requested sensing operation.

- Solution#8 proposed that NEF authorizes the AF request for sensing service and forward the request. SCF performs authorization based on the sensing entities' capabilities and the sensing service context. Besides, AF may trigger revocation for the sensing service.

- Solution#9 proposed that NEF performs sensing AF authorization based on the existing procedure. SSF performs authorization based on the AF authorization data (AF ID, Allowed Area, Allowed Service Operations, and Allowed time period) retrieved from the SPF. Revocation can be triggered by AF or SPF.

- Solution#10 proposed that sensing gateway performs authorization of the sensing service request from AF. Sensing gateway request authorization info of the sensing area from the sensing authorization information function. The sensing authorization profile includes parameters such as the sensing authorization indication and the sensing notification indication.

- Solution#11 proposed that there are two types of authorization involved in sensing service, i.e., initial authorization and re-authorization. For initial authorization, NEF authorizes the sensing service consumer. SF performs the authorization for the sensing service request, by checking the conditions with NF (e.g. UDR, or CCF) configured with service profile (i.e. service information containing specific conditions). For re-authorization, SF may re-select sensing entities by checking the sensing service specific information. Triggers for revocation of the on-going sensing service could be from 3rd party AF, or internal NF (e.g. SF, etc.).

- Solution#12 proposed that the NEF authorizes the AF request. The SF (SCF) authorizes the AF request received from the NEF and stores the information in the SF (SDR) as "Target Mapping Data". The SF (SCF) then sends a "SensingEntityAuthorization_Request" to obtain the authorization information for the mapped entities. The sensing entities provide a "SensingEntityAuthorization_Response" to indicate their authorization status and possibly resource availability status to the SF (SCF).

- Solution#19 proposed that the SeMF forwards the sensing service request for validation and authorization to a Sensing Policy and Authorization Function (SPAF).

- Solution#27 proposed that NEF performs authorization for the sensing service request. SF shall perform authorization validations depending on the type of the sensing service request, the target sensing area, the target objects sensing based on UE’s subscription data.

- Solution#33 proposed that for AF triggered sensing service, the gateway sensing NF may authorize the sensing service request from the AF. For UE triggered sensing service, the sensing NF performs authorization of the sensing service request.

- Solution#36 proposed that SF may trigger sensing service revocation to AF/NEF based on receiving notification from the sensing RAN node, e.g., sensing object is approaching Sensing Non-Allowed area, or sensing object is out of coverage etc.

- Solution#37 proposed that the Sensing Service authorization consists of two steps: authorization of the AF performed by the NEF and authorization of the specific sensing service request performed by the Sensing Control Function. When the AF is the consumer, the sensing service for an AF can be revoked by either the NEF or the Sensing Control Function.

- Solution#38 proposed that a new function, the SDM, is introduced to store sensing authorization information (e.g. the authorization information of sensing areas). NEF performs authorization of the AF request by retrieving sensing authorization information from the SDM.

In short, 7 solutions out of 14 solutions propose both NEF and SF to perform authorization of the sensing service request. 4 solutions out of 14 solutions introduce a new NF (or part of the SF) to store the sensing authorization information. 5 solutions mention that AF, NEF and SF may trigger the revocation of the sensing service.

# 3. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

- The authorization of the AF request for the sensing service includes two parts:

  - NEF performs AF request authorization for the sensing service based on SLA between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501. This includes: 1) checking whether the AF is authorized for the requested sensing service operation; 2) checking whether the AF is authorized for the requested target sensing area;

  - SF further performs authorization of sensing service for the requested parameters (e.g., sensing service type, sensing service requirement etc.) based on local configuration and sensing entities’ capabilities.

    - If there is hierarchical deployment of SFs, the GW SF performs the authorization of sensing service.

- No new NF is introduced for storing the sensing authorization information.

- The revocation of sensing service request can be triggered by NEF or SF.

  - The NEF may revoke the sensing service request due to SLA change.

  - The SF may revoke the sensing service request due to local configuration change. Besides, the SF may terminate/release the sensing service based on notification from the sensing RAN node, e.g., sensing object is approaching Sensing Non-Allowed area, or sensing object is out of coverage etc.

NOTE: AF can terminate/release the sensing service request at any time. Meanwhile, revocation of sensing service means to revoke the authorization of the sensing service.

# * * * * End of changes * * * *