3GPP TSG-WG SA2 Meeting #171	S2-250AAAA

Wuhan, China, October 13 - 17, 2025	(revision of S2-250xxxx)

Source:	Nokia

Title:	KI#1 conclusion for reader selection

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2 / Rel-20

Abstract: The contribution discusses and proposes interim agreements for KI#1.

# 1. Introduction/Discussion

This pCR proposes an interim agreement regarding the AIoT reader selection as part of KI#1 discussion.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-30.

FIRST CHANGE (All text is new)

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

- UE Reader selection aspects:

- When the target area information is provided by the AF

  - The target area information is forwarded to the responsible NG-RAN, which then selects the UE readers. If both RAN readers and UE readers are co-deployed and the requested area is covered by both, the NG-RAN remains responsible for deciding which readers to use.

  - The AIOTF selects a set of candidate UE readers available in the requested area using CN information (e.g., subscription, reachability, session status, etc.) and provides this list to the responsible NG-RAN. The NG-RAN then performs the final selection from the candidate list based on its own local information (e.g., radio link quality, resource availability, etc.). If both RAN readers and UE readers are co-deployed and cover the requested area, the AIOTF determines which type of readers to use according to operator policy.

- When specific reader information is provided by the AF

  - This indicates that the requested UE readers are designated to act as readers for the service. In this case, the AIOTF uses the specified UEs directly, without performing any reader selection procedures.