SA WG2 Meeting #171	S2-2508841

13–17 October 2025, Wuhan, China	(revision of S2-2507663)

Source:	NEC [LG Electronics, Toyota]

Title:	KI#1: Interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#1 based on the latest version of S2-2507663 as per the agreement at SA2#170 meeting in Gothenburg.

# Discussion

[SA2#170 history]

There are 8 solutions for KI#1 "Network energy related information exposure" TR 23.700-67v0.2.0.

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution # & title | 1 | 2 | 3 |
| 1: Enhancement of Exposure API to include UE categorization data | X |  |  |
| 2: Policy Control Considering Slice or 5GC Energy Consumption | X | X |  |
| 8: Estimation of Renewable Energy consumption | X |  |  |
| 9: Consumption energy source exposure | X |  |  |
| 10: Energy consumption calculation enhancements | X |  |  |
| 11: Energy Consumption estimation considering idle energy consumption | X |  |  |
| 12: Enhancement of energy related information exposure | X |  |  |
| 22: Support of UP path adjustments to maximize the usage of renewable energy | X |  | X |

Observation#1: Sol#2 is considered more related to KI#2 "Support of service adjustments for the UE" because this solution mainly proposes AM/SM/BDT policy adjustment.

Proposal#1: It is proposed to discuss Sol#2 when handling interim agreements/conclusions for KI#2.

Observation#2: Sol#22 is considered more related to KI#3 "NF (re)selection and/or UP path adjustment based on energy related information" because this solution mainly proposes UPF (re)selection by considering energy related information such as renewable energy usage along with the existing parameters.

Proposal#2: It is proposed to discuss Sol#22 when handling interim agreements/conclusions for KI#3.

Other solutions than Sol#2 and Sol#22 can be discussed related to KI#1 interim agreements.

**************************

The following tdocs were merged:

| S2-2506241 | 23.700-67: KI#1: proposed agreement of principles | Nokia |
| --- | --- | --- |
| S2-2507017 | 23.700-67: WT#1: Accuracy evaluation of estimation | Huawei, Hisilicon |
| S2-2506621 | 23.700-67: KI#1: Interim agreements | LG Electronics, Toyota |
| S2-2506770 | 23.700-67: Key issue#1, the interim agreement | ZTE |
| S2-2506893 | 23.700-67: KI#1: Renewable energy exposure principle proposal | NEC |
| S2-2507337 | 23.700-67: Interim principle for KI#1 | China Telecom |
| S2-2507047 | 23.700-67: KI#1: proposal for agreed principles | vivo |

[SA2#171 updates]

- On renewable energy exposure - Deleted Note 2 which suggests SA5 feedback on the renewable aspect. The energy consumption exposure functionality is not dependent from the type of the energy. Any energy consumption, renewable or non-renewable, is expressed with the same consumption measuring units. As about the availability of the renewable energy provided by the SA5, it would be exposed when it is available. Same applies for the non-renewable energy consumption by RAN for example which is provided from the SA5 and not available yet. So, no need to delay defining the renewable energy exposure aspect by asking SA5 for feedback.

- On the idle mode energy consumption – Contributions to SA2#170 on the idle mode energy consumption calculation considerations were not agreed. The study requirements are to consider the energy consumption on the user plane only. We suggest that any further potential improvements to the energy consumption calculations considering the idle mode are left out of this release.

It is proposed that we continue with any further principles updates or new principles based on this updated version.

# Proposal

It is proposed to agree the following changes into TR 23.700-67.

* * * * Start of Changes * * * *

!! All New Text !!

### 7.2.Y	Topics for further consideration for KI#1

Topics for further consideration for KI#1 are as below:

-	The following new/additional energy related information is exposed by EIF:

-	UE categorization data at the desired granularity e.g. percentile of the consumption of the UE at the granularity of the estimate as proposed by Solution#1 described in clause 6.1.

-	Application ranking related information, i.e. list of top contributing applications and/or ordered list of contributing applications for a UE or for a PDU Session as proposed by Solution#12 described in clause 6.12.

NOTE 1:	For Solution#1 and Solution#12, clarification regarding e.g. case of multiple EIFs, etc is needed to conclude these solutions.

-	Renewable energy aspect is taken into account for collecting, calculating and exposing energy consumption information by EIF, as proposed by Solution#8 and Solution#9 described in clause 6.8 and clause 6.9, respectively.

* * * * End of Changes * * * *