SA WG2 Meeting #170	S2-2509146

13th – 17th October, 2025, Wuhan, China

Source:	ETRI

Title:	New Sol: Emergency SMS over IP

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#1 and KI#3 of TR 23.700-65.

1. Introduction

The key issue #1 of the TR 23.700-65 is:

This key issue aims to study solutions to:

-	identify whether an SMS should be routed to a PSAP;

-	identify the type of emergency service (police, ambulance, fire brigade, etc.) required;

-	address both the case of the UE detecting and the UE not detecting an SMS that is to be routed to Emergency Response Centre;

-	consider the scenario where the serving PLMN does not support routing an SMS to an Emergency Response Centre.

The key issue #3 of the TR 23.700-65 is:

The following aspects are to be studied:

-	how and when to trigger location determination and location data sharing when sending short messages using SMS over IP for routing to Emergency Response Centre, considering both 3GPP and non-3GPP access scenarios;

-	how to route short messages using SMS over IP to Emergency Response Centre serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location);

-	how to support PSAP callback capabilities for sending and routing regular short messages using SMS over IP.

The key issue covers SMS delivery via IP both in EPS and 5GS.

NOTE 1:	The solutions for EPS and 5GS are based on same principles.

The key issue covers both roaming and non-roaming scenarios.

2. Proposal

It is proposed to agree the following updates to TR 23.700-65.

*** Start of Changes ***

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| X | x |  | x |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |

*** Next Change ***

6.X	Solution #X: <Solution Title>

6.X.0	High level principles

This solution addresses KI#1 and KI#3.

The principles of this solution are listed below:

-	The UE sets the R-URI to the SOS URN if the entered number is identified as an emergency number; otherwise, the UE sets the R-URI to the input number.

-	The PLMN sends an error response to the SIP MESSAGE if it does not support routing of SMS to an Emergency Response Centre.

-	The P-CSCF determines whether the SMS is an emergency SMS based on the R-URI and its internal database of local emergency numbers.

-	The E-CSCF selects the appropriate PSAP based on the R-URI and the UE’s location information (ULI).

6.X.1	Description

For SMS over IP, the UE constructs and sends the emergency SMS as follows. For emergency SMS, the R-URI is set to the SOS URN and may include the UE’s location information. When the access network is non-3GPP access, the UE’s location information shall be included in the SIP MESSAGE.

The P-CSCF determines whether the SMS is an emergency SMS by checking the R-URI against the local emergency number database. If it is determined to be an emergency number, the P-CSCF replaces the number in the R-URI with the corresponding SOS URN (e.g. urn:service:sos) to route the message to the E-CSCF for emergency handling.

The E-CSCF checks the location information in the SIP MESSAGE and may query the LRF to obtain the UE’s location information if it is not present or insufficient to select the PSAP. The E-CSCF selects the PSAP based on the R-URI and the location information, and modifies the SIP MESSAGE to include the PSAP address.

If the serving PLMN does not support routing of emergency SMS to an Emergency Response Centre, the E-CSCF returns an appropriate SIP error response to the UE.

Upon receiving error response for SIP MESSAGE, the UE determines whether the emergency SMS has been successfully delivered to the PSAP.

6.X.2	Procedures

#### 6.x.2.1 Emergency SMS over IP

Figure 6.x.2-1: Emergency SMS over IP

1.	When the UE sends an SMS over IP, it sends a SIP MESSAGE to the P-CSCF in the serving network. For an emergency SMS, R-URI is set to SOS URN and may include the UE’s location information. When the access network is non-3GPP access, the UE’s location information shall be included in the SIP MESSAGE.

2.	The P-CSCF determines whether the SMS is an emergency SMS by checking the R-URI against the local emergency number list. If it is determined to be an emergency number, the P-CSCF replaces the number in the R-URI with the corresponding SOS URN (e.g. urn:service:sos) to route the message to the E-CSCF for emergency handling. The P-CSCF forwards the emergency SMS to the E-CSCF, i.e. an S-CSCF with the capability to handle emergency SMS.

3-4.	The E-CSCF checks the location information in the SIP MESSAGE and may query the LRF to obtain the UE’s location information if it is not present or insufficient to select the PSAP. The E-CSCF selects PSAP based on the R-URI and the location information and modifies the SIP MESSAGE to include the PSAP address.

6b-6c.	The E-CSCF returns an appropriate error response to the UE when the serving PLMN does not support routing to an Emergency Response Centre.

5-6.	The E-CSCF forwards the SIP MESSAGE to IP-SMGW.

7-8	.	The IP-SMGW forwards the emergency SMS to PSAP.

9-10.	A Submit Report is returned from the SMSC to the UE.

11.	The UE returns a successful response (e.g. 200 OK) to the IP-SMGW.

6.X.3	Impacts to Services, Entities and Interfaces

UE:

-	Support to detect emergency numbers based on the input number.

-	Support to include SOS URN in R-URI for the detected emergency input number.

-	Support to check the error response for the SIP MESSAGE to determine whether the serving PLMN supports routing of SMS to an Emergency Response Centre.

P-CSCF:

-	Support to determine whether the message is an Emergency SMS based on the R-URI and local emergency number database.

-	Support to modify R-URI of SIP MESSAGE to SOS URN for numbers not detected by the UE.

E-CSCF:

-	Support to check UE’s location information in SIP MESSAGE.

-	Support to query LRF to obtain UE’s location.

-	Support to select PSAP based on the R-URI and UE's location.

-	Support to forward emergency SMS to PSAP.

-	Support to return an error response when the PLMN does not support routing of SMS to an Emergency Response Centre.

*** End of Change ***