SA WG2 Meeting #172	S2-2509913

17 – 21 November 2025, Dallas, US

Source:	LG Electronics

Title:	KI#6: Conclusions and Interim Agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes conclusions and interim agreements for KI#6.

Discussion

[Summary] KI#6: Configuration of Parameters for Sensing Entities

For KI#6 "Configuration of Parameters for Sensing Entities", 7 solutions have been proposed. An analysis and summary of these solutions are provided in the following table.

| Sol.# | Parameter Exchange Method (SE-CN) | Parameter Types for Configuration |
| --- | --- | --- |
| #3 | SF or SCF provides configuration parameters to sensing entity(ies); details on signalling path (direct or via AMF) are FFS | Configuration parameters for sensing measurement (No details) |
| #15 | Sensing Entities register/deregister/update capabilities to SF; SF activates/deactivates/reconfigures SEs based on service request | SF profile (supported sensing area); SE profile (supported area, sensing Tx/Rx) |
| #20 | Trusted AF sends configuration request directly to SCF; Untrusted AF send configuration request indirectly to SCF via NEF; SCF provides configuration parameters to sensing entity(ies) | AF ID; sensing-related parameters; target/area information |
| #27 | SF sends configuration to SE(s) with sensing operation parameters | Sensing service type; sensing target area; sensing time parameters; sensing contextual info; requested sensing QoS; sensing result config |
| #34 | SSC sends the sensing service request NEF/GSF; SF sends directly the sensing request to sensing entity with the configuration parameters | Service parameters from AF (e.g., sensing object type, target sensing service area, QoS); Parameters configured by SF (e.g., RCS range list, object velocity range) |
| #35 | Direct connection between SF to SE; Indirect connection where SF sends the parameters via AMF to SE | Service parameters from AF/NEF (e.g., sensing service type, target area, sensing duration, KPI, SSF ID). |
| #39 | SF provides sensing configuration to gNB via AMF | Configuration related to the sensing request (e.g., target sensing area, sensing accuracy, sensing mode, object characteristics vertical/horizontal sensing resolution) |

Summary

- 7 solutions (Sol#3, Sol#15, Sol#20, Sol#27, Sol#34, Sol#35, Sol#39) agree that sensing configuration parameters are provisioned from sensing function(s) to sensing entity(ies). - 3 solutions (e.g., Sol#3, #35, #39) introduce indirect provisioning via AMF, while 6 solutions (e.g., Sol#3, #15, #20, #27, #34, #35) support direct provisioning from sensing function(s) to sensing entity(ies). - Sol#34 proposes that configuration parameters may originate from both AF inputs and SF-generated parameters. - Solutions such as Sol#20 and Sol#34 highlight the distinction between trusted and untrusted AFs, introducing gateway or intermediate functions (e.g., NEF, GSF).

[Additional Issue] Sensing Configuration Parameters

The configuration parameters need to be identified to support the configuration of SEs by the SF. The detailed definition of these parameters will be further discussed and aligned during the normative phase.

| Messages | Parameters |
| --- | --- |
| Sensing Configuration Parameters (SF→SE) | - Sensing Service Area, Sensing Service Type. - (Optional) Sensing Correlation ID. - (Optional) Required Sensing QoS (e.g., sensing duration, sensing object type, sensing resolution). |

LGE Conclusions

- Before sending an LS to RAN WGs regarding sensing configuration parameters sent to the SE(s), SA2 needs to conclude a set of sensing configuration parameters.

Proposal

It is proposed to agree the following changes into TR 23.700-14.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1	Agreed Principles

7.1.6	Agreed Principles for KI#6: Configuration of Parameters for Sensing Entities

Based on the explanation in Discussion clause the following principles are proposed for interim agreement.

-	The configuration parameters for the sensing service may be provided by the AF and are received by the Sensing Function (SF).

-	The SF derives the parameters received from the AF and may generate additional parameters applicable to the sensing service.

-	The configuration parameters for the Sensing Entity (SE) are as follows:

-	Sensing Service Area.

-	Sensing Service Type.

-	(Optional) Sensing Correlation ID.

-	(Optional) Required Sensing QoS (e.g., sensing duration, sensing object type, sensing resolution).

-	The SF sends the configuration parameters directly to the SE to configure the sensing service.

* * * * Start of 2nd Change * * * *

!! All New Texts !!

7.2	Topics for further consideration

7.2.6	Topics for further consideration for KI#6

-	Whether and how the sensing service requirements are mapped into sensing configuration parameters.

* * * * Start of 3rd Change * * * *

!! All New Texts !!

8.6	Conclusion on KI#6

The key issue #6 covers “Configuration of Parameters for Sensing Entities”.

The following aspects common for Sensing Service are concluded as principles for normative work:

-	The configuration parameters for the sensing service can be provided by the AF and are received by the Sensing Function (SF).

-	The SF derives the parameters received from the AF and may generate additional parameters applicable to the sensing service.

-	The configuration parameters for the Sensing Entity (SE) are as follows:

-	Sensing Service Area.

-	Sensing Service Type.

-	(Optional) Sensing Correlation ID.

-	(Optional) Required Sensing QoS (e.g., sensing duration, sensing object type, sensing resolution).

-	The SF sends the configuration parameters directly to the SE to configure the sensing service.

NOTE:	The details of configuration parameters including whether and how the sensing service requirements are mapped into sensing configuration parameters will be coordinated with RAN WGs during the normative phase.

* * * * End of Changes * * * *