3GPP TSG-WG SA2 Meeting #172	S2-2509994

Dallas, US, 17th – 21st Nov, 2025

Source:	CEWiT

Title:	Conclusion proposal for KI#6

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide conclusions for KI#6 to be incorporated into Clause 8.

# 1.	Discussion

This contribution proposes the conclusions for KI#6. The conclusions are drafted based on the proposals from different companies for KI#6 in 3GPP TR 23.700-14 V1.1.0.

# 2.	Text Proposal

It is proposed to capture the following changes in TR 23.700-14 in Clause 8.

# * * * * First change * * * *

# 8	Conclusions

## 8.f	Conclusion for KI#6: Configuration of Parameters for Sensing Entities

## The following proposed principles are to be agreed for KI#6 and taken as baseline for normative work:

- The SeMF configures the sensing entity(ies) for a sensing service request received from NEF/AF

- The SeMF will derive the configuration parameters based on the received sensing service request. The SeMF can also generate certain parameters that are applicable for the sensing service request based on internal logic.

- The SeMF configures the sensing entity(ies) for the sensing service request either directly or via the AMF

- The configuration parameters may include Sensing Service Type, Target Sensing Area, Duration of Sensing, Sensing KPI/QoS, Sensing Object Type, RCS (Radar Cross Section) range list, Object velocity range, sensing contextual information, location and speed accuracy, address for receiving sensing data.

NOTE: The configuration parameters provided by the SeMF to the gNB(s) has RAN WG3 impact and needs to be expanded during the normative phase.

# * * * * End of changes * * * *