3GPP TSG-SA WG2 Meeting #172	S2-2510107

Nov 13-17, 2025, Dallas, USA

Source: 	China Telecom, NTT DOCOMO, CSCN

Title: 	Updated Interim conclusion for KI#1 Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an updated interim conclusion for KI#1 of FS_5GSAT_Ph4_ARC on IMS signalling data transport.

1	Discussion

8.1	Interim conclusions for Key Issue #1

Editor's note:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

In SA2#171 meeting, the interim conclusion for voice packets transport has been agreed in S2-2509293:

How to transport IMS signalling is FFS.

The discussion paper S2-2510108 provides technical comparation and evaluation between IMS signalling transport on CP and UP with following observations:

- Observation 1: For UE in ECM-CONNECTED, transportation Signalling via CP is prior over UP hence less ECST, although reduced ECST value can not be quantified.

- Observation 2: For UE in ECM-CONNECTED, extra ECST of CP due to NAS head is negligible.

- Observation 3: For UE in ECM-IDLE, ECST of CP is significantly less than UP due to quick state transition.

- Observation 4: For both non-roaming and roaming case, CP approach has benefit for Signalling security.

- Observation 5: Signalling via CP has benefit over UP for IMS SMS service

- Observation 6: Existing spec supports simultaneously transport of CP and UP but separated in PDN connections to SCEF and PGW, then for CP approach the enhancement is support of CP and UP within a single PDN conncetion.

Based on above observations, following proposal for updated interim conclusions is provided:

- The IMS signalling shall be transported over the NB-IoT (GEO) control plane, i.e. using SRB and S11-U.

- For the UE, MME and SGW, IMS PDN connection in NB-IoT accesss shall support default bearer of CP for signalling and dedicated bearer of UP for voice packet.

2 Proposal

It is proposed to add the following conclusion into TR 23.700-19 v1.1.0.

# * * * * First Change* * *

8.1	Interim conclusions for Key Issue #1

Editor's note:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-  The IMS signalling shall be transported over the NB-IoT (GEO) control plane, i.e. using SRB and S11-U.

-  For the UE, MME and SGW, IMS PDN connection in NB-IoT access shall support default bearer of CP for signalling and dedicated bearer of UP for voice packets

# * * * * End of change * * * *