SA WG2 Meeting #172	S2-2510111

17 - 21 November 2025, Dallas, USA	(was XXXX)

Source: 	Nokia

Title: 	Interim conclusions on KI#2 Support of SMS Delivery to Emergency Response Centre via NAS

Document for: 	Approval

Agenda Item: 	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: This contribution provides an interim conclusion of KI#2 Support of SMS Delivery to Emergency Response Centre via NAS.

# 1	Discussion

In the SA2#171 meeting, a total of 8 solutions were proposed for Key Issue #2, forming a comprehensive set of “SMS over NAS” routing options for SMS to EC.

Given the common framework proposed in all the solution papers, this paper proposes an interim conclusion for KI#2

# 2 	Proposal

It is proposed to capture the following contents into TR 23.700-65 v0.2.0.

# * * * * First change* * * *

# 8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.1	Interim conclusions for Key Issue #2

Editor's note: Other interim conclusions for Key Issue#2 are FFS.

The following interim conclusions for Key Issue #2 Support of SMS Delivery to Emergency Response Centre via NAS are made:

-	When the serving network supports SMS to EC capability, the serving network sends an indication in response to the Attach/Registration request whether the serving PLMN supports emergency SMS over NAS and/or emergency SMS over IP.

-	If the serving PLMN does not support SMS to EC capability in either SMS over NAS or SMS over IP, then the UE can select another serving PLMN when it intends to send an SMS to EC.

-	A UE receiving an indication that SMS to EC over NAS is supported in a serving PLMN but failed to send the SMS over NAS, may attempt emergency SMS over IP if configured in the UE and supported by the network.

-	The serving PLMN will provide the local emergency number(s) to the UE during the Attach/Registration procedure, as per the existing mechanism used for emergency numbers for emergency calls.

-	If the UE need to send SMS to EC, the UE generates the SMS-SUBMIT TPDU with the local emergency number in the TP-DA field.

-	The SMSF/MME/MSC shall check the TP-DA field for the local emergency number. If the TP-DA contains a local emergency number, the SMSF/MME/MSC will replace the RP-DA with a local SMSC number earmarked to handle EM SMS as per local configuration. The SMSF/MME/MSC will forward the SMS after the RP-DA replacement to GMSC/SMS-IWMSC/SMS Router.

-	The local SMSC then routes the SMS to a local PSAP based on the emergency number.

# * * * * End change* * * *