SA WG2 Meeting #172	S2-2510116

Dallas, USA, 17 – 21 November 2025

Source:	vivo

Title:	[WT#1.2, Network Exposure] Support for network exposure to UE in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to study the WT and KI for network exposure to UE for 6G network.

Introduction:

The approved SA2 6G SID includes the following work task:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

…..

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

……

This paper aims to focus on network exposure to UE in 6G under WT#1.2.

1. Justifications

1.1 Architectural Motivation on network exposure to the UE

The network exposure enables the network to expose capability and information to the third party service providers. However, for the UE side, the network information is also needed from the following aspects:

- UE application client needs the network status information for the optimization of the UL data transmission, e.g., adjustment of the codec algorithms according to the network status information. The information exposed to the UE should be real-time. Since the network status may be changed frequently, so the UE application needs the latest network status information for the UL traffic optimization.

- Improve end-user QoE experience: When the network transmission performance is getting worse, the UE can identify that the problem is due to the communication network and provides the solution from the UE's perspective to improve the service experience

- The UE is a whole system including OS, modem, application clients, etc. The network information exposure to the UE could benefit other layers within the UE, not only the application clients. The benefits include the following aspects:

  - Improvement of the UE’s internal control efficiency;

  - Multi-layer optimization within the UE;

  - Support for real-time optimization of service flows at the UE.

- In 6G, the UE may be enabled with AI capabilities, e.g., QoS evaluation and QoS prediction capability. The information (e.g. per UE or per flow QoE, network resource status) exposed to the UE may help the UE to train the AI capability for QoS prediction. This AI capability enables the UE to get the information on whether the QoS requirements for certain applications can be satisfied in the near future.

  - For example, when the user is on the subway during the morning rush hour, the network is congested and the service experience is bad; however, when the user is at the office, the network status is good and service experience will be much better than on the subway. So the network status information for different time and location varies a lot and is beneficial for the AI model training for QoS prediction within the UE.

1.2 Gap Analysis on network exposure

In 5G network, network capability and information exposure to AF has been specified. However, it is not supported for the 5G network to expose the information to the UE directly. The UE may get the exposed information from the AS/AF via application layer; however, this aspect is not fully specified due to the fact that the application layer between the UE and AS/AF is out of 3GPP SA2 scope. The current mechanisms for the network information exposure to UE via AF/AS has the following issues:

- It relies on the deployment of AF at the application server side. And third party AFs are not commonly deployed in the real business.

The 3rd party application providers need to deploy AF for the interaction with the mobile network. Since the users of the application may belong to different operators, the 3rd party AF needs to interact with multiple operators to get certain network information and apply this information for its applications.

Furthermore, the development and deployment of AF in the 3rd party may require extra capabilities to understand and realize APIs defined in the 3GPP standards. Furthermore, different operators may support different APIs towards the AF. So this may not be an easy work for the 3rd party to support the AF.

Figure 1: Network exposure to the AF

- The information exposure to UE from AF/AS may introduce extra delay.

Currently, the network exposes the information to AF via NEF/PCF/UPF. Then the application server sends the information to the UE via application layer. To the delay sensitive applications, this information may not be useful if the information can’t be received by the UE in time.

- The information exposure to the UE via AF/AS can only be exposed to the UE’s App Client. However, as the motivation stated, the UE’s OS or modem may also need the information. If the UE’s App Client doesn’t expose the info to the UE’s OS or modem, these parts within the UE can’t get this information.

1.3 Technical Impact

- The exposure architecture, including whether the new 6G entities or 5G NEF enhancements are needed for the management of the network capability exposure to the UE?

- Network capability/information exposure from the core network to the UE (e.g., via control plane, or via user plane);

- Network capability/information request from the UE and the authentication of the UE request for network capability/information exposure;

2 Proposals for S2-2509787 (the postponed version from pen holder in SA2#171 meeting)

Proposal1: For Bullet1: it is proposed to remove the following text from the postponed version(S2-2509787), and the corresponding reasons is provided in the table:

| Text | Reason |
| --- | --- |
| - Whether and how to support the co-existence with the 5G exposure frameworks. | Whether and how 6G co-exists with 5G is pending to migration of WT#2 |
| NOTE 0: The Common Network Exposure Framework in SA2 does not duplicate with the SA WG6 CAPIF Framework and, depending on SA WG6 decisions, CAPIF may be reused. | We don’t know whether SA6 reuse CAPIF in 6G or not. Cannot capture it as it is. |
| NOTE 1: Examples of network capabilities include information exposure such as UE related events, network events, analytics, and information provisioning such as UE related parameters, network configuration parameters, and service specific parameters. | Up to study. |
| - Whether and how to support privacy protection, user consent, data anonymization, and service authorization mechanisms. NOTE 1a: Authorization, Privacy protection and user consent aspects require coordination with SA WG3. | Pending to SA3, it is difficult to determine without SA3 input. |
| Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary. | Roaming related should be studied in WT#2 |

Regards to Bullet 4, this bullet highly depends on the study of WT#3, so it’s better to have the related discussion in the specific WT about the AI related feature first. And then, we align the progress, if needed, in the network exposure framework WT.

Proposal2: For Bullet4, it is proposed to update NOTE 8 as: The study for bullet#4 is postponed until it is decided that intent-based exposure is supported in the 6G system

**** First Change to S2-2509787 ****

Annex A.X. WT#1.2 Network Exposure Framework

This work task is to study whether and how to support a network exposure framework in 6G. The capabilities that are exposed via the framework fall into one of two categories.

- The first category is new capabilities. New capabilities refer to capabilities that are not exposed in the 5G network. What new network capabilities are exposed in 6G is dependent on other work tasks.

- The second category refers to capabilities that are exposed in the 5G network and are used as a starting point for discussion. Whether and what information are exposed in 6G is dependent on this work task and may also be dependent on other work tasks.

Since this work task depends on other work tasks, time will not be allocated for solutions for this work task by SA2 in Q1 of 2026.

Aspects that can be studied are:

- How to support a common network exposure framework for the exposure of network capabilities to an AF.

  - Whether and how to support a common framework for the exposure of network capabilities that is exposed to AFs. The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this work task.

- How to support on-path Network Capability Exposure to Application Endpoints.

  - Whether, how, and what capabilities (e.g. QoS control) to expose on-path between application endpoints (i.e. in the UE or AS) and the 6G network in an operator-controlled manner.

NOTE X:	The network capabilities defined in 5G can be considered as start point for the study.

  - Whether the exposure/reporting path of an exposed capability can be optimized (e.g. on-path) and if so, how.

  - How to discover, authorize and activate on-path exposure services offered by the network and whether and how to integrate on-path exposure into the common exposure framework.

NOTE 4f:	Authorization aspects require coordination with SA WG3.

NOTE 4c:	"on-path" refers to exchanging the information via a PDU Session.

NOTE 5:	On-path information exposure will not replace existing NEF APIs, e.g., for parameter provisioning.

NOTE 6:	This work task will cover how capabilities and information are exposed on-path. What capabilities and information are exposed on-path and how they will be exposed will be coordinated with other work tasks (e.g. the work that is done under the User Plane and QoS aspects of Work Task 1.2).

NOTE 7:	On-path information exposure may require coordination with WorkTask 1.1.

- Whether and how to support Intent-Based Exposure.

  - If, as a result of work that takes place under other work tasks (e.g. WT#3), it is decided that intent-based exposure is supported in the 6G system, what capabilities, if any, are needed in the network exposure framework to support intent-based exposure. The scenarios for intent-based exposure (e.g. the goal of an intent) will be defined by the work that is done under Work Task #3. This wok task will study how the procedures on the southbound interfaces of the NEF can achieve any intents that are identified within work task #3.

NOTE 8:	The study for bullet#4 is postponed until it is decided that intent-based exposure is supported in the 6G system.

**** Second Change ****

5.X. Key Issue #X: System architecture design to support network exposure for 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This Key Issue will study:

- How to support on-path Network Capability Exposure to Application Endpoints.

  - Whether, how, and what capabilities (e.g. QoS control) to expose on-path between application endpoints (i.e. in the UE or AS) and the 6G network in an operator-controlled manner.

NOTE X:	The network capabilities defined in 5G can be considered as start point for the study.

  - Whether the exposure/reporting path of an exposed capability can be optimized (e.g. on-path) and if so, how.

  - How to discover, authorize and activate on-path exposure services offered by the network and whether and how to integrate on-path exposure into the common exposure framework.

NOTE 4f:	Authorization aspects require coordination with SA WG3.

NOTE 4c:	"on-path" refers to exchanging the information via a PDU Session.

NOTE 5:	On-path information exposure will not replace existing NEF APIs, e.g., for parameter provisioning.

NOTE 6:	This work task will cover how capabilities and information are exposed on-path. What capabilities and information are exposed on-path and how they will be exposed will be coordinated with other work tasks (e.g. the work that is done under the User Plane and QoS aspects of Work Task 1.2).

NOTE 7:	On-path information exposure may require coordination with Work Task 1.1.

*** End of Changes ****