SA WG2 Meeting #172	S2-2510134

17 – 21 November, 2025, Dallas, USA	(revision of S2-250xxxx)

Source:	CATT

Title:	Update the agreed principles and add conclusion for KI#3

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This pCR is proposed to update the agreed principles and add conclusion for KI#3.

- Introduction

This pCR is proposed to update the agreed principles and add conclusion for KI#3.

**** First Change ****

7.1.3	Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection:

-	The Sensing Function is selected by Sensing Gateway Function based on NRF query.

-	The Sensing Function registers itself to the NRF, with its NF profile.

-	Sensing Function is selected based on at least its supported sensing area.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

-	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

-	The OAM configures the Sensing Function with the information of Sensing Entities, i.e. gNB, e.g. its address and supported sensing area.

-	The Sensing Entity is re-selected when, e.g. the sensing release is received from the existing Sensing Entity, the sensing target is going to move out of the supported sensing area of the existing Sensing Entity based on the trajectory of the sensing target.

-	The Sensing Function is re-selected when, e.g. the sensing failure is received from the existing Sensing Function, the sensing target moves out of the supported sensing area of the Sensing Function.

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

**** Second Change ****

8.1.x	Conclusions for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following conclusions for Key Issue #3 of sensing entity and sensing function discovery and (re-)selection are made:

-	The Sensing Function is selected by Sensing Gateway Function based on NRF query.

-	The Sensing Function registers itself to the NRF, with its NF profile.

-	Sensing Function is selected based on at least its supported sensing area.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

-	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

-	The OAM configures the Sensing Function with the information of Sensing Entities, i.e. gNB, e.g. its address and supported sensing area.

-	The Sensing Entity is re-selected when, e.g. the sensing release is received from the existing Sensing Entity, the sensing target is going to move out of the supported sensing area of the existing Sensing Entity based on the trajectory of the sensing target.

-	The Sensing Function is re-selected when, e.g. the sensing failure is received from the existing Sensing Function, the sensing target moves out of the supported sensing area of the Sensing Function.

**** End of Changes ****