SA WG2 Meeting #171	S2-2510507

17th – 21th November, 2025, Dallas, USA

Source:	ETRI

Title:	KI#1: update of agreed principles

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes updates on Agreed principles for KI#1 of TR 23.700-04.

1. Introduction

This contribution proposes updates on the agreed principles for KI#1 Transfer of data over UP for UE data.

2. Proposal

It is proposed to agree the following updates to TR 23.700-04.

*** Start of Change ***

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	A new 5GC entity called DCF is defined as the UP path termination entity to support the transfer of data over UP for UE data. The DCF will further transfer the received data to the AF for UE side model training. The DCF can be standalone or co-located with an existing function in 5GC

-	The DCF may send its user plane address information (i.e. IP address or FQDN) to the UE via NAS.

-	As part of the URSP rule, the operator may provide DCF(s) address information within the Traffic Descriptor to identify the collected UE data traffic, and provide dedicated PDU session parameters (e.g., DNN and S-NSSAI) within the Route Selection Descriptors to specify the required connection.

-	DCF sends UE data transfer control request to a UE over NAS or over UP including data collection parameters.

-	NEF discovers the DCF based on NF discovery procedure from NRF or local configuration.

*** End of Change ***