3GPP TSG-SA2 Meeting #172	S2-25xxyyy

# Dallas, US, 17 November 2025 – 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | Draft | rev | - | Current version: | 19.x.y |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indication of ECN marking for L4S to the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | C |  |  |  |  |  | Release: |  |  | 20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | ECN marking for L4S is enabled per QoS Flow in 5G. In a tethered mode of operation UE may be independently scheduling uplink traffic originating from multiple devices behind the UE. The knowledge of whether ECN marking for L4S is enabled for a QoS Flow in the 5GS is useful for UE to understand whether L4S techniques can be applied. Currently there is no explicit indication from the network to the UE whether a QoS Flow is enabled for L4S. As per the NOTE 3 in clause 5.37.3.1, a QoS Flow may be enabled for L4S statically or dynamically based on detection of L4S traffic. There are also differences between ECN marking performed in the NG-RAN and in the UPF. If the ECN marking is performed in the NG-RAN, then dedicated QoS Flow(s) are used for carrying L4S enabled IP traffic (either when it is statically enabled or dynamically enabled). In such cases, UE may be configured with packet filters which map L4S traffic to a QoS Flow. This can give an indication to the UE that L4S is enabled on a QoS Flow. If the ECN marking is performed in the UPF, a QoS Flow level ECN marking for L4S indicator is sent by the SMF to the UPF. There is no need to have a dedicated QoS Flow for L4S traffic. If ECN marking for L4S is statically enabled in the UPF, UE may not be configured with packet filters mapping L4S traffic to a QoS Flow. In this case UE may not be aware of the support for L4S on a QoS Flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed to add a new indication from SMF to the UE in N1 signalling to inform UE whether L4S is enabled in a QoS Flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature remains sub-optimal since UE may not be able to make use of L4S techniques since it may not be aware if ECN marking for L4S is enabled for a QoS Flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.502.. CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 5.37.3	Support of ECN marking for L4S to expose the congestion information

#### 5.37.3.1	General

L4S (Low Latency, Low Loss and Scalable Throughput) is described in IETF RFC 9330 [159], IETF RFC 9331 [160] and IETF RFC 9332 [161]. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaptation.

In 5G System, ECN marking for L4S may be supported. ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows. In the case of 3GPP access, ECN marking for the L4S in the IP header is supported in either the NG-RAN (see clause 5.37.3.2 and TS 38.300 [27]), or in the PSA UPF (see clause 5.37.3.3). In the case of untrusted/trusted non-3GPP access, ECN marking for L4S in the IP header is supported in the N3IWF/TNGF (see clause 5.37.3.4, clause 6.2.9 and clause 6.2.9A).

NOTE 1:	Based on operator's network configuration and policies, SMF decides whether NG-RAN or PSA UPF based ECN marking for L4S is used.

In the case of ECN marking for L4S by PSA UPF, the NG-RAN is instructed to perform congestion information monitoring and report to the PSA UPF the congestion information (i.e. a percentage of packets that UPF uses for ECN marking for L4S) of the QoS Flow on UL and/or DL directions via GTP-U header extension to PSA UPF and accordingly, the PSA UPF may mark the UL and/or DL direction packets of the QoS Flow.

NOTE 2:	As for any QoS Flow, QoS rules in the UE and PDRs in the PSA UPF control which packets are bound to the L4S enabled QoS flow. The Packet Filter Set in the QoS rule or PDR can use packet filter(s) in clause 5.7.6.2 (e.g. match packets with ECT(1) or CE (See RFC 9331 [160]) together with IP 5 tuple) to steer traffic to an L4S enabled QoS Flow.

NOTE 3:	A QoS Flow may be enabled with ECN marking for L4S requirement e.g. statically when a PDU session is established based on configuration in SMF or PCF, or dynamically based on detection of the L4S traffic or by requests by an AF.

NOTE 4:	For enabling ECN marking for L4S based on dynamic detection of L4S traffic, according to local configuration, SMF instructs UPF (as described in clause 5.4.11 of TS 29.244 [65] for Application Reporting) to detect L4S traffic (e.g. match packets with ECT(1) or CE (See RFC 9331 [160]) in the IP header) on a QoS Flow. Upon receiving report from UPF, the SMF establishes another QoS Flow enabled for L4S, executing the actions defined in clause 5.7.1.5 (as if a PCC rule enabling L4S is processed and an appropriate QoS profile is derived e.g. identical to the QoS Flow where the L4S traffic was detected) such that the detected L4S traffic is now mapped to the newly established QoS Flow. It is recommended that other traffic processing rules are aligned to those of the PDR of the QoS Flow where L4S traffic was detected (different configuration for specific traffic processing rules can exist).

NOTE 5:	To support this functionality, the UE needs to support L4S feedback as described in IETF RFC 9330 [159], which is not in the scope of 3GPP.

When serving PSA UPF or NG-RAN is changed e.g. due to inter-NG-RAN handover or PSA UPF relocation, target NG-RAN and target PSA UPF, if supported, should continue to perform ECN marking for L4S for the QoS Flow. However, if not available (i.e. ECN marking for L4S is not supported in both, target NG-RAN and target PSA UPF), AF should be notified that ECN marking for L4S can no longer be performed if ECN marking for L4S had been enabled for the QoS Flow based on AF request. When ECN marking for L4S is supported again either in target NG-RAN or in target PSA UPF, AF should be notified that ECN marking for L4S can be performed again if ECN marking for L4S had been enabled for the QoS Flow based on AF request.

When ECN marking for L4S is enabled for a QoS Flow, the SMF informs the UE through N1 signalling.

NOTE 6: The UE can use this information to decide whether CE marking can be applied on the uplink e.g., for any tethered traffic it is handling.

End of changes