3GPP TSG SA WG2 Meeting #172	S2-2510788

Dallas, US,  17 - 21 November 2025

# Source:	Apple

# Title:	New WID: Indication of ECN marking for L4S to the UE

# Document for:	Information

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Indication of ECN Marking for L4S to the UE

Acronym: TEI20_UE_L4S

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | x |  | X |  | x |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

L4S as described in IETF RFC 9330, RFC 9331 and RFC 9332 is supported in 5GS from Rel-18. This feature exposes congestion information by marking ECN bits in the header of the IP packets between UE and the application server to help with application layer/transport layer rate adaptation.

ECN marking for L4S can be enabled on a QoS Flow basis for the uplink and/or downlink direction. For 3GPP access, ECN marking for L4S in the IP header is supported either in the NG-RAN or PSA UPF. For non-3GPP access, ECN marking for L4S is supported in the N3IWF/TNGF in either direction. For Wireline access, ECN marking for L4S is also supported in W-AGF and 5G-RG (uplink only).

A UE may be scheduling uplink traffic from independent sources over Uu interface (e.g., in tethered mode of operation). In such cases, UE may have to rely on L4S techniques to provide feedback to the source applications for rate adaptation. The knowledge of whether ECN marking for L4S is enabled for a QoS Flow in the 5GS is useful for UE to understand whether L4S techniques can be applied on the traffic it is carrying. UE may then perform ECN marking on the uplink direction based on the congestion it has observed on the uplink direction. This scheme of UE performing CE marking is applicable for both 3GPP and non-3GPP access.

Currently there is no explicit indication from the network to the UE whether a QoS Flow is enabled for L4S in the uplink direction. As per the 3GPP TS 23.501, clause 5.37.3.1 NOTE 3, a QoS Flow may be enabled for L4S statically or dynamically based on detection of L4S traffic.

There are also differences between the cases where ECN marking is performed in the NG-RAN or in the UPF. If the ECN marking is performed in the NG-RAN, then dedicated QoS Flow(s) are used for carrying L4S enabled IP traffic (either when it is statically enabled or dynamically enabled). In such cases, UE may be configured with packet filters which map L4S traffic to a QoS Flow. This can give an indication to the UE that L4S is enabled on a QoS Flow.

However, if the ECN marking is performed in the UPF,  a QoS Flow level ECN marking for L4S indicator is sent by the SMF to the UPF. There is no need to have a dedicated QoS Flow for L4S traffic. If ECN marking for L4S is statically enabled in the UPF, UE may not be configured with packet filters mapping L4S traffic to a QoS Flow. In this case UE may not be aware of the support for L4S on a QoS Flow.

There is a need to define a uniform solution in 5GS to inform UE about enabling ECN marking for L4S on a QoS Flow such that UE can benefit from this.

# 4	Objective

This new WID aims to enable the following enhancements in 5GS.

WT 1: Enhance the support of L4S in 5GS to inform UE about QoS Flows that are enabled for ECN marking for L4S.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Adding description about informing enabling of ECN marking for L4S in a QoS Flow to the UE. | - |  |
| TS 23.502 | Update SM signalling for informing UE about L4S support per QoS Flow | - |  |

# 6	Work item Rapporteur(s)

Apple

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Apple