3GPP TSG SA WG2 Meeting #170	S2-2506359

25- 29 August, 2025, Goteborg, SE	(revision of S2-220xxxx)

Source:	China Mobile, AT&T, Boost Mobile Network, KDDI, KPN, Rakuten Mobile, vivo, CATT

Title:	[WT#5] Data framework design for 6G network

Document for:	Agreement

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: This paper proposes motivation of 6G architecture design to support efficient and scalable data handling through data framework.

- Justifications

1.1 SA1 requirements related to data service

In TR 22.870 clause 5.9.2, the requirements of efficient data collection and consumption for 6G system are proposed. Table 5.9.2.1-1 lists several potential use cases for data service in 6G, and illustrates the corresponding data type, data provider, data consumer, and data volume.

Table 5.9.2.1-1: Heterogeneous 6G System Data

| Use case category | Data types | Data provider | Data Consumer | Data volume |
| --- | --- | --- | --- | --- |
| Sensing data collection/sensing result exposure (e.g. measurement) | Sensing measurement data requires further study and discussion. The potential sensing measurement can be channel estimation results, delay, Doppler, angle, signal strength, etc. | UE, RAN | Sensing Function | Related to use cases: e.g. data rate of measurement is about 10Mbps for objects creating hazards on roads |
| AI/ML data collection / analytics exposure (e.g. AI/ML for air interface, AI/ML for core network) | - For RAN data collection for Air interface, see TR 38.843 [243] - For core network data collection, see the data collection framework in clause 6.2, TS 23.288 [114], and the detailed data type are defined from clause 6.3 to clause 6.21, TS 23.288 [114]. | CN, RAN, UE, AF, OAM | CN, RAN, UE, OAM | Typically, the internal data collection for AI model training yields tens of thousands of samples. Related to AI model: e.g. the data volume is about 10k bytes to 100M bytes for AI model of beam management |
| Energy Efficiency (EE information exposure to AF) | still under the discussion in SA2 R19 EnergySys WID | CN, OAM | EIF (Energy Information Function) | Related to use cases: e.g. could be similar to data volume for charging |
| Positioning, | See clause 6, TS 23.273 [240] | UE, RAN | LMF, UE | Related to use cases |
| Network Exposure in core network | See clause 5. 20, TS 23.501 [140], and clause 4.15, TS 23.502 [30] | 5GC NF, AF | 5GC NF, AF | Related to use cases |

As shown above, there are 5 main uses cases that related to data management in 6G, sensing data collection and exposure, AI/ML data collection and exposure, EE information exposure, Positioning data processing, and Network data exposure.

The commonality of the above use case should support data collection, storage and exposure.

Observation 1: In 6G SA1 requirement, Sensing data, AI data, EE data, positioning, and other network data, all of these require almost similar data handling mechanism including data collection, data distribution, data processing, data storage, and data exposure.

1.2 SA2 previous work for AI and sensing data handling

5GC network AI data collection and transmission: In SA2, the NWDAF has utilizes DCCF and ADRF for AI training data collection and storage. However, the interface between NWDAF and NF using SBI may not be suitable for large data transmission (e.g., UPF event exposure data per UE can be 1~5Mbps, while the SBI signalling is generally Kbps), therefore the data collection mechanism should be upgraded to support large AI training data collection.

In SA2 5G-A R20 AIML, the discussion of how to retrieve UE side training data is under discussion, and the requirement for core network is the training data can be visible or control by core network.

Sensing data transmission and exposure: In SA2 5G-A R20 sensing architecture design, two typical requirements are to study Sensing Data and the Associated Information Collection and Transport (KI#4), and Sensing Result Exposure (KI#5).

Observation 2: In 5G era, multiple data handling scenarios have variant handling mechanisms. In 6G, it is expected to define a unified data framework to simplify the network architecture and provide data services.

1.3 Impact for SA2 6G architecture design

6G network data has following characteristics:

-	Various data type: sensing data, AI data, other potential data e.g., positioning, EE;

-	New data provider: not only UE, but also RAN can provide the data to CN.

-	New data consumer: UE, RAN and NF can be the data consumer.

-	New data processing requirement: sensing data (especially retrieved from RAN) is expected to be processed in CN to get the sensing result; AI4NET case, the AI inference can be happened in CN or RAN to do the intelligent network optimization e.g., policy adjustment or mobility management.

Facing these new data characteristics, the 6G network capability of data handling needs to be extended based on the 5G network including: multi-source heterogeneous data collection, efficient and guaranteed data transmission, data processing within the network, and unified data storage.

In a word, 6G network will break through the "pipeline" capability and go towards to a new type of information service network by realizing diversified data collection, transmission, processing and storage, including

-	End-to-end data framework architecture design including UE, RAN, core network as both data service consumer and provider.

Observation 3: The 6G data framework is an end-to-end architecture including UE, RAN, and core network as both data service consumer and provider.

1.4 Gap analysis between potential SA2 data management in 6G and existing SA5 data management.

Based on 6G SA1 requirements related to data service in clause 1.1 and existing SA2 architecture and mechanism, it is necessary to design a common data framework to manage data in 6G. SA5 has discussed data management in 5G and also designs a data management system for Management data. However, there are differences between potential SA2 data management in 6G and existing SA5 management, which is listed in Table 1.1-3. The Management data specified by 3GPP for 5G management is classified into 5G performance measurements as defined by TS 28.552, 5G end to end key performance indicators as defined by TS 28.554 , Trace metrics as defined by TS 32.423 and alarms data as defined in TS 28.545.

Table 1.1-3: Different data management between SA2 and SA5

|  | SA2 | SA5 |
| --- | --- | --- |
| Data type | AI model/training data; sensing data; other dynamic policy/configuration information | Management data, including configuration/performance data, NF status data/information, KPIs |
| Data source | From UE, RAN, NFs(e.g., UPF other CP NFs) | UE, RAN ,NFs |
| Handling feature | Store, processing, exposure; Consumer can be AF, UE or RAN | Store, processing, exposure; potential consumer is NWDAF, RAN, operator management system or AF |

As shown above, the data type in SA2 data management and SA5 data management are for different purpose, while the SA5 does not support data consumption from UE.

Observation 4: The SA2 and SA5 handle different data types and for different purpose.

**** First Change ****

Annex A.X. WT#X Scope

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination

**** Second Change ****

5.X. Key Issue #X: Support of data framework for efficient data handling

This key issue will address the system architecture and function required to provide end-to-end support for data framework to achieve common data framework for all aspects related to efficient and scalable data handling including data collection, distribution, processing, storage, data access and data exposure. The aspects to be studied in this key issue include:

-  What specific data should be handled e.g., sensing data and AI data.

-	What data handling functionality should be supported, e.g., data collection, processing, distribution, storage and exposure.

-	How to control the data collection, e.g., select the data provider, and what kind of collection feature should be supported e.g., frequently reporting.

- 	What kind of efficient data handling method should be studied e.g., data publishment.

- 	How to support data exposure to UE, RAN or AF/AS.

NOTE 1:  The security and privacy aspect should be considered.

- 	How to support system and procedure design with considering user consent requirement.

NOTE 2:	Coordination with SA3, SA5 and RAN WGs are required.

NOTE 3:  The charging aspect of data framework depending on SA5.