SA WG2 Meeting #170	S2-2506466

25 – 29 Aug, 2025, Goteborg , SE

Source:	China Telecom

Title:	[WT#1.2, SBA famework] Network simplification

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: It is proposed to study simplified SBA framework in 6G SID. This  contribution analyses issues and difficulties regarding the complexity of 5G SBA network and the necessity to study a new 6G SBA framework with flexibility, scalability and resilience by means of simplifying the overall system.

1. Justifications

Architectural Motivation

As described in clause 5.9 of TR 22.870, there are potential requirements on the 6G system, e.g. "The 6G system shall be able to support means to simplify network operation and service delivery compared to 5G system, e.g. in order to reduce signalling, minimize connection setup time". These requirements comes from the experiences and lessons from years of operation of complex 5G system.

One idea of the 5G system adopting SBA framework is that the services of the same NF can be invoked by multiple NFs, thereby enhancing interface efficiency and network operation efficiency. However, in practice of 5G SBA network, the service consumers of some NF's service-based interfaces are fixed. For example, the PDU session service of SMF is only used by AMF. Another example is that the AM policy of PCF is only invoked by AMF. This indicates that the division of some NF functions need to be optimized and requires redesign or reconstruction of NF functions. Considering that 6G network will introduce beyond-connectivity services, e.g. sensing services and AI services, and it may face similar situations, the definition of network functions , the service-based interfaces and procedure should be considered comprehensively to avoid some services being used only between two NFs or few NFs. On the other hand, the NFs in the 5GC control plane are interrelated, and the implementation of a new feature or capability requires procedures crossing multiple NFs. Over 50% of features in the current 5GC network involve the collaboration of multiple NFs, indicating that the 5GC control plane design has the possibility to be optimized towards a simplified 6G network control plane.

To achieve flexibility in network deployment, 5G networks have defined dozens of NFs and interfaces, which has brought challenges to network deployment and operation. During the deployment phase, IoT testing between NFs from different vendors is required before the functionality launch, which can lead to a delay of new features; In the operation phase, after a network failure occurs, trouble-shooting takes longer time due to the large number of network nodes, Multiple accidents that have occurred globally have proven the severity of this issue. At the same time, considering that 6G networks will also support services beyond connectivity, network functions will become more complex, and it becomes more important to research how to define the scope of NF functionality. Therefore, In the design of 6G network architecture, we should learn from the operational experience of 5GC, research how to define NF functions and services between NFs. It should be studied to determine which signalling need to support multi-vendor interoperability and which signalling can be handled internally within a vendor’s system. Only the signalling that need to support multi-vendor interoperability should be standardized to avoid overcomplicating network architecture and deployment solutions.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *

Annex A.X. WT#1.2 Scope

Define the 6G SBA architecture, the high-level functionalities and appropriate granularity of NFs, with flexibility, scalability and resilience by means of simplifying the overall system and interoperability within the network. The study can be based on reconstruction and simplification of 5G SBA architecture, with considering new 6G functionalities as required in other WTs, e.g. new 6G functionalities to support beyond connectivity services.

# * * * * Second change * * * *

5.X. Key Issue #X: Network simplification

To simplify the 6G SBA framework, the solutions should study the following Key Issues:

KI#X.1: How to define the functionality of network elements to improve the utilization of service-based interfaces, reduce procedure redundancy, and enhance communication efficiency

KI#X.2: How to design the granularity of NF in the network to keep balance between flexibility of deployment and simplification of interoperability between different NFs within the network.

# * * * * End of changes * * * *