3GPP TSG-WG SA2 Meeting #170	S2-2506705

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source: 	Huawei, HiSilicon

Title: 	KI#2, #3, Sol#2: Update on PDU Session Selection and UE Selection

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: It is proposed to update the solution#2 considering PDU Session / UE to be selected for energy saving.

1	Introduction

In the Solution#2, PCF updates the policy of PDU Session/SDF according to the EnC threshold:

4. Once the EnC for the slice or 5GC exceed the EnC thresholds, the PCF decides to apply service adjustment for PDU sessions or SDFs in the slice or 5GC. 5. The PCF may select the PDU session or SDF from UEs with energy saving indicator which allows energy saving operations. For each affected PDU session where service adjustment can be applied like applying lower Session-AMBR or lower SDF GBR:

However, selecting all the PDU sessions/SDFs in the slice for service adjustment is inefficient. Thus, it is proposed to select PDU sessions/SDFs considering the status related to the PDU session (e.g. UE reachability, Access Network info, Access usability, etc.)

Besides, as described in clause 6.2.3, Apply/revoke the AM policy by monitoring the EnC according the EnC policy per slice, But the AM policy related procedure is missing. It is proposed to add a procedure of PCF initiated AM Policy Association Modification enabling PCF to adjust UE-slice-MBR or UE-AMBR based on EnC.

2	Proposal

It is proposed to capture the following changes vs. TR 23.700-67.

# * * * * First change * * * *

6.2	Solution #2: Policy Control Considering Slice or 5GC Energy Consumption

6.2.0	High-level solution principles

The solution follows the following principles:

-	Existing 5G MM, SM and BDT procedure can be enhanced for energy saving based on coarse-grained energy consumption (EnC) information.

-	Slice EnC can be used to trigger service adjustments on all or some slice users.

NOTE:	The energy-related information at UE or NF level used in KI#2 and KI#3 should be based on robust and accurate enough estimates to be fit for purpose.

6.2.1	Functional Description

This solution addresses KI#1 and KI#2.

Existing 5G MM, SM and BDT procedure can be enhanced for energy saving based on more coarse-grained energy consumption (EnC) information. Using coarser grained EnC information like the slice EnC to trigger service adjustments on all or some slice users offers a robust mechanism and actions may be applied on a sufficient number of users/sessions to have noticeable energy savings.

The AM policy, SM policy and/or BDT policy created by the PCF could consider the requirements of per slice EnC. The slice level EnC thresholds related policy can be added in slice control policy data defined in clause 5.2.15 of TS 29.519 [6].

The PCF retrieves the slice level EnC from the OAM system (using SA5 defined services) and compares the current consumption with thresholds defines in the policy data. Once EnC exceeds the thresholds, the PCF performs AM policy, SM policy adjustment or BDT policy (re-)selection. The actions can be applying alternative lower UE-AMBR for the UEs, applying alternative lower Session-AMBR for session for the PDU sessions etc. The adjustment can be applied to new UE registration or new PDU session (PDU Session establishment) or applied to existing UE registrations and PDU sessions (PDU Session modification). The PCF may consider UE energy saving indicator subscription and only apply service adjustment to UEs which allows energy saving operation instead of all UEs in a slice.

NOTE 1:	Policy modifications should not violate Slice SLAs (when applicable).

NOTE 2:	Whether the solution is to become part of normative work will be based on evaluation of the efficacy of the proposed selection of UEs to be affected by service adjustment and whether the service adjustments provide energy saving in an application-agnostic manner.

6.2.2	Procedures

6.2.2.0	General

The PCF can apply service adjustment based on the triggers mentioned in clause 6.2.1 for various procedure. The clause only describes the enhancement for PCF initiated SM Policy Association Modification procedure as an example on how the procedure is look like.

6.2.2.1	PCF initiated SM Policy Association Modification

The PCF may initiate the SM Policy Association Modification procedure when the EnC per slice exceed thresholds.

Figure 6.2.2.1: PCF initiated SM Policy Association Modification

A SM Policy Association is already established in PCF as described in clause 4.16.4 of TS 23.502 [3] before the SM Policy Association Modification procedure can be triggered.

1.	The PCF retrieves the slice policy related to EnC control trigger. The policy should contain the EnC threshold for triggering and revoking service adjustment or may optionally contains time limit for the service adjustment duration.

2.	The PCF retrieves the slice EnC for each slice that is in used from the OAM system periodically (e.g. per 15 minutes).

3.	PCF compares the EnC value to the EnC threshold for triggering service adjustment upon receipt of the EnC.

4.	Once the EnC for the slice or 5GC exceed the EnC thresholds, the PCF decides to apply service adjustment for PDU sessions or SDFs in the slice or 5GC.

5.	The PCF may select the PDU session or SDF from UEs with energy saving indicator which allows energy saving operations.

NOTE 1:	To not cause unexpected signalling with energy consumption, the PCF may not apply energy saving operations to all related PDU Session(s). Instead, the PCF may only select the activated PDU Session(s) belong to the UE(s) that is reachable, connecting via 3GPP access etc.

For each selected PDU session where service adjustment can be applied like applying lower Session-AMBR or lower SDF GBR:

5a.	The PCF issues a Npcf_SMPolicyControl_UpdateNotify request to the SMF with possibly updated policy information about the PDU Sessions or SDFs.

5b.	The SMF acknowledges the PCF request with a Npcf_SMPolicyControl_UpdateNotify response.

NOTE 2:	The energy saving indicator (within the UE AM subscription) is obtained from AMF via SMF.

NOTE 3:	The exact Session-AMBR or SDF GBR is the implementation of PCF, e.g., based on the number of EnC exceeding the EnC Threshold.

6.	The PCF monitors the slice or 5GC EnC and compares EnC to the thresholds as described in step 2 and step 3. The PCF decides to revoke the service adjustment when:

-	The EnC for the slice or 5GC fall below the EnC threshold for revoking service adjustment.

7.	For each PDU session which requires revoking service adjustment:

7a.	The PCF issues a Npcf_SMPolicyControl_UpdateNotify request with revoked policy information about the PDU Sessions or SDFs to the SMF.

7b.	The SMF acknowledges the PCF request with a Npcf_SMPolicyControl_UpdateNotify response.

6.2.2.2	PCF initiated AM Policy Association Modification

The PCF may initiate the AM Policy Association Modification procedure when the EnC per slice exceed thresholds.

Figure 6.2.2.1: PCF initiated AM Policy Association Modification

An AM Policy Association is already established in PCF as described in clause 4.16.1 of TS 23.502 [3] before the AM Policy Association Modification procedure can be triggered.

1.	The PCF retrieves the slice policy related to EnC control trigger. The policy should contain the EnC threshold for triggering and revoking service adjustment or may optionally contains time limit for the service adjustment duration.

2.	The PCF retrieves the slice EnC for each slice that is in used from the OAM system periodically (e.g. per 15 minutes).

3.	PCF compares the EnC value to the EnC threshold for triggering service adjustment upon receipt of the EnC.

4.	Once the EnC for the slice or 5GC exceed the EnC thresholds, the PCF decides to apply service adjustment for UEs in the slice or 5GC.

5.	The PCF may select the UEs with energy saving indicator which allows energy saving operations.

NOTE 1:	To not cause unexpected signalling with energy consumption, the PCF may not apply energy saving operations to all related UE(s). Instead, the PCF may only select the UE(s) that is e.g., reachable, connected, connecting via 3GPP access etc.

For each selected UE where service adjustment can be applied like applying lower UE-AMBR or lower UE-Slice-MBR:

5a.	The PCF issues a Npcf_AMPolicyControl_UpdateNotify request to the AMF with possibly updated policy information about the AM policy.

5b.	The AMF acknowledges the PCF request with a Npcf_AMPolicyControl_UpdateNotify response.

NOTE 2:	The exact UE-Slice-MBR or UE-AMBR is the implementation of PCF, e.g., based on the number of EnC exceeding the EnC Threshold.

6.	The PCF monitors the slice or 5GC EnC and compares EnC to the thresholds as described in step 2 and step 3. The PCF decides to revoke the service adjustment when:

-	The EnC for the slice or 5GC fall below the EnC threshold for revoking service adjustment.

7.	For each UE which requires revoking service adjustment:

7a.	The PCF issues a Npcf_AMPolicyControl_UpdateNotify request with revoked policy information about the UEs to the AMF.

7b.	The AMF acknowledges the PCF request with a Npcf_AMPolicyControl_UpdateNotify response.

6.2.3	Impacts on existing services, entities and interfaces

UDR:

-	Store and provide slice level energy control policy data.

PCF:

-	Collect slice level EnC from OAM (using SA WG5 defined services). Compare the collected EnC to the thresholds in the policy data.

-	Apply/revoke the AM policy, SM policy and/or BDT policy by monitoring the EnC according the EnC policy per slice.

# * * * * End of changes * * * *