3GPP TSG-SA2 Meeting #171	S2-2508207

# Wuhan, China, 13 – 17 October

Source: 	vivo

Title: 	KI#1: Interim agreements update

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to update interim agreements for KI#1: Transfer of data over UP for UE data collection.

1	Discussion

Some interim principles are proposed for KI#1: Transfer of data over UP for UE data collection.

2. Proposal

It is proposed to add the following changes to TR 23.700-04.

Start of Change

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	Subject to user consent, the 5GC NF (i.e., DCCF) is utilized to control the standardized data transferring from UE to data consumer, i.e. UE model training entity server.

-	A UP path is used between the UE and the 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	The 5GC NF sends the UP information for data reporting (i.e. IP address or FQDN of the 5GC NF) to the UE via NAS signalling, which is used by the UE to establishe the UP path between the UE and the 5GC NF.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules. The URSP includes the FQDN/target address for data reporting in TD and the dedicated DNN/S-NSSAI in RSD.

-	The 5GC NF sends the Data Transfer Request with specific requirements (e.g. data type, reporting time, data format and etc) to the UE (either via the control plane along with the UP information or via the user plane), according to data request from the data consumer.

-	The UE reports the collected data to the 5GC NF via the UP path, based on the requirements received in the Data Transfer Request.

NOTE: which standardized data can be collected by the UE will be defined by RAN1. The mechanism how UE collects data and corresponding RRC configuration depend on RAN1 and RAN2’s decision.

-	The 5GC NF checks the reported UE data by comparing them with the specific requirements (e.g., data type, data format).

-	The 5GC NF exposes the data that meet the requirements to the data consumer with the authorization of NEF.

-	The 5GC NF may store the collected data from UEs in ADRF.

Next Change

7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	How the 5GC NF controls UE data transfer.

-	Whether and how the UE controls UE data transfer.

-	Whether and how to select UEs for UE data collection.

-	Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).

-	Whether the UE needs to be authorized for UE data collection.

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

End of Changes