3GPP SA WG2 Meeting #171	S2-2508226

13 – 17 Oct, 2025, Wuhan, China

Source:	NEC

Title:	[WT#7] Scope and key issue on NTN

Document for:	Agreement

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

This contribution proposes scope and key issue to Work task 7 of the 6G study on NTN.

1. Justifications

This contribution discusses the scope and proposes a corresponding key issue to facilitate work on Work Task #7 of the approved 6G SID (SP-250806):

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 8:	The detailed scope for WT#7 will be coordinated and aligned with RAN

The contribution builds on the baseline of S2-2508134, which was postponed at the end of SA2#170.

Architectural Motivation: 6G NTN is expected to support multi-orbit satellite access and the mobility, including between LEO/MEO/GEO satellites. The harmonization between NTN and TN for access and mobility including radio design may be a 6G study point.

The satellite access technology has been specified in 4G/5G, enabling a UE to receive communication services from satellites that are deployed at different altitudes by means of transparent or regenerative payload. Normal communication mode and store-and-forward communication mode have been supported to provide data transmission in connected and intermittent network environments, respectively. However, a LEO satellite, which can provide lower delay and higher bandwidth, provides network coverage only in a certain area for approximately several minutes due to its faster moving speed. At the same time, frequent handovers cause interruptions, which can degrade performance of  some applications.

To improve the services availability via NTN, it is common to deploy multiple satellites for continuous coverage. They can be in the same or different orbits. The satellite in the same orbit can be interconnected with each other and can constitute one type of “constellation”.

Gap Analysis: 5G satellite solution does not support multi-orbit satellite access and the harmonization between NTN and TN, including radio design.

A “constellation” can provide broader network coverage and a more persistent communication service than a single satellite, including both the connected and intermittent communication service modes. However, it is unknown how to define the functions of each satellite, a UE and a ground network and how to support service continuity for a UE in a “constellation”. In a “constellation”, both a UE and a satellite are moving. The UE handover between satellites may trigger signalling messages between the UE and the satellites, namely the source and the target satellite, as well as between the satellite and the ground network. Moreover, there are multiple ground networks; the handover procedure of a satellite in a “constellation” between two ground networks should also be taken into account.

Technical Impact: How to support multi-orbit satellite access and the harmonization between NTN and TN.

How would this WT/KI influence:

•	Improvements to ensure efficient mobility and service continuity for UEs in a “constellation”, which includes:

-	the UE handover between satellites in a “constellation”, including signalling interaction between the UE and the satellites, the source and the target satellite, as well as between the satellite and the ground network;

-	the handover between the ground network for a satellite in a “constellation”, including signalling interaction between the UE and the satellite, as well as the satellite and the ground network.

2. Proposal

This contribution proposes to update the annex of TR 23.801-01 to include the scope of WT#7 and the related Key Issue.

**** Tracked changes on top of S2-2508134 ****

**** First Change – all text new ****

Annex A.X 	WT#7 Scope

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN

NOTE 2:	The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G.

**** Second Change – all text new ****

5.X. Key Issue #X: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects

5.X. Key Issue #Y(7.1): study how to support 6G RAT for NTN in 6G system

This key issue will study how to support 6G RAT for NTN in 6G system, especially the following aspects:

- Support for 6G satellite access in the 6G system.

- Support of both transparent and regenerative payload.

- Support of Store and Forward (S&F) mode.

- Support for policy/charging differentiation and (re)selection among TN and satellite accesses with different orbit types (LEO, MEO, GEO).

- Lawful Interception.

- Study how to mitigate the effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance.

NOTE 1: The detailed scope on supporting 6G RAT for NTN will be coordinated and aligned with RAN.

NOTE 2: The detailed scope on the overall 6G architecture will be coordinated and aligned with key issues related to WT#1.

NOTE 3:  Whether and how to re-use or enhance the 4G/5G satellite specific functions(e.g. earth fixed tracking areas, satellite specific RAT types etc.) should be evaluated.

NOTE 4:  The detailed scope of Lawful Interception will be coordinated and aligned with SA3-LI.

NOTE 5:  The deployment of CN functionalities on-board a satellite should support the optimization of the UP path (e.g. by the Inter-satellite link) and take the deployment cost into consideration.

NOTE 6:  The mechanism design to tackle intermittent connectivity via NTN should follow the general principle designed to tackle intermittent connectivity via TN.

NOTE 7:  The policy/charging including the (re)selection among TN and satellite accesses with different orbit should be coordinated and aligned with general policy/charging framework in WT#1.2 and CT1/RAN2’s conclusions.

5.X. Key Issue #Z(7.2): How to support mobility and service continuity for NTN

This key issue will study how to support mobility and service continuity for NTN due to user mobility or other network conditions, especially the following aspects:

- Whether and how to enhance mobility and service continuity between TN and NTN, including simultaneous connectivity via TN and NTN (intra- and inter-PLMN cases).

- Whether and how to enhance mobility and service continuity between the satellite accesses with the same and different orbit types (LEO, MEO, GEO).

5.X. Key Issue #M(7.3):Whether and how to support interworking with legacy system/RAT

This key issue will study whether and how to support interworking for NTN with legacy system/RAT, especially the following aspects:

- Whether and how to support interworking with legacy system/RAT.

NOTE:	 How to handle interworking with legacy system/RAT will be coordinated and aligned with key issues related to WT#2.

**** End of Changes ****