3GPP SA WG2 Meeting #171	S2-2508228

Wuhan, CN, 13 – 17 Oct 2025	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6424 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indirect Network Sharing for disaster condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_NetShare_Ph2-ARC |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Indirect Network Sharing can be used for the UE of the participating operator with the disaster condition to reduce the service interruption. In the case that the RAN of participating operator may be broken down during the disaster condition, the shared RAN of the hosting operator can broadcast the PLMN ID representing the participating operator so that the UE of participating operator can access the network via Indirect Network Sharing. In order to support the Indirect Network Sharing for disaster condition, the NFs of participating operator network (i.e. UDM, AUSF, SMF) should be aware of which UE(s) are in disaster condition via the message sent from the hosting operator network instead of in the normal Indirect Network Sharing case. This CR proposes to introduce and describe the Indirect Network Sharing for disaster condition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the Indirect Network Sharing for disaster condition and describe the related function enhancement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Indirect Network Sharing for disaster condition cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (All the text is new)

5.18.x	Indirect Network Sharing for disaster condition

Subject to operator policy and national/regional regulations, the Indirect Network Sharing can be used for disaster condition with minimization of service interruption. In the case that the RAN of the participating operator is broken down during the disaster condition, the shared RAN of the hosting operator broadcasts the PLMN ID representing the participating operator so that the UE of the participating operator can access the network via Indirect Network Sharing.

NOTE 1:  In the case of the Indirect Network Sharing for disaster condition, the UE of the participating operator will select the PLMN ID representing the participating operator broadcast by the shared RAN as specified in the clause 5.18.3, thus the UE does not need to be aware of disaster condition.

NOTE 2:	How the hosting PLMN is notified that another PLMN (as the participating operator network) is a PLMN with disaster condition and how the hosting PLMN is notified of the area where the disaster condition applies is out of scope of 3GPP.

When the disaster condition arises, the AMF of the hosting operator’s network receives the NAS Registration Request message sent by the UE of the participating operator. The AMF identifies the UE registering to the network via the Indirect Network Sharing for disaster condition by checking if the UE is from a participating operator that can use Indirect Network Sharing for disaster condition and the UE is registering to the network via the Indirect Network Sharing in the area with disaster condition. If the above conditions are satisfied, comparing to the normal Indirect Network Sharing case, the AMF provides the additional “Indirect Network Sharing for disaster condition indication” to AUSF and UDM during the registration procedure. In addition, the AMF also provides this indication to V-SMF during the PDU session related procedure.

NOTE 3:  In the case that more than one hosting operators can provide the Indirect Network Sharing for disaster condition for the participating operator in the disaster area, the existing congestion control mechanism can be used to distribute the UE access to the different hosting operator networks in order to avoid the overload condition of each hosting operator network.

NOTE 4:  The access class barring can be used to ensure the priority of the specific UE access during the disaster condition.

End of CHANGES