3GPP TSG-WG SA2 Meeting #171	S2-2508237

Wuhan, CN, 13th – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2: Conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#2 on authorization is proposed in this document.

# 1. Introduction

Based on the discussions and analyses in S2-2508235 and the architectural conclusion of KI#1 in S2-2508236, this paper provides the conclusion on KI#2 on Authorization and Revocation to Support Sensing Service.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * * all new text

### 7.1.2	Agreed Principles for KI#2 Authorization and Revocation to Support Sensing Service

The following principles are agreed for KI#2:

a)-	The Sensing Service authorization consists of two steps:

1)	Authorisation of the AF performed by the NEF;

2)	Authorisation of the AF's Sensing Service Request performed by the Sensing Control Function;

b)	The NEF authorises the AF as Sensing Service Consumer as defined in TS 23.502 [10].

c)	If the AF is authorised to request sensing service by NEF, the NEF forwards the request to the Sensing Control Function which determines whether the AF is authorized to the Sensing Service Request considering the AF's authorisation information described in the table 7.1.2-1.

Table 7.1.2-1: AF Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to trigger the sensing services operations. If not present, this indicates that all area are allowed if not explicitly excluded via Not allowed area parameter (Note 1) |
| Not allowed area | Indicate the not allowed area for the indicated AF to trigger the sensing services operations. If not present, this indicates that all area are allowed (Note 1) |
| Allowed time period | Indicate the allowed within which AF can trigger a particular sensing service operation |
| Note 1: the Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

NOTE 1:	The list of parameters in service request considered for authorisation will be determined in detail during normative phase.

d)	The information for the authorisation of Sensing Service Request is stored in NEF for authorisation of whether the AF is allowed to request Sensing Service and in Sensing Control Function for authorisation of the sensing request;

e)	If the authorisation is not granted, the SCF sends a response with reject cause to the requesting AF.

f)	At any time the SCF may determine to revoke the Sensing Service Request when the authorisation criteria are no longer met, for example changes of the Target Sensing Service Area from allowed to not allowed, or changes of SLA, etc. The Sensing Control Function notifies the AF that the authorisation has been revoked and cancels all relevant sensing procedures related to the Sensing Service Request whose authorisation has been revoked.

g)	The authorisation and revocation procedures are not applicable to the gNB as Sensing Entities which is assumed to always be authorised to perform sensing operation, since it is deployed by operator.

The Figure 7.1.2-1 depicts the procedure for authorisation

Figure 7.1.2-1: Procedure for authorization.

1.	The AF sends a Sensing Service Request to the NEF.

2.	The NEF authorises the AF as Sensing Service Consumer as defined in TS 23.502 [10].

3.	The NEF forwards the Sensing Service Request to the Sensing Control Function.

4.	The Sensing Control Function determines whether the AF is authorised to perform the Sensing Service Request considering the AF's authorisation information described in the Table 7.1.2-1.

The Figure 7.1.2-2 depicts the procedure for the revocation of granted authorisation.

Figure 7.1.2-2: Procedure for the revocation of granted authorisation.

1a.	The NEF may decide that the authorisation previously granted to the AF is invalid any more, e.g. due to change of SLA, and send the revocation request to the Sensing Control Function and continues with step 2.

1b.	The Sensing Control Function may decide that the authorisation previously granted to the AF is invalid any more, for example due to change of SLA, the previously authorized service area is no longer authorized, etc and so the sensing service has to be stopped. In case that AF has been granted for different Sensing Service Requests, the Sensing Control function may decide to revoke the authorisation for all requests or some of the requests based on the Target Sensing Service Areas of the requests. For example the AF has been authorised for different Target Sensing Service Areas and for some of them are no more authorised.

2.	Sensing Control Function cancels all relevant sensing procedures related to the Sensing Service Requests whose authorisation have been revoked in step 1a or 1b.

3.	The Sensing Control Function sends a Response to NEF if has received the request from NEF (step 1a) or a Notification to the NEF related to the sensing service(s) which has been revoked (step 1b).

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Principles in clause 7.1.2 are agreed as the conclusion.

# * * * * End of changes * * * *