3GPP TSG-WG SA2 Meeting #171	S2-2508239

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#4: Conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#4 on authorization is proposed in this document.

# 1. Introduction

Based on the discussions and analyses in S2-2508235 and the architectural conclusion of KI#1 in S2-2508236, this paper provides the conclusion on KI#4 Sensing Data and the Associated Information Collection and Transport.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * *

### 7.1.4	Agreed Principles for KI#4 Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

a)	The gNB sends the collected 3GPP sensing data containing intermediate data (i.e., Point Cloud format) to the Sensing Processing Function via data connection for Sensing Result generation.

b)-	The Point Cloud format is the list of the reflection of the object and at least consists of the position and velocity for each point. The description in Geographical Area Description (GAD) as defined in TS 23.032 [xx] can be considered as the starting point.

NOTE 1:	The detailed information provided by the gNB will be determined in coordination with RAN WG3 and specified during the normative phase.

NOTE 2:	Whether the description in TS 23.032 [xx] needs to be extended to fulfill sensing needs will be considered in normative phase.

c)-	The 3GPP Sensing Data for a certain Sensing Service Area will be shared among the different sensing service. That means:

1)-	If the Sensing Service Request received by the SCF matches any on-going Sensing Service Request (e.g., they share the same Sensing Track Area/Sensing Cell), SCF can determine to reuse 3GPP Sensing Data of the on-going Sensing Service Request. SCF may further update the on-going collection for fulfilling the new requirement (e.g., higher accuracy).

d)	If there's no existing data connection between gNB and the Sensing Processing Function, a data connection needs to be established.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Principles in clause 7.1.4 are agreed as the conclusion.

# * * * * End of changes * * * *