SA WG2 Meeting #172	S2-2508251

# 13 - 17, October, 2025, Wuhan, China

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1515 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on ADRF related features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-9-8 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | t is clear that DSC information falls outside the scope of 3GPP standardization and, therefore, does not need to be detailed in the note under Clause 6.2B.2. Additionally, the terminology in various parts of Clause 6.2B.2, including the figure and the corresponding step descriptions should consistently use the phrase “data and/or analytics”. As agreed in the last meeting, the ADRF downloads the model via an address provided by the NWDAF and stores the model. In parallel, the ADRF may also generate and store an address of model itself for potential future retrieval of the model by the NWDAF. Therefore, Clause 10.3.3 should be further clarified to reflect this behavior. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Refine the note of DSC information. Refine the figure and step descriptions of 6.2B.2 with phrase “data and/or analytics”. Adding a note to avoid confusion in clause 10.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion persist in the normative text of ADRF related features. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2, 10.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.2B.2	Historical Data and Analytics storage

The procedure depicted in figure 6.2B.2-1 is used by consumers (e.g. NWDAF, DCCF or MFAF) to store historical data and/or analytics, i.e. data and/or analytics related to past time period that has been obtained by the consumer. After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an ADRF. Whether the consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.

The consumer may include in the storage request the DataSetTag attribute which the data records are to be associated to when stored by ADRF. The DataSetTag attribute is defined in Table 6.2B-1. Data records can be associated to multiple DataSetTag attributes.

Table 6.2B-1: DataSetTag attribute

| Information | Description |
| --- | --- |
| DataSetTag | Identifies the data set. |
| DataSetDescription | Provides human-readable information about the characteristics of the data set. |

The consumer may include in the storage request the Data Synthesis and Compression (DSC) information.

NOTE: The details of DSC information are up to implementation, which are out of 3GPP scope and are not standardized.

Figure 6.2B.2-1: Historical Data and Analytics storage

0a-c.	NWDAF, DCCF or ADRF are configured with default operator storage policies as described in clause 5B.1.

1.	The consumer sends data and/or analytics to the ADRF by invoking the Nadrf_DataManagement_StorageRequest (collected data with timestamp, analytics with timestamp, Service Operation, Analytics Specification or Data Specification, Storage Handling information, optionally DataSetTag, optionally DSC information) service operation. The NWDAF or DCCF may provide notification endpoint information to the ADRF for use by the ADRF to send notifications (implicit subscription) alerting the DCCF or NWDAF that data are about to be deleted (see step 6).

2a-c.	Based on Storage Handling information (if available) and Storage Policy, the ADRF, DCCF or NWDAF determines the Storage Approach (lifetime for storing data and whether consumer is notified prior to data and/or analytics deletion).

3.	The ADRF stores the data and/or analytics sent by the consumer. The ADRF may, based on implementation, determine whether the same data and/or analytics is already stored or being stored based on the information sent in step 1 by the consumer NF and, if the data and/or analytics is already stored or being stored in the ADRF, the ADRF decides to not store again the data and/or analytics sent by the consumer. If the DataSetTag attribute is included for data and/or analytics already stored or being stored, then ADRF associates the data and/or analytics records with such DataSetTag.

4.	The ADRF sends Nadrf_DataManagement_StorageRequest Response message to the consumer indicating that data and/or analytics is stored, whether the ADRF determined at step 3 that data and/or analytics is already stored and the Storage Approach.

Conditional on ADRF Managing the Storage Approach

5.	The ADRF determines that the lifetime of the stored data and/or analytics has expired (according to the Storage Approach).

6.	If indicated by the Storage Approach, the ADRF sends a notification alerting the DCCF or NWDAF that data and/or analytics are about to be deleted.

NOTE:	This is an implicit subscription.

7.	The DCCF or NWDAF indicate in the response to the ADRF whether they will retrieve the Data and/or Analytics.

8	The DCCF or NWDAF may retrieve the Data and/or Analytics from the ADRF as described in clause 5B.1.

Conditional on the DCCF or NWDAF Managing the Storage Approach

9	The NWDAF or DCCF determine that the lifetime of the stored data and/or analytics has expired (according to the Storage Approach).

10. The NWDAF or DCCF optionally retrieve the data and/or analytics from the ADRF as described in clause 5B.1.

11.	The NWDAF or DCCF request that the data and/or analytics be deleted from the ADRF.

12.	The ADRF deletes the data and/or analytics if:

-	the Storage Approach in the ADRF indicates alerting the consumer is not required prior to data and/or analytics deletion;

-	in step 7 the DCCF or NWDAF indicated data and/or analytics will not be retrieved prior to deletion;

-	data and/or analytics retrieval in step 8 has completed; or

-	A request to delete data and/or analytics was received in step 11.

If the ADRF received a response from the NWDAF or DCCF in step 7 indicating data and/or analytics will be retrieved but retrieval is not initiated before an adequate fixed time has elapsed, the ADRF may autonomously delete the data and/or analytics.

* * * 	Next Change * * *

10.3.3	Nadrf_MLModelManagement_Delete service operation

Service operation name: Nadrf_MLModelManagement_Delete

Description: This service operation instructs the ADRF to delete stored ML Model file(s) or ML Model address(es).

NOTE: The ML Model address(es) to be deleted in this service is generated by ADRF, not the address(es) given by NWDAF containing MTLF in the ML Model storage request.

Inputs, Required:

-	Storage Transaction Identifier; or

-	one or more unique ML Model identifier(s).

When a Storage Transaction Identifier is given, ADRF shall delete all the ML models that stored in the corresponding storage transaction.

Inputs, Optional: None.

Outputs, Required: One of the following:

-	Operation execution result indication (i.e. ML Model deleted, ML Model not found, ML Model found but not deleted), if for all of the involved ML Model(s) have the same execution result.

-	One or more tuples (each includes the unique ML Model identifier and corresponding operation execution result indication).

Outputs, Optional: None.

* * * 	End of Changes * * *