SA WG2 Meeting #172	S2-2508259

# 13 - 17, October, 2025, Wuhan, China

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1522 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VFL high level description refinements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-3-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some clarifications in the high level description clause of VFL, specifically: The AF in the VFL process can be trusted or untrusted. Clarifications on the type of NF that can be included in the VFL training and inference. The VFL correlation ID is used to bind the VFL training and inference processes therefore should be unique. There is no ML Model accuracy in the VFL process, we only have a whole VFL accuracy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Terminology alignment, wording improvement of high level description of VFL. Add note saying that the VFL correlation ID needs to be unique among VFL processes. Change the wording of accuracy monitoring of VFL server. Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

5.4	Vertical Federated Learning (VFL)

Vertical Federated learning is a machine learning technique working without exchanging/sharing of local data set, while maintaining some level of coordination amongst VFL server and VFL clients, when training and inference are performed on local ML Models, wherein the local data set in different VFL clients for local model training have different feature spaces for the same samples (e.g. UE IDs). Vertical Federated Learning may involve multiple NWDAFs and AFs.

For Vertical Federated Learning, there may be one NWDAF or one trusted/untrusted AF acting as a VFL server and one or multiple NWDAF(s) and/or one or multiple trusted/untrusted AF(s) acting as VFL Client(s). Vertical Federated Learning is available among NWDAFs or between NWDAF(s) and trusted/untrusted AF(s) within a single PLMN or between an trusted/untrusted AF and NWDAF(s) in a single PLMN. When NWDAF is the VFL server, the VFL clients can be NWDAF or trusted/untrusted AF. When trusted/untrusted AF is the VFL server, only NWDAF can be the VFL clients.

The main functionalities of VFL server and VFL client include:

VFL server:

-	An NWDAF or trusted AF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL procedure.

NOTE 1:	When an untrusted AF is acting as VFL server, NEF discovers and selects candidate VFL client NWDAFs, then the AF determines final set of VFL clients.

-	It requests VFL clients to do local ML model training for an Analytic ID, it assigns VFL correlation ID, and it requests to report intermediate training results.

-	It optionally locally trains ML Model with the available local data set.

-	It combines intermediate training results from VFL client(s) and VFL server, computes intermediate model training information (e.g. gradient information, loss information) using label for updating its own local ML Model and sends the intermediate model training information to VFL clients for updating the ML Models of VFL clients that involved in the VFL training process. VFL server may send and receive separate messages for each VFL client.

NOTE 2:	If VFL server does not have a local ML Model, it only combines intermediate training results from VFL client(s), computes intermediate model training information and send them to VFL client(s).

NOTE 3:	NEF forwards the message from/to NWDAF to/from untrusted AF.

-	It determines to terminate the VFL training process.

-	It stores VFL correlation ID and locally trained ML Model after VFL training process.

-	It initiates the VFL inference process using VFL correlation ID.

-	It combines local inference result from VFL clients and generates the final VFL inference result.

-	It may send the final VFL inference result to the consumer.

-	It supports to monitor the accuracy of the VFL process.

VFL client:

-	It locally trains ML Model with the available local data set, which includes the data that may not be allowed to be shared with other VFL clients or VFL server due to e.g. data privacy, data security, data access rights.

-	It computes the intermediate training results for their local ML Models involved in the VFL training and provide reports with the intermediate training results to the trusted/untrusted AF or NWDAF acting as VFL server.

-	It updates their local ML model based on the intermediate model training information received by the VFL server.

-	It stores VFL correlation ID and locally trained ML model after VFL training process.

-	It performs inference based on the local model and local data and provides intermediate local inference results to VFL server.

Note X: The VFL correlation ID is unique among VFL training and inference processes.

Vertical Federated Learning includes the following procedures:

-	Registration of the NF profile including a list of VFL related information to NRF. Registration of the NWDAF profile to NRF is described in clause 5.2. Registration of the AF profile to NRF is described in clause 5.5. The procedure for registration and discovery of VFL server and VFL client is described in clause 6.2H.2.1.

-	Preparation for VFL including sample alignment to ensure that all the VFL participants have common samples when training ML models as described in clause 6.2H.2.2.

-	Training for VFL as described in clause 6.2H.2.3.

-	Inference for VFL as described in clause 6.2H.2.4.

In this Release, the same NF associated with a VFL Server or VFL Client capability during the VFL training for a VFL correlation ID is also the same NF during the VFL inference.

* * * End of Changes * * *