SA WG2 Meeting #171	S2-2508287

13 – 17 October 2025, Wuhan, China

Source:	LG Electronics

Title:	KI#6: Interim agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#6.

Discussion

For KI#6 "Configuration of Parameters for Sensing Entities", 7 solutions have been proposed. An analysis and summary of these solutions are provided in the following table.

| Sol.# | Parameter Exchange Method (SE-CN) | Parameter Types for Configuration |
| --- | --- | --- |
| #3 | SF or SCF provides configuration parameters to sensing entity(ies); details on signalling path (direct or via AMF) are FFS | Configuration parameters for sensing measurement (No details) |
| #15 | Sensing Entities register/deregister/update capabilities to SF; SF activates/deactivates/reconfigures SEs based on service request | SF profile (supported sensing area); SE profile (supported area, sensing Tx/Rx) |
| #20 | Trusted AF sends configuration request directly to SCF; Untrusted AF send configuration request indirectly to SCF via NEF; SCF provides configuration parameters to sensing entity(ies) | AF ID; sensing-related parameters; target/area information |
| #27 | SF sends configuration to SE(s) with sensing operation parameters | Sensing service type; sensing target area; sensing time parameters; sensing contextual info; requested sensing QoS; sensing result config |
| #34 | SSC sends the sensing service request NEF/GSF; SF sends directly the sensing request to sensing entity with the configuration parameters | Service parameters from AF (e.g., sensing object type, target sensing service area, QoS); Parameters configured by SF (e.g., RCS range list, object velocity range) |
| #35 | Direct connection between SF to SE; Indirect connection where SF sends the parameters via AMF to SE | Service parameters from AF/NEF (e.g., sensing service type, target area, sensing duration, KPI, SSF ID). |
| #39 | SF provides sensing configuration to gNB via AMF | Configuration related to the sensing request (e.g., target sensing area, sensing accuracy, sensing mode, object characteristics vertical/horizontal sensing resolution) |

Summary

- 7 solutions (Sol#3, Sol#15, Sol#20, Sol#27, Sol#34, Sol#35, Sol#39) agree that sensing configuration parameters are provisioned from sensing function(s) to sensing entity(ies). - 3 solutions (e.g., Sol#3, #35, #39) introduce indirect provisioning via AMF, while 6 solutions (e.g., Sol#3, #15, #20, #27, #34, #35) support direct provisioning from sensing function(s) to sensing entity(ies). - Sol#34 proposes that configuration parameters may originate from both AF inputs and SF-generated parameters. - Solutions such as Sol#20 and Sol#34 highlight the distinction between trusted and untrusted AFs, introducing gateway or intermediate functions (e.g., NEF, GSF).

Proposal

It is proposed to agree the following changes into TR 23.700-14.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#6

The interim agreements on principles for KI#6 are as follows:

-	The Sensing Function configures parameters for sensing operations to the Sensing Entity(ies).

7.2	Topics for further consideration

7.2.Y	Topics for further consideration for KI#6

Topics for further consideration for KI#6 are as below:

-	How to define the set of sensing configuration parameters.

-	Whether and how sensing service requirements are mapped into sensing configuration parameters.

-	How for the Sensing Function to configure parameters for sensing operations to the Sensing Entity(ies), e.g. via direct connection, via AMF if a direct connection between the SF and SE does not exist yet.

* * * * End of Changes * * * *