SA WG2 Meeting #171	S2-2508318

# Wuhan, China, 13 – 17 October 2025                           (revision of S2-25xxxxx)

Source: 	CATT

Title: 	KI#2, New sol: AIoT Device autonomously initiated registration and data transfer

Document for: 	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes a new solution for KI#2.

1	Introduction

This paper proposes a new solution for KI#2.

2	Proposal

It is proposed to update the TR 23.700-30 as followings.

# * * * * Start of change (all texts new) * * * *

6.X	Solution #2: AIoT Device autonomously initiated registration and data transfer

6.X.0	High-level Solution Principles

This solution addresses Key Issue #2 "Support of DO-A Capable AIoT Devices".

This solution proposes an AIoT Device autonomously initiated registration-like procedure and data transfer procedure, and is based on the following high-level principles:

-	The DO-A capable AIoT Device initiates registration-like procedure autonomously when it has pending data to be sent to the network or it moves out of the AIoT Area.

-	Once registered, the AIoT Device sends the AIoT data autonomously to the AIOTF and the AIoT data is routed to the AF via the NEF.

6.X.1	Description

This solution is applicable to both the Direct Connectivity and the Indirect Connectivity architectures as defined in TS 23.369 [3].

For a DO-A (Device-originated - autonomous) capable AIoT Device, the device originated traffic is triggered by the AIoT Device itself for, e.g. sending data to the network or moves out of the AIoT Area.

The principles of this solution are as follows:

-	When a DO-A capable AIoT Device has pending data to be sent to the network or it moves out of the AIoT Area, the AIoT Device autonomously sends an AIoT NAS message Registration Request to the AIOTF.

-	The AIOTF authorizes and authenticates the AIoT Device and sends Registration Accept to the AIoT Device.

-	The AIoT Device autonomously sends AIoT data to the AIOTF using an AIoT NAS message.

-	The AIOTF sends the AIoT data received from the AIoT Device to the AF via the NEF.

6.X.2	Procedures

6.X.2.1	AIoT Device initiated registration-like procedure

Figure 6.X.2-1 describes the AIoT Device initiated registration-like procedure.

Figure 6.X.2-1: AIoT Device initiated registration-like procedure

1.	The AIoT Device sends the UL AS D2R message (Registration Request) to the NG-RAN. The AS D2R message includes the AIOTF selection information including PLMN ID and/or NID and/or Third Party ID, or AIOTF ID which is used by the NG-RAN to select the AIOTF. The PLMN ID/NID/Third Party ID is derived from the AIoT Device Permanent Identifier. The AITOF ID is derived from the Temporary ID.

The Registration Request message is an AIoT NAS message and includes the AIoT Device ID, the pending data indication indicating there is pending AIoT data in the AIoT Device, size of AIoT data and last visited AIoT Area. The AIoT Device ID may be a Permanent ID as defined in TS 23.369 [xx] or a Temporary ID including AIOTF ID and a temporary ID assigned by AIOTF.

NOTE 1:	The AS layer message in this procedure are to be defined by RAN WGs.

2.	The NG-RAN selects the AIOTF based on the AIOTF selection information received in step 1 and the local configuration information.

3.	The NG-RAN sends the Uplink AIoT Info message (Registration Request) to the AIOTF directly or via an AMF.

4.	The AITOF performs Authorization and authentication procedure for the AIoT Device.

NOTE 2:	Security aspects are to be defined by SA WG3.

5.	The AIOTF sends the Downlink AIoT Info message (Registration Accept, AIOTF AIoT Device NGAP ID, size of AIoT NAS message with AIoT data) to the NG-RAN directly or via the AMF. If pending data indication is received, the AIOTF allocates AIOTF AIoT Device NGAP ID for the AIoT Device. The AIOTF determines the size of AIoT NAS message with AIoT data based on the size of AIoT data.

6.	The NG-RAN sends the DL AS R2D message (Registration Accept) to the AIoT Device. The Registration Accept message is an AIoT NAS message. The Registration Accept message includes a list of AIoT Areas a Temporary ID.

6.X.2.2	AIoT Device initiated data transfer procedure

Figure 6.X.2-2: AIoT Device initiated data transfer procedure

1.	The AIoT Device sends the UL AS D2R message (UL AIoT NAS message) to the NG-RAN. The UL AIoT NAS message includes AIoT data.

2.	The NG-RAN sends the UL AIoT Info message (UL AIoT NAS message, AIOTF AIoT Device NGAP ID) to the AIOTF directly or via the AMF. The AIOTF determines the AIoT device context by the AIOTF AIoT Device NGAP ID received.

3.	The AIOTF sends the AIoT data to the AF via the NEF. The AF ID and NEF ID for routing AIoT data may be configured in the AIOTF.

4.	The AIOTF sends the DL AIoT Info message (DL AIoT NAS message) to the NG-RAN directly or via the AMF. The DL AIoT NAS message includes the result of AIoT data transfer by the AIOTF.

5.	The NG-RAN sends the DL AS D2R message (DL AIoT NAS message) to the AIoT Device.

6.X.3	Impacts on Services, Entities and Interfaces

AIoT Device:

-	AIoT Device initiates registration-like procedure autonomously.

-	AIoT Device sends the AIoT data to the AIOTF autonomously.

AIOTF:

-	AIOTF handles AIoT Device initiated registration-like procedure and manages the AIoT Areas.

-	AITOF receives AIoT data from AIoT Device and sends the AIoT data to the AF via the NEF.

NG-RAN:

-	NG-RAN performs AIOTF selection.

# * * * * End of Changes * * * *