SA WG2 Meeting #171	S2-2508319

# Wuhan, China, 13 – 17 October 2025                           (revision of S2-25xxxxx)

Source: 	CATT

Title: 	Resolving ENs for Solution#2

Document for: 	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes to resolve the ENs for Solution#2.

1	Introduction

A new solution “AIoT services support for DO-A capable AIoT Devices” was agreed to be captured in the TR 23.700-30 as Solution#2 in the last SA2#170 meeting, and some Editor’s notes were added into this solution.

This paper proposes to resolve these ENs.

2	Proposal

It is proposed to update the TR 23.700-30 as followings.

# * * * * Start of change * * * *

6.2	Solution #2: AIoT services support for DO-A capable AIoT Devices

6.2.0	High-level Solution Principles

This solution addresses Key Issue #2 "Support of DO-A Capable AIoT Devices".

This solution combines registration-like procedure and data transfer procedure, and is based on the following general principles to support AIoT services for DO-A capable AIoT Devices:

-	The DO-A capable AIoT Device initiates registration-like procedure autonomously when it has pending data to be sent to the network.

-	After registration-like procedure, the AIOTF uses the Command Procedure as defined in TS 23.369 [3] to read the AIoT data from the AIoT Device and sends the AIoT data to the AF via the NEF.

6.2.1	Description

This solution is applicable to both the Direct Connectivity and the Indirect Connectivity architectures as defined in TS 23.369 [3].

For a DO-A (Device-originated - autonomous) capable AIoT Device, the device originated traffic is triggered by the AIoT Device itself for, e.g. sending data to the network.

The principles of this solution are as follows:

-	When a DO-A capable AIoT Device has pending data to be sent to the network, the AIoT Device sends Registration Request to the AIOTF.

-	The AIOTF authorizes and authenticates the AIoT Device and sends Registration Accept to the AIoT Device.

-	The AIOTF uses the Command Procedure as defined in TS 23.369 [3] to read the AIoT data from the AIoT Device.

-	The AIOTF sends the AIoT data received from the AIoT Device to the AF via the NEF.

6.2.2	Procedures

Figure 6.2.2-1 describes the DO-A Procedure.

Figure 6.2.2-1: DO-A Procedure

0.	AIoT Device obtains radio resource for D2R message.

NOTE 1:	How DO-A capable AIoT Device obtains radio resources for the first D2R message is to be defined by RAN WGs.

1.	The AIoT Device sends the AS D2R message (Registration Request) to the NG-RAN. The AS D2R message includes the AIOTF selection information including PLMN ID and/or NID and/or Third Party ID, or AIOTF ID which is used by the NG-RAN to select the AIOTF. The PLMN ID/NID/Third Party ID is derived from the AIoT Device Permanent Identifier. The AITOF ID is derived from the Temporary ID.

The Registration Request message is an AIoT NAS message and includes the AIoT Device ID, the Pending data indication indicating there is pending AIoT data in the AIoT Device and size of AIoT data. The AIoT Device ID may be a Permanent ID as defined in TS 23.369 [xx] or a Temporary ID including AIOTF ID and a temporary ID assigned by AIOTF.

NOTE 1:	The AS layer message in this step and the following steps are to be defined by RAN WGs.

2.	The NG-RAN selects the AIOTF based on the AIOTF selection information received in step 1 and the local configuration information.

3.	The NG-RAN sends the Uplink AIoT information message (Registration Request) to the AIOTF directly or via an AMF.

4.	The AITOF performs Authorization and authentication procedure for the AIoT Device.

NOTE 2:	Security aspects are to be defined by SA WG3.

5.	The AIOTF sends the Downlink AIoT information message (Registration Accept, AIOTF AIoT Device NGAP ID) to the NG-RAN directly or via the AMF.

6.	The NG-RAN sends the AS R2D message (Registration Accept) to the AIoT Device. The Registration Accept message is an AIoT NAS message. The Registration Accept may include a Temporary ID.

7.	Based on the Pending data indication received or there is an on-going Command procedure initiated by the AF, the AIOTF sends the Command Request message (NAS Command Request, AIOTF AIoT Device NGAP ID, Size of the Command Response message) to the NG-RAN directly or via the AMF. The NAS Command Request is Read Request. The AIOTF AIoT Device NGAP ID is used by the NG-RAN to determine the AIoT device context in NG-RAN.

If the NAS Command Request is triggered by the Pending data indication, the Read Request is constructed by the AIOTF based on local configuration.

If the NAS Command Request is triggered by the on-going Command procedure initiated by the AF, the Read Request is constructed by the AIOTF based on the AF provided Command type specific parameters as defined in TS 23.339 [xx].

8.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device.

The steps 5-6 can be combined into the steps 7-8, and the NAS Command Request implicitly indicates registration is successful.

9.	The AIoT Device sends the D2R message (NAS Command Response) to the NG-RAN. The NAS Command Response is Read Response and includes the AIoT data.

10.	The NG-RAN sends the Command Response message (NAS Command Response, AIOTF AIoT Device NGAP ID) to the AIOTF directly or via the AMF. The AIOTF determines the AIoT device context by the AIOTF AIoT Device NGAP ID received.

11.	The AIOTF reports the AIoT data to the AF via the NEF. The AF ID and NEF ID for routing AIoT data may be configured in the AIOTF.

6.2.3	Impacts on Services, Entities and Interfaces

AIoT Device:

-	AIoT Device initiates registration-like procedure autonomously.

-	AIoT Device sends the AIoT data to the AIOTF by responding to the Command Request.

AIOTF:

-	AIOTF handles AIoT Device initiated registration-like procedure.

-	AITOF retrieves AIoT data from AIoT Device by using Command Request.

-	AIOTF sends AIoT data to the AF via the NEF.

NG-RAN:

-	NG-RAN performs AIOTF selection.

# * * * * End of Changes * * * *