3GPP SA WG2 Meeting #171	S2-2508320

13 – 17 Oct, 2025, Wuhan, China	(revision of S2-2508110)

Source:	CATT

Title:	[General aspects] Architectural Assumptions

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes a revision of S2-2508110 on Architectural Assumptions.

**** First Change (all text new) ****

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

6G CN: In this study, it refers to the Core Network for the 6G System.

NOTE:	Whether 6G CN can be considered as evolution of 5GC or as a new Core Network will be determined during the study.

6G RAN: In this study, it refers to the radio access network for 6G radio access technology (6GR) being studied by 3GPP RAN WGs [ref: RP-251881].

6G System (6GS): 3GPP System consisting of 6G RAN, non-3GPP access, 6G CN and UE.

**** Next Change (all text new) ****

4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

The following is assumed for 6GS architecture:

1.	The framework of SBA specified for 5GC is assumed as a starting point for discussion.

2.  The study assumes a standalone architecture [ref: RP-251881].

NOTE: approaches beyond stand-alone architecture will be considered in coordination with TSG RAN [ref: RP-251881].

3.	The 6G system is assumed to maintain the existing RAN and CN functionality split as supported in 5GS, and avoid duplication of functionality in 6G RAN and CN.

4.	Support of multi-media services such as voice, video (essential and regulatory services) will be using IMS.

5.	Support of orthogonal functionality split between 6G CN NFs.

6.   6G CN shall support NFs that use 5GC NFs as basis and any enhancements, that need further study and may be redesigned, or that to be added for supporting new features when identified during the study.

**** End of Changes ****