3GPP TSG-SA WG2 Meeting #171	S2-2508334

Wuhan, China, October 13 – 17, 2025

Source:	vivo

Title:	New Solution: Support Report operator for DOA device

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#2 for fupporting DOA capable device.

1. Discussion

The KI#2 describes following tasks that need to be resolved:

-	How the AIoT Device informs the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure) and what are the triggers for the DO-A capable device to inform the network of its presence.

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

This solution propses a simple way with following principle for DO-A device report its presence or sending data to an AF:

-	No registration-like procedure needed for the DO-A device, so no context in AIoTF for the device.

-	The AIOTF for routing the DO-A report and the AIOTF for inventory and command operations can be different, so AIoTF is stateless.

-	The DO-A report includes target AF and may include data from the AIoT Device. The network authorizes the report.

2. Proposal

It is proposed to agree the following changes in TR 23.700-30 v0.1.0.

* * * Start of Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | Key Issue #1 | <Key Issue #2> |
| #1 |  | X |
| #2 |  | X |
| #3 |  | X |
| #4 |  | X |
| #X |  | X |

* * * Second of Change (all new text) * * * *

## 6.X	Solution #X: Support Reporting operator triggered by events without registration for DO-A capable AIoT Devices

### 6.X.0	High-level Solution Principles

This solution addresses Key Issue #2 "Support of DO-A Capable AIoT Devices".

AIoT device sends report message when triggered by events, includes pending data, power on, device specific events (e.g., temperature higher than a threshold). The information for the device specific events can be provisioned by AF via Command operation, e.g., writing the threshold to the AIoT device.

### 6.X.1	Description

This solution is applicable to both the Direct Connectivity and the Indirect Connectivity architectures as defined in TS 23.369 [3].

For a DO-A capable AIoT Device, the device is triggered by events to report its presence and/or send data to the network.

The principles of this solution are as follows:

-	No registration-like procedure needed for the DO-A capable AIoT Device.

-	The AIOTF for routing the DO-A report and the AIOTF for inventory and command operations can be different.

-	The events triggered DO-A report includes target AF and may include data from the AIoT Device.

### 6.X.2	Procedures

Figure 6.X.2-1 describes the Reporting procedure from DO-A capable AIoT Device.

Figure 6.X.2-1: DO-A device reporting procedure

0.	The AIoT device is pre-configured with target AF information, e.g., FQDN, by the owner of the AIoT device for routing the information in the Report Request. Allowed AF(s) may be pre-configured in the AIoT device profile data in ADM. The information for device specific events may be provisioned by the AF using the Command operation, e.g., write a shreshold related to a device specific event to the AIoT device.

NOTE 1:	The allowed AF(s) in the ADM can be changed via OAM. The target AF information may be configured by the owner of the device via Inventory and Command operator.

1.	The AF requests to setup AIOT session with the AF information and optional the service area towards the NEF. The NEF selects AIOTF(s), e.g., based on pre-configuration or the service area, and requests to setup AIOT session towards the AIOTF(s).

2.	The AIoT device is triggered by events, e.g., pending data, power on, or device specific events.

3.	The AIoT device sends D2R message including AIoT NAS Report Request (Device ID, target AF, [data]) to the RAN Reader. In case there are pending data in the AIoT device, the data is included in the AIoT NAS Report Request message.

NOTE 2:	Security aspects related to the Device ID and the data protection are to be defined by SA WG3.

4.	The NG-RAN selects an AIoTF, e.g., based on local policy or pre-configuration.

5.	The NG-RAN sends a N2 request message (indirect) or AIOT2 request message (direct) including the Reader ID and the AIOT NAS Report Request message towards the selected AIOTF.

6.	The AIOTF requests authorization data from ADM.

7.	The AIOTF authorizes the Report Request from the device, e.g., based on whether the target AF is in the allowed AF(s) per the device, per the group of the device, or per the network, whether the device is allowed to initiate Report Request, whether the location is allowed to send the report per the device, etc. If the authorization of report from the AIoT device is successful, goes to step 8, otherwise goes to step 9 with proper cuase value.

8.	The AIOTF maps the Reader ID into location information, and sends a Notify (Device ID, location info, [data]) message towards the AF via the NEF based on the target AF. The NEF forwards the Notify message towards the AF.

9.	The AIOTF responds to the RAN Reader.

The AIOTF may update the last known reader information in the AIoT device profile data in the ADM.

10.	The RAN Reader sends R2D message to the AIoT Device.

### 6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

AIoT Device:

-	Support initiate Report Request procedure triggered by events.

NG-RAN:

-	Support AIOTF selection.

AIOTF:

-	Support Report Request authorization.

-	Support sending Device ID, location information, and optional data to target AF indicated in the Report Request.

NEF:

-	Support managing AF sessions for AIOT service between AFs and AIOTFs.

ADM:

-	Support providing authorization data to AIOTF when requested.

* * * End of Change * * * *