SA WG2 Meeting #171	S2-2508346

Wuhan, China, 13 – 17 October 2025

Source:	vivo, Tencent, Meta, China Unicom, CAICT, China Mobile, China Telecom, Ofinno, NTT DOCOMO, InterDigital Inc.

Title:	[WT#TBD, UE energy saving] UE energy saving

Document for:	Agreement

Agenda Item:	20.6.X.X

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to include the UE energy saving related KI in the FS_6G_ARC study.

1 Introduction

During SA2#170 discussion, some companies have concerns on mixing UE power saving and other energy feature proposal together within one WT.

This contribution mainly focuses on: UE energy saving, including:

- 1) UE power saving while QoS fulfilment.

NOTE: UE power saving means the communication energy saving.

- 2) Optimized saving for the UE combined energy consumption (composed of computing energy consumption and communication energy consumption).

2 Discussion

2.1 UE power saving while QoS fulfilment

2.1.1 Architectural Motivation

<Scenario1: Lightweight glasses: >

Figure 2.1-1. lightweight glasses

In addition to smart phone, new types of devices are emerging, e.g. light weight glasses (e.g. AR/AI glasses), robots.

The lightweight glasses can be foreseen to be one kind of promising new mobile terminal, because: 1) they can bring AR/AI traffic to outdoor scenarios and, 2) compared with smartphones, they can enable hands-free operation. 3) More importantly, they have the potential to be accessible to every individual and they will integrate into our daily lives as an essential device as smartphones. It is anticipated that they could achieve the similar success as smartphones in business.

Currently, numbers of AR/AI glasses products have been launched.  Due to constraints such as power supply, heat dissipation, size, and technical maturity, the battery and computing power of AR/AI glasses are highly restricted.

Table-1

|  | AR/AI Glasses | Smartphone |
| --- | --- | --- |
| Weight | About 48g-79g | About 200g |
| Battery capacity | 200~260mAh | 3500-5000mAh |

The technology for light weight glasses is constantly evolving. The early-stage light weight glasses relied on smartphones or some external box for use, while the current light weight glasses can be used alone by integrating Wi-Fi and Bluetooth. In the future, they will be able to carry wireless modules. With regard to the 6G standard oriented toward commercial scenarios a few years later, it could be reasonably assumed that lightweight glasses can be used in the cellular network at that time.

Besides, AR/AI glasses belong to XR devices and the XR traffic drain battery faster. The AI capabilities in lightweight AR/AI glasses also consumes a lot of power. To support the adoption of AR/AI glasses in outdoor cellular environments, mobile networks need to meet their requirements:

- In terms of traffic, they still require a certain level of high bandwidth and low latency.

- In terms of power consumption, they are more extreme and sensitive compared to ordinary smartphones and robots and they require stronger power saving.

Observation 1: The lightweight glasses require concurrent power saving and meeting the QoS requirements, e.g. comparably low latency and high bandwidth.

<Scenario2: Other UE types>

Except for light weight AR/AI glasses, a growing number of terminals will be equipped with AI capabilities, these devices with AI capability also require to extend UE battery life while guaranteeing certain service quality. But the power saving requirement may not be as extreme as AR/AI glasses.

When a smartphone is low on power, for example, at 10%, the situation is similar to that of AR/AI glasses. When a smartphone has 30% power, its power-saving requirements may differ from those at 10%.

In a summary, UE power saving may include different level of power saving requirement, e.g.:

- Different UE types may require different level of UE power saving, e.g. smart phone, glasses, other devices with AI capabilities.

- Even for the same kind of UE, it may also require different levels of UE power saving in different power situation.

Observation 2: Different UE may have different level of power saving requirements, and for a given UE, it may have different levels of UE power saving in different power situation.

Observation 3: The concurrent UE power saving and QoS fulfillment are also beneficial for other types of UE (e.g. Smartphones in low power situation).

<SA1 and RAN requirement for UE power saving>

Furthermore, ITU-R, SA1 requirement and RAN 6G SID also highlighted the requirement of UE energy saving:

- In the ITU-R Recommendation "Framework and overall objectives of the future development of IMT for 2030 and beyond", sustainability is considered as the design principles commonly applicable to all usage scenarios.

- In 3GPP SA1 TR 22.870 for 6G requirements, several use cases related with UE energy saving and energy efficiency are proposed. At lease the following service and architecture requirements are included:

- [PR 5.8.1.6-1] The 6G system shall improve energy efficiency of 6G system compared to 5G system.

- [PR 5.8.1.6-2] The 6G network shall be able to support mechanisms to improve UE energy efficiency when providing services to the subscribers.

- [PR 5.8.1.6-3] The 6G system shall provide means for a user to provide a preference for UE energy efficiency.

- At 3GPP RAN SID proposal (FS_6G_RAN_Scen_Req), it includes the following objective:

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

Energy efficiency and energy saving: both for network and device.

In the RAN 6G SID, there are clear requirements to ensure e user experience, which include the UE power saving needs. The concurrent UE power saving and QoS fulfilment is in line with the UE power saving with user experience requirements specified in the 6G SID.

Generally, the main work for UE power saving lies in RAN, SA2 is involved to ensure the study is completed from a system-level perspective. For example, in the R18/R19 XRM study, both SA2 and RAN have included UE power saving in their study scopes. The SA2’s responsibility is the collaboration between UE and CN, CN and RAN Service QoS requirements and user experience generally fall within SA2’s scope. Additionally, since UE power saving features consume more resources, per-UE authorization by the CN could be considered and this also falls within SA2’s scope. SA2 can formulate the procedures for negotiating UE power saving requirements and auxiliary information to the RAN.

Observation 4: RAN 6G SID has included UE power saving with user experience, SA2 should also include the UE power saving into the 6G SID as well to ensure the study is completed from a system-level perspective and includes a note: this study requires coordination with the RAN WG.

< Possible new business opportunity for UE power saving with QoS guarantee>

The 6G system is oriented towards to future a few years later business world and should be ready in advance for the emerging new terminal.

It is anticipated that they will achieve a market scale comparable to that of smartphones in the Mobile Internet era. It is believed that the lightweight glasses can bring the following benefits to operators:

- Significant growth in new subscribers. Since each pair of AR/AI glasses means a potential mobile contacts.

- They can bring AI/AR applications and related traffic into mobile environments, which is conducive to expanding the traffic in 6G network pipelines.

- More valuable data bits. AR/AI glasses should give rise to new billing requirements, and each bit of data consumed by them should be priced higher than that by other terminals, given their greater potential value.

2.1.2 Gap Analysis

In 5G system EE discussion, some network energy saving mechanisms is realized by means of QoS degradation. The network energy saving actions are mainly based on UE subscription and are lack of the user consent.

In 5G system, some UE power saving technologies (e.g. MICO mode, C-DRX, etc.), have been specified. But they are not sufficiently to support the new requirement of AR/AI glasses:

  - The longer sleep time, the greater energy saving. The sleep time may impact the latency of packet delivery. The extending latency may impact the QoS fulfillment. So far, these technologies applied on a best-effort basis without collaboration of the QoS guarantee for the user traffic and cannot reach the optimized effort between energy saving and QoS fulfillment.

  - 5GS has specified that the CN provides the RAN with the following assistant information for C-DRX: UL and/or DL Periodicity, N6 Jitter Information associated with the DL Periodicity. However, this type of information reflects traffic characteristics rather than QoS requirements or per UE’s power-saving requirements. For example, periodicity can indicate the data arrival interval but cannot reflect the delay budget. A gap remains in how to further balance power saving and QoS fulfillment under specific traffic characteristics.

  - 5G Connection drain battery faster and the immersive services also drain battery faster.

  - 5GS doesn’t support different levels of UE power saving requirements and Energy-Saving Service to different UE.

  - Legacy power saving has to sacrifice performance of packet delivery. The sacrificed performance may impact the service experience, concurrent QoS f and power Saving is new requirement

  - There is no UE like light weight AR/AI glasses before, whose battery capacity is one-twentieth to one-tenth to that of a smart phone, its power saving requirement is stronger than that of smart phone.

Observation 5: The current UE power saving technologies lack of collaboration of the QoS guarantee for the user traffic and cannot reach the optimized effort between energy saving effort and QoS fulfillment.

So the 6G system needs to consider both the QoS requirements and UE power saving in order to improve the service experience for the light-weighted XR devices, e.g., AR/AI glasses.

In 5G network, the main business model between the operators and subscribers is based on the data traffic transmission. Considering more and more communication technologies are applied into the 6G system, the exploration for the business model is also an important part for the operators to consider whether to deploy these technologies.

UE power saving while meeting the QoS requirement as service is a new business model in 6G system, which can provide the high-value added scenario for the operators. This new business model has the following two advantages:

- The operators could provide the new value-added service and attract more subscribers by deploying the UE power saving technologies.

- The devices could get better service experience with guaranteed QoS and reduced communication power consumption.

2.2 Optimized saving for the UE combined energy consumption

2.2.1 Architectural Motivation

The “UE combined energy” is composed of the UE computing energy and the UE communication energy. When the computing task (e.g. rendering) is offloaded, the UE computing energy consumption reduces, however, the latency and throughput of the traffic increase because the amount of data after rendering has significantly increased compared to the amount of data before rendering.

The significantly increased rendered data increases the communication energy consumption. According to the reduced computing energy consumption and increased UE communication energy consumption, whether and how to achieve optimized saving for the UE combined energy is unclear yet.

When a UE offloads computing (e.g. rendering) tasks to a server or network, whether the computing energy saved by offloading rendering is greater than the increased communication energy consumption caused by the larger amount of data transmitted back to the device after rendering depends on the specific device performance, network conditions, and task characteristics (e.g. the complexity).

Generally, if the computing task is very complex while the amount of data after rendering to be transmitted is relatively small or the network bandwidth is high and no congestion, the saved computing energy is likely to be greater than the increased communication energy consumption. For example, some high-performance gaming.

On the contrary, if the computing task is relatively simple, but the amount of data after rendering to be transmitted is large or the network bandwidth is low and congested, the increased communication energy consumption may be greater than the saved computing energy.

Observation 6: It is worth to consider how to achieve the optimal saving for combined UE energy consumption (composed of computing energy consumption and communication energy consumption), when the computing task (e.g. rendering) is offloaded.

2.2.2 Gap Analysis

So far, there is no integrated consideration on UE combined energy (including computing energy and communication energy) in pre-6G system.

Observation 7: There is no integrated consideration on optimized saving for the UE combined energy consumption (composed of computing energy consumption and communication energy consumption) in pre-6G system.

2.3 Technical Impact

In order to save the energy consumption and QoS fulfillment, e.g. via traffic shaping mechanism to make the UE have more sleep time but the data volume is transmitted in a shorter period, by the above mechanism, the network still guarantee the data rate in the required delay budget and achieve more power saving. It is suitable for the traffic without extreme delay requirement, e.g. traffic in AR/AI glasses, which can be also beneficial for smart phones in the low power situation.

The technical impact includes the following details:

-   X.1 Whether and how to collaborate among UE, RAN and CN to support optimized effort between UE power saving and QoS fulfillment on UE (e.g. lightweight AR/AI glasses, smart phone with low power), including the following aspects:

-     Whether and how the CN negotiates different levels of UE power saving requirement and QoS requirement for a given UE or different UEs.

-     Whether and how the CN enables RAN to perform different levels of UE power saving without sacrificing the QoS and provides RAN with assistance information for a given UE.

NOTE1: It needs coordination with RAN WGs.

-    X.2 Whether and how achieve the optimized saving for the UE combined energy consumption (composed of computing energy consumption and communication energy consumption) when the computing task (e.g. rendering) is offloaded.

However, this contribution is open to which WT could include this UE power saving related WT or KI.

Proposal 1: It proposes to agree the UE energy saving related study and KI.

Proposal 2: Which WT to include UE energy saving related KI can be further determined.

Conclusions

Observation 1: The lightweight glasses require concurrent power saving and meeting the QoS requirements, e.g. comparably low latency and high bandwidth.

Observation 2: Different UE may have different level of power saving requirements, and for a given UE, it may have different levels of UE power saving in different power situation.

Observation 3: The concurrent UE power saving and QoS fulfillment are also beneficial for other types of UE (e.g. Smartphones in low power situation).

Observation 4: RAN 6G SID has included UE power saving with user experience, SA2 should also include the UE power saving into the 6G SID as well to ensure the study is completed from a system-level perspective and includes a note: this study requires coordination with the RAN WG.

Observation 5: The current UE power saving technologies lack of collaboration of the QoS guarantee for the user traffic and cannot reach the optimized effort between energy saving effort and QoS fulfillment.

Observation 6: It is worth to consider how to achieve the optimal saving for combined UE energy consumption (composed of computing energy consumption and communication energy consumption), when the computing task (e.g. rendering) is offloaded.

Observation 7: There is no integrated consideration on optimized saving for the UE combined energy consumption (composed of computing energy consumption and communication energy consumption) in pre-6G system.

Proposal1: It proposes to agree the UE energy saving related study and KI.

Proposal2: Which WT to include UE energy saving related KI can be further determined.

**** First Change (All new text)****

Annex A.X. WT#TBD

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#X: TBD

-   X.1 Whether and how to collaborate among UE, RAN and CN to support optimized effort between UE power saving and QoS fulfillment on UE, including the following aspects:

-     Whether and how the CN negotiates different levels of UE power saving requirement and QoS requirement for a given UE or different UEs.

-     Whether and how the CN enables RAN to perform different levels of UE power saving without sacrificing the QoS and provides RAN with assistance information for a given UE.

NOTE1: It needs coordination with RAN WGs.

-    X.2 Whether and how to achieve the optimized saving for the UE combined energy consumption (composed of computing energy consumption and communication energy consumption) when the computing task (e.g. rendering) is offloaded.

Editor's Note: Which WT includes the above UE power saving aspect is FFS.

**** Second Change (All new text)****

5.X. Key Issue #X: UE Energy Saving

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This Key Issue on UE Energy Saving (e.g. UE energy saving for lightweight AR/AI glasses, smart phone with low power) will study the following aspects:

-   X.1 Whether and how to collaborate among UE, RAN and CN to support optimized effort between UE power saving and QoS fulfillment on UE, including the following aspects:

-     Whether and how the CN negotiates different levels of UE power saving requirement and QoS requirement for a given UE or different UEs.

-     Whether and how the CN enables RAN to perform different levels of UE power saving without sacrificing the QoS and provides RAN with assistance information for a given UE.

NOTE1: It needs coordination with RAN WGs.

-    X.2 Whether and how to achieve the optimized saving for the UE combined energy consumption (composed of computing energy consumption and communication energy consumption) when the computing task (e.g. rendering) is offloaded.

**** End of Changes ****