3GPP TSG-SA WG2 Meeting #171	S2-2508359

Wuhan, China, October 13 – 17, 2025

Source: 	NTT DOCOMO

Title: 	KI#1 interim agreements

Document for: 	Approval

Agenda Item: 	20.3.1 (FS_AIML_CN_Ph2)

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution updates the interim agreements for KI#1 to transfer data from UE via UP.

# 1	Discussion

Existing 5GC NF or a new 5GC NF

In SA2, there is ongoing discussion on whether the 5GC NF responsible for managing UE data collection should be realized by enhancing existing 5GC NFs (e.g., NWDAF, DCCF) or by introducing a new NF (e.g., DCF). As stated in the LS from RAN to SA (RP-242389), the UE data collection solution is expected to be futureproof and extendable. If existing NFs are enhanced for this purpose, compatibility with their current service responsibilities must be maintained, which may limit flexibility and hinder extensibility. Moreover, current 5GC NFs such as NWDAF and DCCF do not support user plane path establishment with the UE, nor do they provide coordination with RAN nodes for data collection—both of which are essential features for UE data collection. For these reasons, it appears reasonable to consider the introduction of a new 5GC NF (e.g., DCF) dedicated to managing UE data collection.

User plane path establishment for the data transfer

In the SA2#170 meeting, the establishment of PDU session dedicated for the data collection traffic was made the agreed principles. This approach provides clear separation between regular traffic and data collection traffic. Then, it is not necessary to introduce other differentiation mechanism by QoS flow.

In order for the UE to establish a user plane path with the 5GC NF managing UE data collection, user plane (UP) information (i.e., the IP address or FQDN of the NF) must be provided to the UE. In our view, this can be achieved using a similar mechanism to Rel-18 UP positioning, where the 5GC NF provides the required UP information to the UE via NAS signalling. This reuse of a known principle ensures alignment with existing 5GC architecture while minimizing the need for new signalling mechanisms.

Full controllability

Full controllability of the data transfer by the MNO is a fundamental requirement for UE data collection. The 5GC NF managing UE data collection should be capable of sending control messages to the UE, specifying what data shall be transferred and when data shall be trasnferred. Upon receiving the control message from 5GC NF, UE transfers data if UE can transfer.

Two main approaches for delivering these control messages between the 5GC NF and the UE can be considered: control plane solution and user plane. In control plane solution, NAS message can be used for carrying the data transfer control message. Instead of control plane solution, the user plane can be used too. In UE data collection via UP, the user plane connection between UE and 5GC NF needs to be established for the data transfer. This user plane can be used even for control messages of the data transfer. The user plane solution offers potential benefits. One benefit is to reduce the involvement and signalling overhead of intermediate nodes (e.g., gNB and AMF). Also, it can improve the implementation simplicity, because there is no impact on the structure and content of lower layer signalling (e.g., RRC or NAS message). SA2 should discuss and determine which option is more suitable for user plane UE data collection.

Full visibility

Full visibility for the MNO is another key requirement for UE data collection. The network must be aware of what data is being transferred and must have access to the collected data. This can be achieved by ensuring that UE data is transferred in a standardized format to the 5GC NF managing UE data collection. When standardized UE data is terminated at the NF, the MNO can verify compliance with standardized data formats and access the content of the collected data. This ensures transparency, supports interoperability, and enables effective data management. Hence, the transferred standardized UE data should be terminated in 5GC NF for MNO full visibility.

# 2 	Proposal

It is proposed to update TR 23.700-04 according to the following text.

********** First Change **********

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

- 	New 5GC NF is introduced for controlling UE data transfer.

-  5GC NF sends its UP information (i.e., IP address or FQDN of 5GC NF for controlling UE data transfer) to the UE via NAS for establishing the UP path.

-	5GC NF controls UE data transfer via sending data transfer command including what data to be transferred and start/stop of data transfer.

-	The UE transfers the collected data according to the received data transfer command, when UE can transfer.

-  The transferred standardized UE data is terminated in 5GC NF and forwarded to the consumer to satisfy the full visibility.

********** Second Change **********

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	Whether the configuration information for UE data transfer is sent to the UE over CP or UP established for the data transfer.

-	Whether and how to select UEs for UE data collection.

-

-	Whether the UE needs to be authorized for UE data collection.

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

********** End of Changes **********