SA WG2 Meeting #171	S2-2508442

13 - 17 October, 2025, Wuhan, China	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TR 23.700-56: KI#1, New Sol: Early Media in Alerting Phase Based on DC AS Initiated A2P DC

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: The paper adds a new solution for supporting early media in alerting phase based on DC AS initiated A2P DC in TR 23.700-56.

1. Discussion

The proposal addresses the Key Issue:

Key Issue #1: ADC establishment in alerting phase for early media

This key issue will investigate how to support IMS application data channel establishment in alerting phase to provide early media for a normal IMS DC session.

This Key Issue will study:

-	Whether and how the capability of supporting ADC establishment in alerting phase is exchanged between UE and IMS network.

-	How the use of ADC during alerting phase is controlled.

-	Whether and how to download a specific list of DC applications that can be used during alerting phase.

-	How to establish and terminate an ADC during alerting phase.

The paper adds a new solution for supporting early media in alerting phase based on DC AS initiated A2P DC.

2. Proposal

It is proposed to capture the following changes into TR 23.700-56.

* * * * Start of Changes * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 | X |  |  |
| #X | X |  |  |

* * * * Next Change (All new text) * * * *

6.X.0	High-level solution Principles

This solution addresses Key Issue #1: ADC establishment in alerting phase for early media.

The solution focuses on the DC based Customized Alerting Tones (CAT) service which is subscribed by the called party and it will provide customized content to the calling party as an early media during the calling process.

The solution has the following principles:

-	The called UE B has subscribed to the DC based CAT service.

-	The calling UE A and called UE B have the DC based early media capability, and both of them have negotiated with the serving network regarding this capability.

-	The solution needs an enhancement of the subscribe/notify framework for event monitoring procedure to notify the DC AS the calling event which needs an A2P DC to deliver the early media

-	The solution reuses the IMS DC capability exposure procedure to establish an A2P DC which is used to transmit the early media to the UE.

-	The IMS-AS serving the called UE B notifies the calling event to the DC AS; the DC AS triggers the A2P DC establishment process to the calling UE A.

6.X.1	Description

This solution provides a procedure on how to establish a P2A ADC in alerting phase to provide customized DC content to the calling party as an early media.

6.X.2	Procedure

The pre-condition of the following procedure is that the called UE B has subscribed to the DC based CAT service, and the calling UE A and called UE B have the DC based early media capability, and both of them have negotiated with the serving network regarding this capability.

Figure 6.X.2-1: Procedure of A2P ADC establishment in alerting phase

The steps in the procedure are as follows:

1. UE A initiates SIP INVITE including Audio/Video/BDC SDP; step 1-13 of Figure AC.7.1-1 in TS 23.228 for BDC establishment apply.

2. BDC has been established for UEA and UE A may download the application list.

3. UE B is alerting and 180 Ringing is sent to IMS AS-2.

4. Based on the subscription of UE B, IMS AS-2 decides to notify the DC AS this calling event.

5. The IMS AS-2 sends the Nimsas_ImsEE_Notify Request to the NEF the calling event including the calling UE A and the called UE B information, as well as IMS Session ID.

6. The NEF sends the Nnef_ImsEE_Notify Request to the DC AS the calling event including the corresponding information.

7. The DC AS decides to establish A2P ADC to UE A to deliver the DC early media.

8. The DC AS establishes A2P ADC with UE A; and step 1-21 of Figure AG.2.1.1-1 in TS 23.228 for adding ADC to an existing IMS session apply. UE A may download the application used in the alert phase.

9. The UE B/terminating network and UE A/originating network exchange 180 Ringing, PRACK and 200OK (PRACK) SIP signals.

10. The A2P ADC between UE A and the DC AS which goes through the MF is activated, and the early media (customized alerting tones) of this IMS session is delivered from the DC AS to UE A. The user of UE A experiences with the early media.

11. The user of UE B picks up the phone, and the UE B/terminating network and UE A/originating network exchange 200 OK (INVITE) SIP signal.

12. The IMS AS-2 notifies the DC AS the IMS session establishment event.

13. The DC AS releases the A2P ADC with UE A.

14. Subsequent procedures continue.

NOTE:	The detailed steps for interaction between the IMS-AS and the DCSF, the IMS-AS and the MF are not illustrated in the procedure, which reuse the corresponding steps in Figure AC.7.1-1 and Figure AG.2.1.1-1.

Editor's note:	The procedure for Customized Ringing Signal (CRS) service is FFS.

6.X.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

-	subscribing to the DC based CAT service as early media;

-	supporting ADC establishment in alerting phase.

IMS AS:

-	supporting report of IMS session establishment events to DC AS;

-	supporting A2P DC establishment for delivering early media.

MF:

-	receiving early media from DC AS and providing it to UE through A2P DC.

DC AS:

-	supporting receiving IMS session establishment events from IMS AS through NEF;

-	supporting initiating and releasing A2P DC for delivering early media.

Editor's note:	Additional Impacts are FFS.

* * * * End of Changes * * * *