3GPP TSG-WG SA2 Meeting #171	S2-2508469

WUHAN, China, Oct 13 - 17, 2025

Source:	China Mobile

Title:	Proposal on Interim Agreements for KI#4

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements for KI#4 for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

Solutions #1, #3, #5, #18, #21, #22, #23, #24, #25, and #26 address KI#4 based on TR 23.700-14 V1.0.0.

Here is a summary of the data collection-related aspects of the above solutions:

- Solution #1: Direct On-Demand Collection establishes direct connections to Sensing Entities only when needed. Uses direct links to request/receive raw sensing data + associated info (e.g., metadata).

- Solution #3: Framework with Functional Split, only high-level procedure defined, details deferred to specific implementations.

- Solution #5: AMF-Centric with Enhanced Interfaces, Requires enhancements to N1/N2 interfaces for sensing data transmission.

- Solution #18: Control messages are delivered via AMF. Sensing data collection via the direct interface between RAN and SF.

- Solution #21: Support sensing message transfer via the direct interface between RAN and SF.

- Solution #22: Support sensing message transfer via the direct interface based on a tunnel between RAN and SF.

- Solution #23: Functional Split –SeMF and SePF, direct interface between RAN and SePF.

- Solution#24: Functional Split – SDCF and SSMF Control message is delivered via AMF, sensing data collection via the direct interface between RAN and SDCF.

- Solution #25: Functional Split –SCF and SPF. Direct Connection between gNB and SPF for Sensing Data Collection and Transport.

- Solution #26: Functional Split –SCF, SPF, and SDMF. Direct Connection between gNB and SPF for Sensing Data Collection and Transport.

Apart from Solution #5, all other solutions assume that sensing data and the associated information are delivered over direct data connections between the SF and the gNB.

Majority of solutions, such as #1, #3, #21, #22, #23, #25, and #26, support delivering sensing service messages directly from the core network to the gNB, without going through the AMF.

Half of Solutions, such as #3, # 23, #24, #25 and #26, support splitting the SF function into the Sensing Control Plane and the Sensing Data Plane.

2	Proposal

It is proposed to include the following changes in TR 23.700-14.

* * * * Start of Changes (all new text) * * * *

7.1.Y	Agreed Principles for KI#4

The sensing data and the related information are sent directly between the gNB and the Sensing Function for generating results over the data connection.

The sensing service request messages are delivered from the Sensing Function to the gNB directly via a data connection.

The sensing function is split into the control plane and data plane.

* * * * End of Changes * * * *