SA WG2 Meeting #171	S2-2508502

13 – 17 Oct 2025, Wuhan, China

Source:	Nokia

Title:	KI#2: Solution for DC application downloading events

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for FS_NG_RTC_Ph3.

# 1. Discussion

This pCR proposes a solution to the KI#2 in Next Generation Real Time Communication Phase 3.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> |
| #X: Subscription and notification of DC application downloading events |  | X |  |

# * * * * next change * * * * ALL text is new

## 6.X	Solution #X: Subscription and notification of DC application downloading events

### 6.X.0	High-level solution principles

This solution targets KI2.

This solution enables an AF (a DC AS) to subscribe to and being notified about events related to the downloading of DC applications. Four types of events are supported : these events can be related to specific or any UE(s), as well as independent or dependent on application(s) (see § 6.X.1).. Subscription requests may optionally include event filters, which can be negotiated with the IMS network, to control the granularity and frequency of notifications. The AF (DC AS) can use the corresponding notifications to assess the popularity of applications (especially P2P applications).

Once the IMS AS has received a subscription request from the AF as defined in Annex AD.3 of TS 23.228 [2], and when a BDC is established, the IMS AS, based on the type of the DC application downloading events (see § 6.X.1), provides the subscription to the DCSF handling the BDC. The selected DCSF is responsible for detecting these events and reports the relevant information directly to the AF/DC AS (or to the NEF).

### 6.X.1	Types of DC application downloading events

To support enhanced exposure capabilities, the IMS DC exposure framework provides mechanism for a DC AS to subscribe to events related to the downloading of DC applications. Four types of DC application downloading events are supported:

-	Type 1: Subscriber-specific and DC application-specific event: The subscription request relates to one or more UEs (identified by their IMPUs) and one or more DC application identifiers. A notification shall be triggered only when one of the UE(s) downloads one of the DC applications.

-	Type 2: Subscriber-specific and DC application-agnostic event: The subscription request relates to one or more UEs, without indicating any specific DC application identifier. The DC AS identifier shall be included in the subscription request. A notification shall be triggered when one of the specified UE(s) download any DC application associated with the subscribing DC AS. Notifications for DC applications associated with other DC ASs shall not be provided.

-	Type 3: Subscriber-agnostic and DC application-specific event: The subscription request relates to one or more DC application identifiers, without specifying any UE. A notification shall be triggered when a UE downloads the specified DC application(s).

-	Type 4: Subscriber-agnostic and DC application-agnostic event: The subscription request shall not specify any UE or DC application identifier. The DC AS identifier shall be included in the subscription request. A notification shall be triggered when a UE downloads any DC application associated with the subscribing DC AS. Notifications for DC applications associated with other DC ASs shall not be provided.

NOTE 1: It is assumed that the DCSF can determine that a DC application is associated with a DC AS either via configuration or via proper application naming convention.

Subscription requests may optionally include event filters to enable the DC AS to specify criteria for event reporting, such as periodic notification preventing excessive signalling for individual events. The IMS network may negotiate the event filter parameters with the DC AS to ensure alignment with network capabilities, resource availability, and operational policies.

The event subscription provided to the DCSF contains the AF Id of the DC AS allowing the DSCF to control whether to send a notification related to a given application to the DC AS.

NOTE 2: This is to ensure that a DC AS can receive notifications only about “its own applications”.

### 6.X.2	Procedure of DC application downloading event subscription and notification

Figure 6.X.2-1 depicts a signalling flow diagram for DC application downloading event subscription and notification.

Figure 6.X.2-1: Procedure of DC application downloading event subscription and notification.

1.	The AF (DC AS) sends a subscription request to IMS AS via the NEF. For Type 1 or Type 2 event targeting specific UE(s), Annex AD.3.2 of TS 23.228 [2] is followed. For Type 3 or Type 4 event targeting any UE, Annex AD.3.3 of TS 23.228 [2] is followed.

The IMS AS instance(s) store the subscription request (see 6.X.1). The IMS AS needs to understand the event type and the targeted UE(s), but does not need to understand other information about this event in the subscription request.

2.	The UE requests BDC establishment as defined in Annex AC.7.1 of TS 23.228 [2]. During this process, the IMS AS selects a DCSF.

3.	The IMS AS subscribes on behalf of the NEF/DC AS to the selected DCSF for the DC application download event(s) indicated in the subscription request received in step 1: when the IMS AS notifies the DCSF of the session establishment event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, etc…) it provides the DC AS / NEF subscription request to the DCSF.

4.	The DCSF responds to the IMS AS, indicating successful subscription.

5.	The UE downloads a DC application via the established BDC.

6.	The DCSF checks whether the downloaded DC application is associated with the DC AS having issued the subscription request. If it does not, the event shall not be notified.

NOTE : 	This is to ensure that a DC AS can receive notifications only about “its own applications”. To be able to detect whether the downloaded DC application is associated with the DC AS having issued the subscription request either the DSCF can be configured with the mapping between DC applications and the AF ID associated with the DC AS or the application Id (negotiated with the DC AS) contains a reference to this AF ID

7-8.	The DCSF notifies the NEF of the DC application downloading event. The NEF responds to the notification.

9-10.	The NEF notifies the DC AS of the DC application downloading event. The DC AS responds to the notification.

### 6.X.3	Impacts on services, entities and interfaces

DC AS:

-	Definition of new event about DC application downloading

IMS AS:

-	Forwarding of new event subscription request to DCSF based on event type and target UE(s)

DSCF:

- Receiving new event subscription request

- checking the association between DC application and DC AS

- Notifying of the event to DC AS

# * * * * End of changes * * *