SA WG2 Meeting #170	S2-2508503

13 – 17 Oct 2025, Wuhan, China	(was S2-2506302/7555)

Source:	Nokia

Title:	KI#3: Solution for DC3/DC4 enhancements

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for FS_NG_RTC_Ph3.

# 1. Discussion

This pCR proposes a solution to KI#3 in Next Generation Real Time Communication Phase 3.

Change marks show technical updates wrt S2-2507555

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #3> |
| #X: DC3/DC4 based on DCSF services |  | X |

# * * * * next change * * * * All text is new

## 6.X	Solution #X: DC3/DC4 based on DCSF services

### 6.X.0	High-level solution principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints.

The DC AS can subscribe to events related with MDC2 channels, targeting specific UE(s) or any targeting any UE. The subscription can be either implicitly or explicitly established. For an implicitly establishment, the subscription is configured at the DCSF. For an explicitly established, the subscription request is delivered to IMS AS by following the procedure defined in clause AD.3.2 of TS 23.228 [2] or in clause AD.3.3 of TS 23.228 [2]. Once, the UE(s) initiates a P2A/P2A2P ADC establishment, the IMS AS selects a DCSF instance for handling the ADC as defined in clause AC.7.2.2 and AC.7.2.3 of TS 23.228 [2] and forwards the DC AS subscription information to the selected DCSF instance. The subscription at DCSF lasts only as long as the corresponding end user session.

Upon receipt of a P2A/P2A2P ADC establishment request meeting the subscription filter conditions, the DCSF invokes a new Ndcsf_SessionEvent_Notify to notify the DC AS of the event that MDC2 needs to be established. This notification includes MF-side information required for MDC2 configuration and a Resource URL where the DCSF expects further instructions from the DC AS.

Based on the received notification from the DCSF, the DC AS derives the DC AS-side MDC2 configuration information and transfers it to the DCSF via a new Ndcsf_ADCControl_Create operation towards the Resource URL received in Ndcsf_SessionEvent_Notify.

### 6.X.1	Description

This solution specifies the DCSF services exposed over the DC3/DC4 interface to support UE-initiated P2A and P2A2P ADC establishment. Specifically, the following services are defined: Ndcsf_SessionEvent and Ndcsf_ADCControl. The associated service parameters and corresponding procedures are defined in clause 6.X.2.

The DC AS subscribes to (or is subscribed to) events notifications related with MDC2 channel establishment need . Such an event subscription can be explicitly initiated by the DC AS or implicitly configured in the DCSF. Events can be UE related (i.e., targeting specific UE(s)) or UE unrelated (i.e., targeting any UE) and associated with filter conditions such as P2A/P2A2P ADC associated with specific applications.

Upon receipt of a P2A or P2A2P ADC establishment request meeting the subscription filter conditions, the DCSF invokes Ndcsf_SessionEvent_Notify to notify the DC AS of the MDC2 channel establishment need event. This notification includes MF-side information required for MDC2 configuration. Based on the notification received, the DC AS derives the DC AS-side MDC2 configuration information and transfers it to the DCSF via the Ndcsf_ADCControl_Create service operation. The DCSF subsequently proceeds with the MDC2 configuration based on the information received from the DC AS.

This enhancement to the DC3/DC4 interface is crucial for ensuring consistent and up-to-date MDC2 media resources between the DCSF and the DC AS. The DC AS and the MF may dynamically deploy/insert MDC2 terminations and thus use the dynamically determined IP address. The DC port and/or DC SCTP port may be dynamically associated to the session by the MF and/or the DC AS. Furthermore, the network (DCSF) may need to provide the DC AS with the UE certificate fingerprint received from the UE over IMS signaling.

The current P2A/P2A2P data channel procedures lack a mechanism for MDC2 media resource negotiation, leading to potential inconsistencies of MDC2 media resource between the two MDC2 endpoints. This proposed enhancement, by enabling the MDC2 media negotiation and dynamic configuration, significantly improves the robustness and flexibility of these procedures, preventing failures due to outdated or missing MDC2 media resource information.

### 6.X.2	Service definition

#### 6.X.2.1	General

The following table shows the DCSF Services and DCSF Service Operations.

Table 6.X.2-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_SessionEvent | Subscribe | Subscribe/Notify | IMS AS |
|  | Unsubscribe | Subscribe/Notify | IMS AS |
|  | Notify | Subscribe/Notify | NEF, Trusted AF/DC AS |
| Ndcsf_ADCControl | Create | Request/Response | NEF, Trusted AF/DC AS |
|  | Update | Request/Response | NEF, Trusted AF/DC AS |
|  | Delete | Request/Response | NEF, Trusted AF/DC AS |

Editor's Note: Further information, e.g., QoS, charging, monitoring, is for FFS.

Editor's Note: The content of UE's and DC AS's certificate fingerprint for end-to-end security between UE and DC AS shall be aligned with SA3.

#### 6.X.2.2	Ndcsf_SessionEvent

##### 6.X.2.2.1	General

Service description: This service enables the consumer to subscribe to and to be notified about events related with MDC2 channels.

The following operations are available for this service:

-	Subscribing and unsubscribing to receive event related with MDC2 channels.

-	Notifying the event to consumers, e.g. NEF, trusted DC AS.

##### 6.X.2.2.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event related with MDC2 channels (in this release only event is defined “the establishment of a MDC2 channel is needed”).

Inputs, Required: Notification Target address, Event ID

NOTE: 	The Notification Target address contains the Notification target address of the NEF or of the subscribing trusted DC AS.

Only one Event Id is currently defined: establishment of a MDC2 channel is needed.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (as an alternative, to be decided by stage 3, an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

##### 6.X.2.2.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from an event .

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.2.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), Resource URL(for any Ndcsf_ADCControl operation that would correspond to the same session), DC application ID

One or two MF-side MDC2 addressing information are provided depending on whether the session is established in P2A or P2A2P mode.

Inputs, Optional: Calling party and called party IDs, application ID, security Setup if UE initiates the DTLS negotiation, , certificate fingerprint of UE for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

.Outputs, Required: Result.

Outputs, Optional: None.

#### 6.X.2.3	Ndcsf_ADCControl

##### 6.X.2.3.1	General

Service description: This service enables the consumer to create, update or terminate an ADC.

##### 6.X.2.3.2	Ndcsf_ADCControl_Create

Service operation name: Ndcsf_SessionEvent_Create

Description: This service operation enables the consumer to create information used by the DCSF for ADC establishment in an IMS session.

Inputs, Required: Session ID, DC AS-side MDC2 endpoint information (e.g. IP address, port number),.

One or two MF-side MDC2 addressing information are provided depending on whether the session is established in P2A or P2A2P mode.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation, certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.3.3	Ndcsf_ADCControl_Update

Service operation name: Ndcsf_ADCControl_Update

Description: This service operation enables the consumer to update information used by the DCSF for ADC updating in an IMS session.

Inputs, Required: Session ID.

Inputs, Optional: DC AS-side MDC2 endpoint information (e.g. IP address, port number),

.Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.3.4	Ndcsf_ADCControl_Delete

Service operation name: Ndcsf_ADCControl_Delete

Description: This service operation enables the consumer to provide information used by the DCSF for ADC termination in an IMS session

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

### 6.X.3	Procedure

#### 6.X.3.1	Person-to-Application (P2A) Application Data Channel Setup

Figure 6.X.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.X.3.1-1: P2A ADC establishment with a trusted DC AS.

0.	A subscription of the DC AS about the need for establishment of a MDC2 channel is created; this subscription is either implicitly configured at the DCSF (e.g., based on the DC Application), or explicitly provided to the IMS AS following the exposure framework defined in Annex AD of TS 23.228 [2]

1.	IMS session and BDC are established..

2-9.	UE#1 requests the establishment of P2A ADC establishment. The IMS AS forwards the DC AS subscription request to the selected DCSF instance: when the IMS AS notifies the DCSF of the session establishment event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, etc…), the IMS AS provides the DC AS subscription request to the DCSF. The DCSF determines the DC control policy and indicates the IMS AS to request MF for reserving the originating media resources.

10.	The DCSF notifies DC AS about the need to establish a MDC2 channel (indicating the session is established in P2A mode) using Ndcsf_SessionEvent_Notify message. The message includes calling and called parting id, application id, session id, MF-side MDC2 media resource info, Resource URL to be used by DC AS to send subsequent ADC Control message, and the certificate fingerprint of UE.

The DC AS responds to (acknowledges) the notification message.

11.	The DCSF waits for ADC control instruction (Ndcsf_ADCControl_Create). If such instructions are not provided then DCSF has the ADC establishment procedure aborted (Nimsas_MediaControl_MediaInstruction).

12.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS sends Ndcsf_ADCControl_Create message with the allocated DC AS-side MDC2 media resource information. The message includes session id, DC AS-side MDC2 media resource info, and may include the certificate fingerprint of DC AS.

NOTE:	Work on the handling of security information like certificate fingerprints is to be co-ordinated with SA3.

13.	The DCSF sends response of Ndcsf_ADCControl_Create to DC AS.

14-17.	The DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF. This follows already specified interactions

18-29.	IMS AS performs SDP negotiation with terminating network. With a successful SDP negotiation, the application data channel is established. This follows already specified interactions

#### 6.X.3.2	Person-to-Application-to-Person (P2A2P) Procedure

Figure 6.X.3.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application to person use case.

Figure 6.X.3.2-1: P2A2P ADC establishment with a DC AS.

0.	As in step 0 of Figure 6.X.3.1-1

1.	As in step 1 of Figure 6.X.3.1-1

2-9.	UE#1 requests the establishment of P2A2P ADC establishment. The IMS AS forwards the DC AS subscription request to the selected DCSF instance: when the IMS AS notifies the DCSF of the session establishment event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, etc…), the IMS AS provides the DC AS subscription request to the DCSF. The DCSF determines the DC control policy and indicates the IMS AS to request MF for reserving the originating and terminating media resources.

10.	DCSF notifies DC AS about the session establishment (indicating the session is established in P2A2P mode) using Ndcsf_SessionEvent_Notify message. The message includes calling and called parting id, application id, session id, MF-side MDC2 media resource info, Resource URL used by DC AS to send subsequent ADC Control message, and the certificate fingerprint of UE.

DC AS responds to (acknowledges) the notification message.

11.	DCSF waits for ADC control instructions (Ndcsf_ADCControl_Create). If such instructions are not provided then DCSF has the ADC establishment procedure aborted (Nimsas_MediaControl_MediaInstruction).

12.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS sends Ndcsf_ADCControl_Create message to the DCSF with the allocated DC AS-side MDC2 media resource information. The message includes session id, DC AS-side MDC2 media resource info, and may include the certificate fingerprint of DC AS.

NOTE:	Handling of security information like certificate fingerprints is to be co-ordinated with SA3.

13.	The DCSF sends response of Ndcsf_ADCControl_Create to DC AS.

14-21.	IMS AS performs SDP negotiation with terminating network. And the IMS AS notifies the DCSF about the successful result of the SDP negotiation. . This follows already specified interactions

22.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information and the media resource information of the terminating network. Then, the IMS AS sends response to DCSF. This follows existing procedures e.g. steps 8 of TS 23.228 [2] Figure AC.7.2.3-1

23-29.	IMS AS continues performing SDP negotiation with the originating network. With a successful SDP negotiation, the application data channel is established. . This follows already specified interactions

### 6.X.4	Impacts on services, entities and interfaces

The solution has the following impacts:

DC AS:

-	subscribing to the event of “establishment of a MDC2 channel is needed”, and receiving corresponding notifications containing MF-side information for MDC2 configuration,

-	sending DC AS-side information for MDC2 configuration,

-	sending/receiving certificate fingerprints for end-to-end security between UE and DC AS,

DCSF:

-	receiving subscription to the “event of establishment of a MDC2 channel is needed”, and sending corresponding notification, containing MF-side information for MDC2 configuration,

-	receiving DC AS-side information for MDC2 configuration,

-	sending/receiving certificate fingerprints for end-to-end security between UE and DC AS,

IMS AS:

-	sending subscription request to the DCSF instance, which handles the P2A/P2A2P ADC,

# * * * * End of changes * * *