SA WG2 Meeting #171	S2-2508534

13 – 17 October, 2025, Wuhan, CN	(revision of S2-2508043)

Source:	Apple, MediaTek Inc.

Title:	[WT#4] WT revision and KI for Integrated Sensing and Communication

Document for:	Approval

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: WT#4 revision and KI proposal on ISAC based on postponed S2-2508043 at SA2#170.

# 1.	Discussion

This paper proposes a WT revision and KI description for ISAC based on the postponed document S2-2508043 at SA2#170.

Two changes are introduced with respect to S2-2508043, which can be explained as follows:

-	It is proposed to study all sensing modes as part of WT#4. This is because at RAN#109, the following proposals were endorsed according to the meeting report excerpt shown below:

where the first bullet of moderator’s proposal 4 in RP-252943 reads as follows:

-     6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic.

-	A checkpoint is proposed in Q1 2026 to confirm scope of WT#4, based on the status of both FS_Sensing_ARC in SA2 for Rel-20 5G-A, and 6G Sensing in RAN. This is in line with the way forward proposal by the FS_6G_ARC rapporteur during the CC on September 24.

In addition, a KI description is proposed based on the same WT structure with additional details, re-using contents from S2-2507900 noted at SA2#171.

# 2.	Proposal

It is proposed to agree the following changes in TR 23.800-01.

**** First Change (based on S2-2508043) ****

# Annex A.X. WT#4 Scope

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

The following aspects will be studied:

a)	Architecture and function support for sensing, including:

b)	Service authorization and revocation.

c)	Discovery and (re-)selection of Sensing Entities.

d)	Configuration parameters and policy provisioning to support sensing service, e.g., to the selected Sensing Entities, Sensing function, etc.

e)	Collection, distribution of sensing related data and exposure of sensing result.

f)	Sensing continuity in consideration of mobility of sensing entities and/or target objects.

g)	Support for non-3GPP sensing data and data fusion.

NOTE 1:	WT#4 needs to consider the work and conclusions of R20 FS_Sensing_ARC on gNB-based sensing.

NOTE 2:	The scope of WT#4 includes the study of all sensing modes and their selection, where sensing may involve RAN node and UE.

NOTE 3:	Aspects a), b) and e) require coordination with WT#5. The data framework applicable for collection, distribution and exposure of sensing related data is in scope of WT#5.

NOTE 4:	Coordination with RAN WG for RAN aspects, SA3 for privacy and consent aspects, and SA5 for OAM aspects will be needed.

NOTE 5:	The scope of WT#4 will be aligned with RAN for 6G. A checkpoint is set for Q1 2026 to confirm scope based on FS_Sensing_ARC and 6G Sensing RAN status before discussing solutions.

**** Second Change (all new text) ****

## 5.X.	Key Issue #X: Support of Integrated Sensing and Communication

### 5.X.1	Description

This Key Issue focuses on the 6G architecture and procedures to support Integrated Sensing and Communication (ISAC). The following aspects will be studied:

a)	Architecture and function definition to support ISAC for all sensing modes;

NOTE 1:	The scope of this key issue includes sensing modes and their selection where sensing may involve RAN node and UE.

b)	Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requirements from the Sensing service consumer, and the authorization and revocation for a UE as Sensing Entity;

c)	Discovery and (re-)selection of Sensing Entity(ies) and Sensing Function(s) based on sensing service requirements from the Sensing service consumer and the capabilities of the Sensing Entity(ies) and Sensing Function(s);

d)	Parameter configuration and policy provisioning of the selected Sensing Entity(ies);

e)	Collection and distribution of the Sensing measurement data and the associated information for Sensing Result calculation.

f)	Exposure of the Sensing Result to the Sensing service consumer.

NOTE 2:	The above-mentioned bullet may require coordination with WT#5. Data framework applicable for ISAC (e.g., Sensing measurement data collection, distribution, Sensing Result exposure, and the storage) will be addressed as part of Key Issues related to WT#5 of 6G SI on Data Framework.

g)	Sensing continuity for the moving of the Object(s) and the UE(s) as the Sensing entity(ies);

h)	Support for non-3GPP sensing data and data fusion with 3GPP sensing data.

NOTE 3:	Functionalities related to RAN nodes will be defined together with the RAN working groups.

NOTE 4:	Privacy and security require coordination with SA3, OAM and charging aspects require coordination with SA5.

NOTE 5:	Solutions to this KI should consider FS_Sensing_ARC conclusions for gNB-based sensing.

**** End of Changes ****