3GPP TSG-WG SA2 Meeting #171	S2-2508570

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[WT#1.1] WT and KI for NAS functionality and UE-CN control signalling

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract: It is proposed to add WT and KI for NAS functionality and UE-CN control signalling.

1. Introduction/Discussion

During the discussion in SA2#170, people have different views on how to differentiate connectivity services vs. beyond connectivity services. There was a proposal to use NAS functionalities vs. operator services. However, operator services is also an ambitious term. For instance, IMS can be considered as operator service but the SIP signalling is not part of NAS.  Besides, there is no need to differentiate different kind of services. The purpose is to try to have a unified mechanism. If one has different solutions for different NAS functionalities, SA2 can discuss it during solution phase.

Thus, it is proposed to unify the terminologies in the WT. For the KI, it is proposed to capture the details of the NAS design, including RAN-CN interaction related to NAS and the high-level architectural aspects of the security framework for NAS distribution.

Besides, it is unclear that why bullet b mentions “Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.” Which is conflict with bullet a, i.e., enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities. It is proposed to remove bullet b.

The Key Issue is updated accordingly based on S2-2507988 (noted) last meeting.2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *All new

Annex A.X. WT#1.1

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#1.1 Study the support for 6G System NAS control signalling, including the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

NOTE: 	It is assumed that this WT covers impacts to 6G System procedures for functionalities by the NAS mechanism.

# * * * * Second change * * * *All new

5.X. Key Issue #X: Aspects for non-access stratum functionality and UE-CN control signalling

This Key issue studies the following aspects:

1)	Whether and how to enable the introduction of new non-access stratum functionality, such that other non-access stratum functionalities as well as associated functional entities are not affected. It includes how to avoid dependency among the associated functional entities for NAS message termination.

NOTE: This includes whether and how to structure NAS functionality. Coordination with CT1 could be required

2)	Whether and how to support flexible NAS distribution considering the termination of the NAS message and how to discover the associated functional entities by the UE.

3)	Identify the high-level architectural aspects of the security framework for NAS distribution.

NOTE Z: Potential security and privacy impacts require coordination with SA3.

4)	Study signaling interactions and system procedures between UE-CN for non-access stratum functionality, including related, potential signaling interactions and system procedures (e.g., Mobility Management) between RAN-CN and related signaling interactions and system procedures within 6G CN. The signalling interactions and system procedures within 6G CN should minimize the inter-NF dependency.

NOTE X: Coordination with RAN3 is required for RAN-CN.

NOTE A: The content of NAS messages depends on the discussion of other WTs (e.g. WT 1.2, 3~6).

NOTE Y: Interworking and migration aspects, especially inter-PLMN aspects (e.g. roaming), where applicable, are covered by WT#2.

NOTE Z: Potential security and privacy impacts require coordination with SA3.

# * * * * End of changes * * * *