3GPP TSG-WG SA2 Meeting #170	S2-2508616

# Wuhan, China, October 13-17, 2025		   (revision of S2-2507147)

Source: 	China Mobile

Title: 	Agreed principles for key issue #3

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: Propose agreed principles for key issue #3.

1	Discussion

This paper is to propose agreed principles for KI#3: NF (re)selection and/or UP path adjustment based on energy related information.

As described in clause 5.3.1, this key issue has two parts: 1) NF (re)selection based on energy related information, and 2) UP path adjustment based on energy related information. In current version of TR 23.700-67, Solutions #19, #20, #21, #22, #23, #28, #29 and #30 address this key issue.

For part 1), Solution #19 and Solution #20 are mainly related to NF (re)selection. Solution #19 proposes an AMF (re)selection. However, AMF (re)selection is performed by NG-RAN, and there is consensus on importance energy saving for UPF and NG-RAN instead of AMF. Therefore, the only NF that agreement can be achieved on is UPF. Solution #20 proposes NF profile enhancement for energy consideration, and that existing NF selection procedures shall be reused. For NF profile, EnergySaving/Not EnergySaving State, Energy Priority Information and Renewable Energy Information is proposed as agreement. Solution #28 introduces EPDI for energy priority per S-NSSAI/group. This is not necessary, and it is proposed to put forward basic agreements in this doc. Solution #29 proposes N3IWF/TNGF reselection based on energy information. The energy saving benefit of N3IWF/TNGF (re)selection is not clear. Solution #30 defines NF status to control the discoverability of the NF. This is detailed solution for energy state of solution #20.

For part 2), Solution #21, #22 and #23 are mainly related to UP path adjustment. The solutions propose different procedures and different granularities to estimate the energy related information of the UP path, which have not reached agreement. It is proposed to conclude the three solutions’ consensus: UP path adjustment may consider the renewable energy information and energy consumption/efficiency.

2	Proposal

It is proposed to include the below changes into TR 23.700-67 v1.0.0.

FIRST CHANGE (all new text)

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	UPF can be (re)selected based on energy related information.

-	The NF profile is enhanced, including the following information:

-  Energy Saving State Information, as EnergySaving /Not EnergySaving State described in clause 6.20.1;

-  Energy Priority Information, as described in clause 6.20.1;

-  Renewable Energy Information, as described in clause 6.20.1.

-	PDU Session Modification procedure is reused for UP path adjustment.

End of CHANGE