3GPP TSG-SA WG2 Meeting #171	S2-2508634

Wuhan, China, 13-17 October 2025

Agenda item:	19.4.2

Source:	Nokia

Title:	Discussion on overlapping Predefined PDRs

WI/Release:	EnergySys /Rel-19

Document for:	Discussion and Decision

# 1	Discussion

At SA2#170 the following papers  S2-2506407 S2-2506408 S2-2506409 were submitted, proposing to shift from the PFCP based approach on N4 for reporting of usage data, at the granularity requested by the EIF, to the SMF and then onwards to the EIF, to the SBI-based UPF exposure service to EIF. The reason to do this was quoted to be the fact that for the case of predefined PDRs (e.g. based on application identifier) the SMF is not aware of the actual Packet Filters, and only the UPF is.

The problem was acknowledged as with the current approach the SMF needs to be aware of the packet filters to ensure that in case of conflicting PDRs, the PDRs can be restructured to eliminate conflicts.

However, companies did not consider the solution to be valid as while it was certainly resolving this case, it was introducing bigger problems as immediate reporting at gNB change or I-UPF change is not possible, and more enhancements would be needed.

Therefore, it was agreed to resolve the issue at SA2#171. This paper provides a way forward on this subject.

The case to be solved seems only the case where there is a partial overlap between a packet filter due to a PDR associated to a request from EIF, and one that comes from the PCF. In all the following cases, however, there is no problem:

- PDU session level reporting

- S-NSSAI level reporting for the UE

- Application ID/Predefined PDR based reporting where there is full overlap between the PDRs of EIF and PCF predefined PDRs

We believe that it is possible to make visible to the SMF the semantics of the predefined PCC rules/PDRs and also of the Application IDs. We therefore assume that the SMF can be aware of the semantics for the predefined PCC rules/PDRs and Application IDs. If the SMF detects a conflict with an existing set of packet filters of an application identifier or Predefined PCC rule or PDR, it can remove the applicable URRs based on predefined PDRs/Application IDs which are causing conflict and replace them with URRs based on non-predefined PDRs which are determined to not cause any conflict. Other implementation options are possible.

The SMF can be aware of the predefined PDRs and Application IDs semantics by OAM.

Also, when an AF provides the PFDs corresponding to Application Identifier to UPFs via the NEF and SMF, all SMFs can subscribe to be updated about the existence of new Application ID/PFDs association and retrieve the Application ID to PFDs mapping from the NEF. .

#### 4.18.3.1 PFD Retrieval by the SMF This procedure enables the SMF to retrieve PFDs for an Application Identifier from the NEF (PFDF) when a PCC rule with this Application Identifier is provided/activated and PFDs provided by the NEF (PFDF) are not available at the SMF. In addition, this procedure enables the SMF to retrieve PFDs from the NEF (PFDF)when the caching timer for an Application Identifier elapses and a PCC Rule for this Application Identifier is still active. The NEF (PFDF) retrieves the PFDs from UDR unless already available in NEF (PFDF). The SMF may retrieve PFDs for one or more Application Identifiers in the same Request. All PFDs related to an Application Identifier are provided in the response from the UDR to NEF (PFDF). Figure 4.18.3.1-1 PFD Retrieval by the SMF

#### 4.18.3.2 Management of PFDs in the SMF This procedure enables the provisioning, modification or removal of PFDs associated with an application identifier in the SMF. Either the complete list of all PFDs of all application identifiers, the complete list of all PFDs of one or more application identifiers or a subset of PFDs for individual application identifiers may be managed. Each PFD of an application identifier is associated with a PFD id if a subset of the PFD(s) associated with an application identifier can be provisioned, updated or removed. If always the full set of PFD(s) for an application identifier is managed in each transaction, PFD ids do not need to be provided. Figure 4.18.3.2-1 Management of PFDs in the SMF

The above is considered to be a feasible option, which impacts the SMF logic, as the existing solution does. Therefore, Nokia proposes this is pursued by a 23.501 CR S2-2508635 that indicates an underlying assumption that the SMF is aware of the predefined PDRs / Application IDs to PFD mappings. Then, it is up to implementors to figure out the details of the handling as specified above.

The alternative option, for which a CR is provided also in S2-2508636 and S2-2508638, which Nokia considers acceptable is the removal of the SDF/Application ID level reporting of Energy Consumption from the release.

# 2	Conclusions

Based on the discussion above it is proposed to not change the specification and just note that the SMF should be aware of the semantics of the Application Identifiers/ Predefined PDRs and apply logic accordingly. Alternatively, Nokia also considers acceptable to not reporting energy consumption at SDF/Application ID level in this release, as this is a more advanced use case that may be justified only by more precise energy consumption information reporting.