3GPP TSG-SA2 Meeting #171	S2-2508642

Wuhan, China, October 13-17 2025	(revision of S2-2507677)

Source:	Nokia, Lenovo

Title:	KI#3: proposed agreement of principles

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to agree principles for KI#3.

# 1.	Introduction

This paper proposes a n initial set of principles for agreement and also outstanding topics that deserve further consideration. (yellow text is new text from last meeting version in S2-2507677)

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change (all new text) * * * *

### 7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in the table below. This is a feature requiring the software to be upgraded in NRF and its consumers.

| Information | Description |
| --- | --- |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1). When the NF profile indicates that the NF is in EnergySaving state, the NF cannot be discovered nor selected. |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, includes the value of EnergySaving or not EnergySaving state which applies at each time interval in the schedule. If this is present, the EnergySaving /Not EnergySaving State attribute is not present. NRF and NF consumers should avoid selecting NFs that include a scheduled EnergySaving State if the NF consumers require the NF to be available (e.g. SSC mode 1) |
| Energy Priority Information | Value of Energy priority Set by the operator according to own policies. It indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF (including aspects like the power source characteristics/cost used for the hosting of the NF) |
| Schedule of Energy Priority Information | The Schedule of the Energy Priority includes the value of Energy Priority information which applies at each time interval in the schedule. If this is present, the Energy Priority information attribute is not present. NRF and NF consumers should avoid selecting NFs that include a scheduled shift to a lower priority that is likely to cause reselection of the NF if the NF consumer require the NF being used to be stable (e.g. SSC mode 1) |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-[	[CONDITIONALLY AGREED] For NF profile, additional parameter such as renewable energy consumption ratio may be considered if SA5 indicates support.

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection subject to SSC mode of the PDU sessions.

### 7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	whether the NF profile needs to include explicit indication of usage of Renewable Energy or using Energy Priority to implicitly take this into account is sufficient. Note that this depends on availability of renewable energy information at a NF which for now is FFS.

-	Whether considering the energy consumption of a PDU sessions or of a NF over a Path should be a trigger for reselection of UPFs (including embedding this information in DNAIs )

-	In case of multiple configured energy policies in network, the "energy priority" is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

# * * * * End of change * * * *