3GPP TSG-WG SA2 Meeting #171	S2-2508700

13th – 17th October 2025, Wuhan, China	  (is revision of XXXX)

Source:	Lenovo, China Telecom

Title:	KI#1 Interim Agreements on UE reader authorization and revocation

Document for:	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes the interim agreements principles for Key Issue 1, especially on UE reader authorization and revocation.

1	Introduction

This paper proposes the interim agreements of Key Issue 1, and more specfically, the UE reader authorization and UE reader revocation part, and is also related to the UE reader subscription data.

1.1    Analysis of UE reader revocation

For UE reader revocation, a general procedure that is described in the contribution (S2-2508697, Lenovo) can be referred as the baseline, and a figure is copied here for easier understanding.

Figure 1.1 UE reader authorization revocation procedure

As depicted in Figure 1.1, after UE is authorized as the AIoT reader, the revocation of the UE reader authorization can be determined at two network functions, i.e., AMF (Step 10-a) and AIOTF (Step 10-b), and it can be based on the following different factors:

Option 1 AMF determination:

Based on the local configuration and policies, the notification from UDM that UE reader is no longer allowed to be operating as an AIoT reader, or the validity information of the UE has become invalid (e.g., duration or area), AMF determines that the UE reader authorization needs to be revoked, and sends the UE reader revocation indication to the NG-RAN and optionally UE reader.

AMF will also send the revoked UE reader information to the AIOTF so that the AIOTF will not select the revoked UE reader for the service request from AF.

Option 2 AIOTF determination:

Apart from the determination at the AMF for UE reader revocation, we are proposing that the AIOTF can also determine to revoke the UE reader authorization, based on the 1) local configuration that related to the SLA between the 3rd party AF and MNO, or 2) based on the request from the 3rd party AF to revoke specific UE reader (s). The reason lies in that the UE reader may be owned by the 3rd party and as the owner, it has the authority to revoke the UE reader towards the MNO, either based on the updated SLA, or based on an explicit control signalling.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-30.

# * * * * First change * * * *

# All new texts

7	Interim agreements

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

- Subscription aspects

  - The UE subscription data in the UDM will be extended with UE Reader subscription information (available to both AIOTF and AMF), which consists of the following:

    - Indication indicating whether the UE is authorized to operate as an AIoT Reader.

    - Validity information: valid time duration and area that allowed for UE to act as an AIoT Reader.

- UE reader authorization

  - If AMF receives, as part of the UE Reader subscription information, the indication that the UE is authorized to operate as a UE Reader, then AMF informs NG-RAN that the UE is authorized to operate as a UE Reader that contains the UE ID and the indication.

  - AMF sends the authorized UE reader information to the AIOTF that contains the UE ID and authorization indication, the serving NG-RAN information of the UE reader.

  - Editor’s note: The UE ID sent from AMF to NG-RAN and AIOTF is FFS.

  - NG-RAN will allocate the radio resources for the UE that is authorized as a UE Reader.

  - Optionally, the indication may also be sent to the UE to notify that the UE is authorized as an AIoT Reader.

- UE reader revocation

  - AMF determines that the UE reader authorization needs to be revoked and sends the revoked UE reader information that contains the UE ID and revocation indication to the AIOTF, based on

    - Local policies and configurations.

    - The notification from UDM that the UE reader is no longer authorized as AIoT Reader.

    - The validity information (time duration and area) of the UE reader becomes invalid.

  - AIOTF determines that the UE reader authorization needs to be revoked and sends the revoked UE reader information that contains the UE ID and revocation indication to the AMF, based on

    - Local configuration that is related to the SLA.

    - AF request to revoke the specific UE reader (s).

  - For both above options, AMF sends the UE reader revocation indication and UE ID to the NG-RAN that serves the UE reader.

  - For both above options, AIOTF deletes the locally stored UE reader information after it is revoked.

  - NG-RAN releases the radio resources allocated for the revoked UE reader and may also send the revocation indication to the revoked UE reader.

  - UE stops acting as UE reader after it is notified, either implicitly (e.g., no radio resources allocated to UE reader), or explicitly (receiving revocation indication).

# * * * * End of changes * * * *